SA WG2 Meeting #171	S2-2508472

13 – 17 Oct, 2025, Wuhan, CN	revision of S2-2508127

Source:	China Telecom

Title:	[WT#1.3, Non-3GPP Access] WT Scope and KI for non-3GPP access

Document for:	Approval

Agenda Item:	20.6.1.3

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes the work task scope and key issues for WT#1.3.

**** First Change ****

Annex A.X. WT#1.3

It proposes to add several sub-WTs to address the non-3GPP access scenario and multi-access data connections.

1.3.	Study whether and how to support and/or enhance different non-3GPP access (e.g. Wi-Fi, wireline) in 6G and support multi-access data connections between 3GPP access and non-3GPP access.

For non-3GPP access:

-	Study how to support untrusted non-3GPP access in 6G System architecture.

-	Study how to support non-3GPP wireline access in 6G System architecture.

NOTE 1:	For wireline access, co-ordination with BBF and CableLabs is needed.

-	How to support service continuity between 3GPP access and untrusted non-3GPP access.

NOTE 2:	Interworking aspects related to non-3GPP access will be studied as part of WT#2.

NOTE 3:  WT#1.2 aspects related to QoS, Policy, and UP applicable to non-3GPP access are studied under this WT with a checkpoint to align with WT#1.2 at TBD.

-	Whether and how to support Non-Seamless WLAN Offload (NSWO).

For the multi-access:

-	Study whether and how to support the multi-access data connectivity (e.g. traffic steering, switching and splitting) between 3GPP access and non-3GPP access for different type of traffic (e.g. IPv4/v6, UDP, TCP, Ethernet) in 6G.

**** Second Change ****

5.X. Key Issue #X: Support of non-3GPP access

This key issue will address the following issues:

For supporting the non-3GPP access:

-	Identify how to support connectivity establishment via non-3GPP access, including the registration and session establishment procedure.

-	Identify how to support service continuity between untrusted non-3GPP access and 3GPP access.

For supporting multi-access data connections between 3GPP access and non-3GPP access:

-	Identify how the Core network and the UE can support multi-access data connectivity between 3GPP and non-3GPP accesses.

NOTE 1:	Multi-access connectivity is dependent on the support of connectivity establishment via non-3GPP access.

**** End of Changes ****