3GPP TSG SA WG2#171	S2-2508521

Wuhan, China, October 13 - 17, 2025

Source:	Lenovo

Title:	Evaluations and proposed Interim conclusion for KI#4

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing-ARC / Rel-20

Abstract of the contribution: Proposes evaluations and interim conclusion for KI#4 Sensing Data and the Associated Information Collection and Transport

# 1. Introduction

This pCR discusses the evaluations and proposes the interim conclusion for KI#4 Sensing Data and the Associated Information Collection and Transport. This paper mainly focuses on the following issues,

- How to collect and transport the sensing data and the associated information? Via control plane or user plane?

- For user plane based solution, whether the data connection between Sensing RAN node and SF is per node or per sensing service request?

- Which node triggers data connection between Sensing RAN node and SF?

# 2. Discussion

There’re 12 solutions which have been proposed to address KI#4, i.e., Solution#1, Solution#3, Solution#5, Solution#18, Solution#21-26, Solution#34 and Solution#39.

- Solution#1 proposed that Sensing Function (SF) to trigger each Sensing Entity (SE) to establish a direct connection between SF and SE, by providing user plane address, transport protocols and required security information etc. SF uses the direct connection to request for sensing service and obtain sensing data and the associated information from the SE. A direct interface is established over the connection, without going through other core network control functions, e.g. AMF. The direct connection can be considered as the user plane solution. The data collection seems to be per node. The SF triggers SE to establish the direct connection upon receiving sensing service request.

- Solution#3 proposed that SF being splits into sensing control function (SCF) and sensing processing function (SPF). The SCF provides configuration parameters to SE and SE may trigger the establishment of the path for 3GPP sensing data reporting towards SPF. It does not address the issue that whether those configuration parameters are directly sent to the SE or not. The path reporting between SE and SPF can be considered as the user plane solution. The data collection seems to be per node. The SCF triggers SE to establish the direct connection upon receiving sensing service request.

- Solution#5 proposed AMF-Inclusive framework for 5GC sensing. SF selects AMF (via NRF/UDM) and AMF is responsible for discovery and selection of sensing entities. The sensing data is transmitted to the SF via the AMF, which processes the data to derive the sensing results. The data collection can be considered as the control plane solution.

- Solution#18 proposed that the Sensing Function provides its connection information (e.g. IP address) for data connection and binding information (e.g. Sensing Entity ID, connection ID or specific binding ID) to the Sensing Entity. The Sensing Entity sends binding information via established data connection to sensing function for connection binding. The data collection can be considered as the user plane solution. The data collection seems to be per node. The SF triggers SE to establish the direct connection.

- Solution#21 proposed that the SF and NG-RAN acting as sensing entity supports direct interface Nx to transfer sensing message. The SF sends DL service message transfer to the selected NG-RAN, which includes correlation ID and sensing request message. The data collection can be considered as the control plane solution.

- Solution#22 proposed that Sensing Consumer sends sensing service request to SF. SF selects Sensing RAN node and triggers data tunnel establishment towards the selected Sensing RAN node by providing SF tunnel info. The data collection can be considered as the user plane solution. The data collection seems to be per sensing service request. The SF triggers SE to establish the direct connection upon receiving sensing service request.

- Solution#23 proposed that SeMF assigns sensing service request to gNB and sensing processing request to SePF. SePF requests gNB to provide sensing data. The SePF#1 may request SePF#2 to process the sensing data. The data collection can be considered as the control plane solution.

- Solution#24 proposed that SSMF selects sensing RAN node and SDCF. SSMF selects SDCF and provides SDCF endpoint information to Sensing RAN node. SDCF is responsible for processing the sensing data and SSMF may aggregate sensing data report. The data collection can be considered as the user plane solution. The data collection seems to be per node.

- Solution#25 proposed that SCF selects SPF and helps gNB and SPF to establish direct connection by providing connection configure information. And the details of connection configure information is FFS. The data collection can be considered as the user plane solution. The data collection seems to be per node. The SSMF triggers SE to establish the direct connection upon receiving sensing service request.

- Solution#26 proposed that upon receiving sensing service request, SCF first checks whether the sensing data is available at SDMF. If not, SCF assigns the sensing service request to the selected sensing RAN node. SCF triggers sensing RAN node to establish data reporting tunnel with SPF. The data collection can be considered as the user plane solution. The data collection seems to be per sensing service request. The SF triggers SE to establish the direct connection upon receiving sensing service request.

- Solution#34 proposed that SCF triggers Sensing RAN no to establish data connection with SPF by providing the data connection address information. The data collection can be considered as the user plane solution. The data collection seems to be applied to both per node and per sensing service request. The SCF triggers SE to establish the direct connection upon receiving sensing service request.

- Solution#39 proposed AMF service to support sensing service. SF assigns sensing task to the sensing RAN node via the AMF. The data collection can be considered as the control plane solution.

In short, 8 solutions out of 12 solutions supports sensing data reporting via user plane. For the UP-based solutions, 6 solutions out of 8 solutions supports per node data connection between Sensing RAN node and SF. All UP based solutions propose the Sensing Function to trigger the establishment of the direct data connection between the Sensing Function and the Sensing RAN node.

# 3. Text Proposal

It is proposed to capture the following changes in TR 23.700-14.

# * * * * First change * * * *

# 7	Interim agreements

## 7.1	Agreed Principles

### 7.1.Y	Agreed Principles for KI#4

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#Y. This may be populated directly or e.g. also when a topic in clause 7.2.Y gets resolved and a principle is agreed.

- There’s direct data connection between Sensing RAN node and Sensing Function. Sensing RAN node reports the sensing data and the associated information to Sensing Function via the direct data connection. The format of sensing data requires coordination with RAN1.

- Sensing Function requests Sensing RAN node to perform sensing service by providing sensing service ID, sensing service requirement and reporting configuration etc. Sensing RAN node provides sensing service ID when reporting the sensing data and the associated information associated with the sensing service ID.

- Sensing Function triggers direct data connection establishment towards Sensing RAN node during the configuration procedure of the Sensing RAN node for the sensing service.

  - Sensing Function exchanges data connection information (e.g. IP address, terminal endpoint info) with the Sensing RAN node via either direct connectivity or indirect connectivity.

  - For direct connectivity, it is assumed that the Sensing RAN node triggers Nx interface establishment towards the Sensing Function. And the Sensing Function triggers direct data connection establishment towards the Sensing RAN node.

  - For indirect connectivity, it is assumed that Sensing RAN node provides sensing capability when establishing N2 interface with AMF. AMF then notifies the Sensing Function about the existence of the Sensing RAN node. Then the Sensing Function triggers data connection establishment towards the Sensing RAN node.

NOTE: it is FFS whether the Sensing Function triggers data connection upon Nx interface establishment, notification from AMF, or sensing service request from AF.

- The data connection between Sensing RAN node and Sensing Function is per node. The detailed protocol used for the data connection requires coordination with RAN3.

# * * * * End of changes * * * *