3GPP TSG-SA2 Meeting #171	S2-2508539

Wuhan,China, October 13th -17th 2025

Source:	Intel

Title:	KI#1, Interim conclusion

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: Interim conclusion for KI#1 – System architecture to support sensing is proposed.

# 1. Introduction

The discussion section here provides some details on the system architecture concepts relevant to KI#1 described in various solutions documented in TR 23.700-14 so far, based on which we propose sensing service system architecture principles for KI#1 interim conclusion.

Observation 1: All the solutions documented in TR 23.700-14 so far addressing key issue 1 propose introducing a new sensing function in the 5G core network responsible for receiving the sensing service request from the sensing service consumer and carry out the actions required to generate the sensing result and send a response to the sensing service consumer.

Proposal 1: To agree the following for KI#1 conclusion –

A new core network Sensing Function (SF) is introduced which comprises of two logical entities – Sensing Service Management Function (SSMF) and Sensing Processing Function (SPF). The two functions may be co-located or deployed as two separate functional entities.

Observation 2: Most of the solutions documented in TR 23.700-14 propose some functional split as logical entities to handle sensing control and processing function in the 5GC though the naming for these logical entities is different in these solutions.

Solution 3, 24, 8, 31, 19,23, 27, 34 propose the functional split as sensing control/service management function and sensing processing function. Solution 5 proposes enhancement to AMF for SE selection and sensing data transfer. Solution 6,33 propose distributed SF which generates sensing result and gateway sensing function for sensing request authorization and SE capability registration with Gateway function.

Solution 9,35 introduces sensing service function receiving sensing service request, authorizing sensing service request, discovering sensing entities and configuring sensing parameters, receiving sensing data, deriving sensing results and then providing results to AF and also introduces sensing profile function that stores the AF authorization data which will be used by sensing service function to authorize AF before accepting any sensing service request.

Solution 13, 22 introduces SeMF performs the discovery and selection of the AMF, Sensing Entity(ies) based on the sensing service request, subscribes/requests the sensing entity information from the AMF for sensing entity discovery and selection. The AMF maintains the SE information.

Based on observation 2, we propose the following for agreement in KI#1 conclusion:

Proposal 2:

The function of SSMF include:

- receiving the sensing service request and authorization of the sensing service request,

- discovery and selection of the sensing entities (SE) and sensing processing functions corresponding to the sensing service request,

- sending the configuration parameters to the SE required for sensing data collection,

- send the sending result generated to the sensing service consumer.

Observation 3: Irrespective of whether a solution proposes a single network function for both sensing service control/management plus sensing data collection and processing (Solution 1, 6, 13) or two separate network functions for the functional split (Solution 24, 18, 19, 23, 26, 34) most solutions propose a data tunnel setup between the SE and this new network function in the 5G core network.

Proposal 3:

- The function of SDCF include:

- sensing data collection from the SE including setup of the data connection required for sensing data collection

- sensing data processing of the data collection from the SE as configured by the SSMF

# 2. Text Proposal

It is proposed to capture the following changes in TR 23.700-14.

# * * * * First change (all text new)* * * *

# 7	Interim agreements

## 7.1	Agreed Principles

### 7.1.Y	Agreed Principles for KI#1

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#Y. This may be populated directly or e.g. also when a topic in clause 7.2.Y gets resolved and a principle is agreed.

- A new core network Sensing Function (SF) is introduced which comprises of two logical entities – Sensing Service Management Function (SSMF) and Sensing Data Control Function (SDCF). The two functions may be co-located or deployed as two separate functional entities.

- The function of SSMF include:

- receiving the sensing service request and authorization of the sensing service request,

- discovery and selection of the sensing entities (SE) and sensing processing functions corresponding to the sensing service request,

- sending the configuration parameters to the SE required for sensing data collection,

- send the sending result generated to the sensing service consumer.

NOTE 1: The details on sensing request parameters required for authorization of the sensing service request and the corresponding principles will be addressed in KI#2 conclusion.

NOTE 2: The parameters required for the discovery and selection of SE, SPF and the corresponding solution principles will be addressed in KI#3 conclusion.

NOTE 3: The configuration parameters sent from the SSMF to the selected SE and the principles on how the configuration parameters are sent to the SE will be addressed in KI#6 conclusion.

- The function of SDCF include:

- sensing data collection from the SE including setup of the data connection required for sensing data collection

- sensing data processing of the data collection from the SE as configured by the SSMF

NOTE 4: The details on the parameters required for sensing data tunnel setup and the corresponding principles will be addressed in KI#4 conclusion.

## 7.2	Topics for further consideration

### 7.2.Z	Topics for further consideration for KI#1

Editor's note:	This clause will include the topics for further consideration as work progresses for the specific KI#Z. Eventually this clause should only contain topics for further consideration that did not result in agreements (i.e. in agreed principle(s) in a clause 7.1.Z) and can either be then marked as not pursued or postponed to a future Release.

Based on the evaluation of solutions (39 solutions) documented in TR 23.700-14 so far, the following topics are to be considered further either to be agreed in clause 7.1 or to be concluded that the topic will be considered in a future release since the topic presented below has architectural impact on the baseline principle to be considered for normative work.

- Few solutions (Solution 6, 9, 19) propose a new network function (Solution 6 – Gateway sensing function, Solution 9 – Sensing profile function, Solution 19 – Sensing policy and authorization function) responsible for sensing service request authorization. It is to be evaluated further whether a new network function is required to fulfil this role or whether SSMF (sensing control and management function can fulfil the role of authorizing the sensing service request).

- Solution 6, 24 propose based on the sensing service area multiple SSMF maybe involved to cater to sensing service request. It is to be evaluated further whether a single SSMF or multiple SSMF in the 5GC is considered in this release.

- Solution 23 proposes distributed sensing processing functions based on specialized processing capability supported by the SePF. The SeMF discovers, selects the SePF based on the processing capability supported and sends the processing configuration request to the selected SePF. It is to be evaluated further whether multiple sensing processing functions based on specific processing capabilities will be supported in this release of the specification.

- Solution 26, 31 propose storage of sensing result in 5GC. Solution 26 introduces SDMF (Sensing data management function) and solution 31 introduces Sensing result storage NF to store the sensing result. It is to be evaluated further whether storage of sensing result in the 5GC will be supported in this release of the

# * * * * End of changes * * * *