3GPP TSG-WG SA2 Meeting #171	S2-2508574

Wuhan, CN, 13th Oct – 17th Oct, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	[WT#6, Computing] WT and KI for 6G computing

Document for:	Approval

Agenda Item:	20.6.6

Work Item / Release:	FS_6G_ARC / Rel-20

Abstract: It is proposed to discuss on the gap for existing SA6 EDGE mechanism and add related WT and KI.

1. Introduction/Discussion

During the discussion last meeting, there is one concern that the computing service can be supported by SA6 current mechanism (e.g., as described in TS 23.558).

How the SA6 design the EAS discovery mechanism.

The current architecture for enabling Edge Applications in SA6 is as Fig 1.

Fig 1

As described in TS 23.558 clause 8.5, the EAS is discovered by the EES considering application layer information (service KPI, E2E response time, Edge/EAS load) and DNAI.

Potential issue:

Issue 1: As discussed in S2-2506995, graceful coordination of communication and computing, service continuity and QoS aspects dynamically is an important aspect. Currently, the network communication performance (e.g., Uu, N3, N6) is not considered into the EAS determination by the edge enabler layer mechanism in SA6.Thus this is no way to allow the dynamic coordination between the communication and computing when issues occur in either communication or computing part. Thus, it is proposed to add a note to describe the potential coordination with QoS Framework and User Plane.

Observation 1: The architecture for enabling Edge Applications in SA6 does not consider dynamic coordination of communication and computing for computing site (re)discovery, either.

Issue 2: Furthermore, the EAS is for a specific application based on SA6’s definition. However, the computing service can be application agnostic depending on the computing resources exposed by MNO. Following are the potential use cases:

Use case 1: Infrastructure as computing resource

In this case, the infrastructure (e.g., Hardware&Firmware) in the computing site is exposed to UE as computing resource. As shown in Figure 2-1, the UE sends Firmware Layer instructions (e.g. that in GPU instruction set) to the computing site and the computing site responds with calculation results. This kind of offloading is application agnostic, i.e. the application does not know whether the computing task for itself has been offloaded to the computing site. The interaction between UE and the computing site is equivalent to access to the hardware (e.g., CPU and GPU) or firmware resources in UE locally. The calculation of one frame is usually started after the previous one. Thus, to support such case, the RTT delay should support to finish the next frame in time., e.g., be less than 16.6 ms for 60 FPS stream.

Figure 2-1

Use case 2: Platform as computing resource

In this case, platform is exposed to UE as computing resource as shown in Figure 2-2. For example, the game engine is deployed in the computing site. To support this use case, the computing site needs to have the execution environment (both applications enable software and base software) to execute the code. This kind of offload is also application agnostic.

Figure 2-2

Use case 3: Application as computing resource with dynamic installation

In this use case, application is exposed to the UE as computing resource while the APP server is installed on the computing site based on request from the UE as shown in Figure 2-3. This kind of offloading is application aware.

Figure 2-3

Use case 4: Application as computing resource with pre-installation

In this use case, the APP server is pre-installed in the computing site in advance as shown in Figure 2-4. It can be the whole functionality of the APP or just part of the APP (e.g., only an inference task). This kind of offloading is application aware.

Figure 2-4

For all use cases, the UE/AF may request computing offloading for a new or ongoing application sessions. The fundamental principle of SA6/SA5 mechanism is to handle the “application server” (in client/server mode communication) in the application layer. However, the computing site may not be application layer aware,  the SA5/SA6’s application layer mechanism does not cover all the above cases either.

Observation 2: the SA5/SA6’s application layer mechanism does not cover all the above cases.

Issue 3: The EAS is located in Data Network, i.e. the computing resource is not controlled by the Core Network NF (e.g., EES cannot get Subscription Data of the subscriber and cannot have better coordination with other CN NF/functionalities). However, if computing resource is located in the CN, the CN can have a central control for both transmission resource and computing resource which has more flexibility to make proper decision of transmission node and computing site selection and allocate the transmission resource and computing resource jointly. Therefore, it is worthwhile to study the computing service where the computing resource is controlled by the CN and study how to do coordination with other Core Network NF/functionalities. Such mechanism is different from the architecture for enabling Edge Applications in SA6.

Observation 3: The architecture for enabling Edge Applications in SA6 cannot have a central control to coordinate the transmission resource and computing resource based on the coordination with other Core Network NF/functionalities easily.

Proposal:

Based on the above Observations, similar like SA2 EDGE design, the SA6 mechanism does not consider the all the uses cases and dynamic coordination of communication and computing, either. And it has some drawbacks to support them (Observation 3). So similar as 5G, there is no limitation that only one WG can study EDGE. SA2 and SA6 can start their work together to find solutions based their own ToRs.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.801-01.

# * * * * First change * * * *All new

Annex A.X. WT#6 Scope

WT#6: Study aspects on support of computing for UE, core network and application server in 6G (e.g. coordination between UE, core network and application server, exposure of computing service in the core network, etc.).

NOTE 1: Application layer mechanism and exposure framework may require coordination with SA6.

In order to support computing, the following aspects need to be studied:

WT#6.1: Enablement/authorization of computing service for UE or AF.

  - Whether and how to enable UE/AF to request computing service.

  - Whether and how to authorize UE/AF request for computing service.

WT#6.2: Dynamic coordination of communication and computing, service continuity and QoS aspects:

- Whether and how to coordinate communication and computing resource(e.g., within the core network by enhancing the existing NFs or introducing a new NF, outside the core network by operator or a 3rd party owned AF).

- Whether and how to identify and/or collect the computing resource related information(e.g., computing resource type and/or status, location of the computing resource, etc.).

- Whether and how to expose the computing resource information and/or network metrics.

NOTE 2:	 Potential SA2 defined exposure of network metrics/computing resource related information will follow the exposure framework defined in WT#1.2. WT#6.2 needs to coordinate with WT1.2 on the QoS Framework and User Plane.

- Whether and how to improve the service experience/QoS(e.g., by defining new metrics or enhancing the existing metrics for QoS).

- Whether and how to support service continuity for computing service upon change of computing site and user plane function (e.g., due to UE mobility, computing load balancing, AF influence, UE request etc.).

WT#6.3: Discovery and (re-)selection of compute site(s)/Application Server(s) for UE/AF requested service:

- Whether and how to discover and (re-)select computing site(s)/Application Server(s) for UE/AF requested service.

- Whether and how to re-use and/or enhance Edge Computing features specified in 5G (i.e., EAS discovery procedure specified in TS 23.548 can be used as starting point for discussion) to enable discovery or (re-)selection of computing site/Application Server providing services to UE/AF.

NOTE 3:  The outcome from SA6 study (if any) can be taken into consideration as an input to this study. The application layer architecture for enabling computing service is in the scope of SA6.

.

NOTE 4: Potential gaps on 5G Edge Computing design will be identified in study phase.

# * * * * Second change * * * * All new

5.X. Key Issue #X: Support for Computing service

It is proposed to study aspects on support of computing for UE, core network and application server in 6G (e.g. coordination between UE, core network and application server, exposure of computing service in the core network, etc.).

NOTE 1: Application layer mechanism and exposure framework may require coordination with SA6.

In order to support computing, the following aspects need to be studied:

- Enablement/authorization of computing service for UE or AF.

  - Whether and how to enable UE/AF to request computing service.

  - Whether and how to authorize UE/AF request for computing service.

- Coordination of communication and computing, service continuity and QoS aspects:

  - Whether and how to coordinate communication and computing resource(e.g., within the core network by enhancing the existing NFs or introducing a new NF, outside the core network by operator or a 3rd party owned AF).

  - Whether and how to identify and/or collect the computing resource related information(e.g., computing resource type and/or status, location of the computing resource, etc.).

  - Whether and how to expose the computing resource information and/or network metrics.

NOTE 2:	 Potential SA2 defined exposure of network metrics/computing resource related information will follow the exposure framework defined in WT#1.2. This needs to coordinate with WT1.2 on the QoS Framework and User Plane.

  - Whether and how to improve the service experience/QoS(e.g., by defining new metrics or enhancing the existing metrics for QoS).

  - Whether and how to support service continuity for, computing service upon change of computing site and/or user plane function (e.g., due to UE mobility, computing load balancing, AF influence, etc.).

3. Discovery and (re-)selection of compute site(s)/Application Server(s) for UE/AF requested service:

  - Whether and how to discover and (re-)select computing site(s)/Application Server(s) for UE/AF requested service.

  - Whether and how to re-use and/or enhance Edge Computing features specified in 5G (i.e., EAS discovery procedure specified in TS 23.548 can be used as starting point for discussion) to enable discovery or (re-)selection of computing site/Application Server providing services to UE/AF.

NOTE 3: The outcome from SA6 study (if any) can be taken into consideration as an input to this study. The application layer architecture for enabling computing service is in the scope of SA6.

NOTE 4: Potential gaps on 5G Edge Computing design will be identified in study phase.

# * * * * End of changes * * * *