3GPP TSG-WG SA2 Meeting #171	S2-2508623

# Wuhan, China, 13th – 17th October, 2025		(revision of S2-2507217)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 1575 | rev | 1 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of PCF configuration for SDF data rate limitation exposure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2 |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The current description for SDF data rate limitation exposure does not cover scenarios where the authorized Session-AMBR is smaller than the authorized maximum bitrate for the SDF. This is however necessary because bitrate control/enforcement for SDF and Session-AMBR are described independently from each other. There is no statement in stage 2 or stage 3 specifications which requires any alignment or dependency between them. Also the enforcement in the UPF is done independently, i.e. there is a separate QER for each of the two and these QERs are applied one after the other (with the SDF related QER being applied first). For the exposure of the Data Rate Limitation Information for the nNon-GBR service data flow it is however required to consider both limitations. Otherwise, the AF would be informed about a bitrate value that can be extremely different from the achievable bitrate. This is especially relevant for situations that lead to a drastical reduction of the authorized Session-AMBR, e.g. when the monthly traffic volume of the subscriber’s contract is reached and the Session-AMBR is drastically reduced unless the subscriber asks for an additional traffic volume allowance. There are basically two options to ensure that the exposed Data Rate Limitation Information for the nNon-GBR service data flow considers the currently authorized Session-AMBR as well: 1) The PCF logic for this exposure is configured appropriately. 2) The PCF is keeping the authorized maximum bitrate values for the SDFs in alignment with the authorized Session-AMBR, so that their values are always lower (or at most equal). While option 1 is only impacting the PCF internal logic, option 2 is also requiring PCC rule and QER modifications in order to update the UPF with the reduced authorized maximum bitrate for the SDFs. And in the case that the subscriber asks for an additional traffic volume allowance short after the reduction, another PCC rule and QER modification would be required to return to the previous values (as the Session-AMBR is now back to its previous value as well). Based on the above considerations, we therefore propose to add a note to suggest that the implementation/configuration of the PCF logic for the exposure is covering these scenarios where the authorized Session-AMBR is smaller than the authorized maximum bitrate for the SDF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A note is added about the PCF configuration for specific scenarios. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Risk of PCF implementations which keep the authorized maximum bitrate values for the SDFs in alignment with the authorized Session-AMBR. This could result in extensive signaling efforts from PCF via SMF to UPF for repeated PCC rule and QER modifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.27.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 6.1.3.27.1	Exposure of network information

For support of real-time media codec/traffic adaptation to the network conditions, the AF may subscribe for exposure of 5GS network information.

When the AF requests the network to provide QoS for the AF session (as described in clause 6.1.3.22), the AF may also provide the subscription to the following information based on QoS monitoring (as described in clause 5.37.4 of TS 23.501 [2]): the UL and/or DL congestion information, UL and/or DL packet delay, the round-trip delay over one or two service data flows, the UL and/or DL Available Bitrate for the service data flow, the UL and/or DL data rate for the service data flow as described in clause 6.1.3.21. The PCF may generatee QoS Monitoring policies for the above network information in the PCC rule of the service data flow as described in clause 6.1.3.21.

If the AF provides an explicit indication (i.e. Indication of ECN marking for L4S support) that the UL and/or DL of the service data flow supports ECN marking for L4S or the PCF decides, based on local configuration, that the service data flow supports ECN marking for L4S, then the PCF may explicitly, or implicitly (based on PCF/SMF local configuration), indicate to the SMF to enable for ECN marking for L4S. The PCF decision may be taken, based on local configuration in PCF and SMF and L4S traffic detection result. If L4S support is detected on the UL and/or DL traffic of the service data flow, the QoS Flow is enabled with ECN marking for L4S, see clause 5.37.3 of TS 23.501 [2].

The PCF shall ensure that only one of QoS monitoring for UL and/or DL congestion information or ECN marking for L4S is activated for the same service data flow (due to the limitation described in clause 5.45.3 of TS 23.501 [2]).

For subscription of UL and/or DL Available Bitrate of a GBR service data flow, the AF also provides a list of thresholds per direction, containing the bitrate values of interest to the AF including one value equal to the Requested Guaranteed Bitrate and one value equal to the Requested Maximum Bitrate (in case it is different). The list of thresholds per direction is to be provided with an increasing order of bitrate values. The PCF then includes the list(s) of thresholds in the QoS Monitoring policy of the corresponding PCC rule (see also clause 5.45.6 of TS 23.501 [2]).

The AF may also provide the value for Averaging Window using AF session with required QoS as described in clause 6.1.3.22.

The AF may subscribe the Data Rate Limitation Information report for a Non-GBR service data flow as described in clauses 6.1.3.18 and 6.1.3.22 (see also clause 5.37.4 of TS 23.501 [2]). The PCF reports the uplink and downlink maximum bitrate authorized for the service data flow to the AF and any change of it.

NOTE:	PCF configuration is expected to ensure that the respective uplink or downlink Authorized Session-AMBR is reported as maximum bitrate authorized for the service data whenever a) uplink or downlink maximum bitrate authorized for the service data flow does not exist or b) the Authorized Session-AMBR is smaller than the maximum bitrate authorized for the service data flow in uplink or downlink direction.

The AF may subscribe the Data Rate Limitation Information report for a PDU Session as described in clause 6.1.3.18 (see also clause 5.37.4 of TS 23.501 [2]). The PCF reports the uplink and downlink Authorized Session-AMBR to the AF and any change of it by means of the Npcf_EventExposure service.

# * * * * End of changes * * * *