SA WG2 Meeting #171	S2-2508689

October 13 – 17, Wuhan, China

Source:	Ericsson, Nokia

Title:	5G VN group membership and SM subscriptions – analysis of CT4 LS C4-253576/S2-2508180

Document for:	Discussion/Agreement

Agenda Item:	4.1

Work Item / Release:	FS_PAIDC_UPF / Rel-19

Abstract of the contribution: This contribution analyses the LS from CT4 on 5G VN group membership aspects.

# Discussion

In the LS from CT4 to SA2 (C4-253576/S2-2508180), CT4 asks four questions:

Q1: Can a UE, that is not member (temporary/permanently) of any 5G VN group, have an SM subscription (in DNN configuration) with an S-NSSAI/DNN that is associated with a 5G VN group?

Q2: Assuming an enterprise deployment with many UEs subscribed to an enterprise specific S-NSSAI/DNN, is it allowed to provision only a subset of these UEs into one 5G VN group with the same aforementioned S-NSSAI/DNN?

Q3: If the answer to Q1 and Q2 is no, before the subset of UEs (of Q2) is added to a 5G VN group, this subset of UEs have no S-NSSAI/DNN subscription for the 5G VN group, does it imply that this subset of UEs needs to be re-configured with the S-NSSAI/DNN associated with 5G VN group when they are added to the 5G VN group by the AF/NEF invoking Nudm_pp service(such that these UEs start using the S-NSSAI/DNN associated with the 5G VN Group when they establish PDU sessions)?

Q4: If the answer to Q3 is yes, how are the UEs re-configured?

Q1 and Q2 are related to a potential scenario where:

- 	some UEs are members of a 5G VN group that is associated to a DNN/S-NSSAI,

- 	while other UEs have a regular subscription to that same DNN/S-NSSAI but are not members of the 5G VN group.

Q3 and Q4 ask whether and how UEs can be configured with DNN/S-NSSAI if the answer to Q1/Q2 is a "no".

Below we analyze these two topics.

# Analysis of Q1/Q2

Below we analyze a situation where the UEs can subscribe to a DNN/S-NSSAI but may or may not be member of corresponding 5G VN group.

### Subscription data handling

5G VN groups are managed by an AF by providing 5G VN Group Data (e.g. DNN, S-NSSAI, PDU Session Type etc.) and 5G VN Group membership information (list of GPSIs) to 5GC via NEF. The NEF provides the received information to UDM using Nudm Parameter Provision service, and UDM stores the information in UDR as 5G VN Group Data and Group Identifier data. The 5G VN Group data is then provided to AMF/SMF as Shared Data. Alternatively, if Shared Data is not used (as it is an optional capability in UDM/SMF), UDM can incorporate the corresponding 5G VN information (DNN, S-NSSAI etc) within the SM subscription data to SMF.

Regular subscription data for a DNN/S-NSSAI is managed via OAM and stored in UDR as UE Subscription data, i.e. separate from the Group Data. This information is provided to AMF/SMF as UE subscription data (e.g. SM subscription data with dnnConfigurations).

Similar, PCF has access to Policy Data in UDR for the regular subscription data, as well as access to the 5G VN Group Data for 5G VN groups. The Policy Data includes e.g. the URSP route selection descriptors containing allowed Route Selection Descriptors for URSP with DNN and PDU Session Types.

Observation 1: 5G VN group data for a DNN/S-NSSAI and regular subscription data for a DNN/S-NSSAI are stored in different data fields in UDR and provided in different data structures to AMF, SMF and PCF.

CT4’s Question 2 indicates that a set of UEs have a regular subscription to a DNN/S-NSSAI and only a subset of these UEs are also members of the 5G VN group associated with the same DNN/S-NSSAI. In other words, the group members have both a regular dnnConfiguration subscription data in UDM/UDR (and regular Policy Data in UDR) as well as a pointer to the 5G VN Group data containing the same DNN/S-NSSAI. Several of the parameters in the 5G VN Group Data overlap with the regular subscription data (green and purple parameters in Figure 1)

Figure 1. 5G VN Group Data

Observation 2: Subscribers to a DNN/S-NSSAI that are also members of a 5G VN group have two sets of parameters for a DNN/S-NSSAI in UDM/UDR

How to handle this duplication has not been discussed in SA2 and is not clear. Would UDM send only one profile to SMF/AMF, or would SMF/AMF receive both the regular subscription data as well as 5G VN Group Data for the same DNN/S-NSSAI? In the latter case, there has to be AMF/SMF logic to handle this, e.g. give precedence to one set of parameters, resolve discrepancies in some other way or reject in case of mismatch. Also, how would it work if Shared Data is not used? Would UDM send two dnnConfigurations to SMF in SM subscription data for the same DNN/S-NSSAI? If so, how would SMF know which one refers to the 5G VN group and which one refers to the OAM-configured regular DNN/S-NSSAI subscription? Similar issues apply to PCF where in addition there is the following question: How does PCF apply the maximum Group Data Rate? If it is based on DNN/S-NSSAI then the maximum Group Data Rate would also be shared with non-members of the 5G VN group.

Observation 3: UDM/AMF/SMF/PCF handling of duplicated DNN/S-NSSAI parameters have not been discussed. This would result in unpredictable outcome for an operator since the behavior would depend on implementation.

### UE being added or removed from the 5G VN group

We can distinguish between two cases:

- 	Case 1: A UE is a non-member subscribed to the DNN/S-NSSAI and has a PDU Session to that DNN/S-NSSAI. The UE is then added to the 5G VN group.

- 	Case 2: A UE is a member of the 5G VN group and has a PDU Session to that DNN/S-NSSAI. The UE is then removed to the 5G VN group.

If the UE is added to the 5G VN group (case 1), the UDM will notify AMF/SMF that the subscription data has changed.

- 	The AMF would notify the UE-PCF that the UE is now member of the Internal Group ID for the 5G VN group (if GroupIdListChange feature is supported by AMF/UE-PCF, see TS 29.525, clause 5.6.3.3). The UE-PCF can then get the 5G VN data from UDR and update the URSP rules correspondingly, if needed.

- 	The SMF actions are however not so clear, due to the duplication of subscription data mentioned in observation 3. Assuming that the parameters in the 5G VN group data match the regular subscription data (PDU Session type etc), the SMF may be able to maintain the PDU Session and possibly activate e.g. 5G VN group communication. The SMF can also notify the SM-PCF about the new Internal Group ID for the UE (if GroupIdListChange feature is supported by SMF/SM-PCF, see 29.512, clause 5.6.3.6).

- 	The SM-PCF to start applying the Group Data Rate for the 5G VN group.

Another option which is simpler for implementation is that SMF triggers a release of the PDU Session and request re-activation. This allows SMF, PCF etc to apply to the 5G VN group data for the new PDU Session.

Similar actions and issues apply if the UE is removed from the group (case 2).

Observation 4: The handling of the issues described in observation 2/3 requires further discussion and are not taken into account by the current 23.501/23.502.

# Analysis of Q3/Q4

Q3/Q4 apply if the answer to Q1/Q2 is a "no".

If the answer to Q1/Q2 is "no", the assumption is that a DNN/S-NSSAI is dedicated to a 5G VN group. I.e. there are no subscribers to that DNN/S-NSSAI that are non-members.

Q3/Q4 asks how the UE can be configured with the DNN and S-NSSAI when they are added to a 5G VN group.

In our understanding this is handled based on already defined mechanisms:

If the UE is added to the 5G VN group for a new DNN/S-NSSAI that the UE is not yet subscribed to, the UDM will notify AMF/SMF that the group membership has changed and will provide the 5G VN group data for that UE to AMF/SMF. The UE would thus become a member of a 5G VN group with a new DNN/S-NSSAI.

- 	The AMF would then update the UE with new Configured NSSAI and Allowed NSSAI containing the new S-NSSAI.

- 	The AMF also notifies the UE-PCF that the UE is now member of the Internal Group ID for the 5G VN group (if GroupIdListChange feature is supported by AMF/UE-PCF, see TS 29.525, clause 5.6.3.3) or the UE-PCF may be notified by the UDR as described in TS 23.502, clause 4.16.12.2. The UE-PCF can then get the 5G VN data from UDR and update the URSP rules correspondingly.

- 	After this, the UE would have an updated Configured/Allowed NSSAI as well as updated URSP rules for the new S-NSSAI and DNN. This allows the UE to request a new PDU Session for the DNN/S-NSSAI corresponding to the 5G VN group.

- 	When the new PDU Session is established, the SMF and SM-PCF will apply the 5G VN group data. There is no issue with a partly overlapping UE subscription data for the same DNN/S-NSSAI.

If the UE is removed from the 5G VN group, the UDM will notify AMF/SMF that the group membership has changed. In this case the UE is no longer subscribed to the DNN/S-NSSAI and the SMF will trigger a release of the PDU Session.

Observation 5: The issues brought up in Q3/Q4 can be handled based on existing standards.

# Conclusion and Proposal

In our understanding the scenario mentioned by CT4 in Q1/Q2 has not been discussed in SA2 so far. There are unclarities on how a duplication of dnnConfiguration (UE subscription data) and 5G VN group data is handled for the same DNN/S-NSSAI. Therefore, it does not seem possible to reply "yes" to Q1/Q2 unless further study work is performed by SA2.

At the same time, if the answer to Q1/Q2 is a "no", it is possible to reply to CT4's Q3/Q4 and describe how existing standards are used to handle a new DNN/S-NSSAI in 5G VN group data.

Conclusions:

-	Further study work is needed by SA2 if there is a desire by companies to reply "yes" to Q1/Q2. Since CT4 asked for a reply during SA2#171, it therefore seems appropriate to answer "no".

Proposal:

- 	It is proposed to answer Q1/Q2 with a "no" and that SA2 provides an answer to Q3/Q4 in line with the discussion above.

- 	A corresponding LS reply is available in S2-2508909.