SA WG2 Meeting #170	S2-2508707

13th – 17th October, 2025, Wuhan, China

Source:	ETRI

Title:	KI#1: interim agreement update

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract of the contribution: The contribution proposes updates on Agreed principles for KI#1 of TR 23.700-04.

1. Introduction

Some basic interim agreements are captured in the last meeting.

This paper proposes to add further principles to the current agreed interim principles.

The proposed principles detail the DCF's role, from receiving the subscription from UMTES (UE-Model Training Entity Server), selecting UEs, providing UP information (e.g., IP address or FQDN) over NAS, managing the data transfer request, verifying the received data, and finally exposing the collected UE data to the UMTES.

2. Proposal

It is proposed to agree the following updates to TR 23.700-04.

*** Start of Change ***

7.1	Agreed Principles

7.1.1	Agreed Principles for KI#1

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#1. This may be populated directly or e.g. also when a topic in clause 7.2.1 gets resolved and a principle is agreed.

The interim agreements on principles for KI#1 are as follows:

-	A UP path is used between the UE and a 5GC NF, for transferring standardized collected data from the UE using PDU connectivity service provided by a PDU session.

-	Dedicated S-NSSAI/DNN is used to differentiate the traffic of collected UE data from the UE regular traffic per PDU Session level.

-	The UE establishes a new PDU Session, or modifies an existing PDU Session which has been established for UE data transfer, to transfer the standardized UE data, based on URSP rules.

-	DCF receives UE data collection subscription from UE-model training entity server (UMTES) including UE selection criteria and data context to be collected.

-	DCF selects UEs with data collection capability for the purpose of UE data collection.

-	DCF provides the UE with its own UP information (e.g., IP address or FQDN) over NAS for the subsequent UP data transfer.

-	DCF sends UE data transfer request to a UE over NAS or over UP including data collection parameters.

-	DCF receives the collected data from UEs over UP.

-	DCF verifies the received data from UEs.

-	DCF exposes the collected UE data to UMTES.

*** End of Change ***