3GPP TSG-SA2 Meeting #171	S2-2508730

# Wuhan, China, 13th Oct 2025 - 17th Oct 2025										 was S2-2506795

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1505 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifying Signalling Storm Analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SK Telecom, Huawei, HiSilicon, Oracle |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-10-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There are some typos and ambiguous description in clause 6.22.2 need to be corrected and clarified: - Table 6.22.2-8 is omitted in the first paragraph. - The former items in Table 6.22.2-8 are NF service granularity, but item#3 is similar with item#4. It needs to be removed. - The former items in Table 6.22.2-8 are statistics on the number of messages received by SCP. The description of the former items is ambiguous and needs to be clarified. - The capacity and priority value in the analytics output could be different for different analytics request/subscription, similar as NRF can provide different priority and capacity values in the NF discovery responses to avoid congestion oscillation problem. - Figure 6.22.4-1 has red colored (AF/OAM) and also step 2 added AF/OAM for consistency. There are some other typos and clarifications to be resolved, e.g., the NF in the analytics filter is ususally the NFs who sending messages to the NF that to be protected or the target of the analytics. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Add Table 6.22.2-8 in the first paragraph. 2. Remove item#3 from Table 6.22.2-8. 3. Update the description of the former items in Table 6.22.2-8. 4. Clarify the analytics target and filter information. 5. Clarify the capacity and priority value. 6. Procedure color changed from red to black and fixed small typos. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The input data of Signaling storm analytics and potential NF actions are unclear and confusing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.22.1, 6.22.2, 6.22.3, 6.22.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

6.22	Signalling Storm Analytics

6.22.1	General

This clause specifies how NWDAF supports analytics for network abnormal behaviours (i.e. signalling storm) mitigation and prevention.

The signalling storm analytics provides analytics information (statistics or predictions) regarding the abnormal signalling rate condition. An abnormal signalling rate condition is when signalling rate is experiencing an abnormal growth. The abnormal signalling growth may indicate a potential signalling storm due to a significant deviations from the signalling level in the normal conditions. The analytics of signalling storm provides the identification of whether the signalling storm is due to massive signalling from NF(s) or  from UEs based on the analytics consumer request.

The NWDAF provides statistics and/or prediction analytics to consumer entity (e.g. SCP) or consumer NFs, e.g. AMF, SMF, UDM, PCF, NRF, OAM and AF.

NOTE 1:	When the consumer is SCP, the target NF(s) (i.e. The NF that signalling storm analytics should be provided for) might be deployed either in front of the SCP (egress) or behind the SCP (ingress).

The consumer of these analytics may indicate in the request or subscription:

-	Analytics ID = "Signalling Storm";

-	Target of Analytics Reporting: a list of NF instance ID(s) or NF Set ID(s) which may suffer signalling storm or any NF if the target is the whole network;

-	Target Cause ID(s): Indicates specific Cause ID(s) as Analytics Output (e.g. signalling storm caused by UEs, signalling storm caused by NF, or all);

-	Analytics Filter Information:

-	NF ID List: instance IDs or NF Set IDs of NF(s) which may cause signalling storm to the target NF (e.g. NF service consumer(s) of the NF(s) indicated in the Target of Analytics Reporting);

-	a group of UEs or several groups of UEs identified by internal Group ID(s) or a SUPI list which may cause signalling storm to the Target of the Analytics Reporting;

-	Area of Interest (AoI) and/or S-NSSAI which restricts the scope of signalling storm analytics to the specific area and/or slice.

-	Analytics Reporting Information:

-	Expected Report Time: indicates the time limit by which the analytics results need to be received, e.g. 10 seconds, 5 minutes, etc.;

-	An Analytics target period indicates the time period over which the statistics or predictions are requested;

-	Optionally, preferred level of accuracy of the analytics;

-	Signalling frequency threshold: indicates the frequency of the received signalling within the time period;

-	UE number threshold: indicates a threshold of the amount of UEs of which the number of one type of request messages meets the frequency of UE requests threshold. For example, if the number of UEs that initiate registration requests exceeds the pre-set rate threshold (10 times per hour) and the number of such UEs exceeding the pre-set rate threshold exceeds 100 (the UE number threshold is set to 100), the NWDAF will provide the analytics.

6.22.2	Input data

The NWDAF collects the UE related context data, NF context, AF data, MDAF data and SCP signaling information as listed in Tables 6.22.2-1, 6.22.2-2, 6.22.2-3, 6.22.2-4, 6.22.2-5, 6.22.2-6, 6.22.2-7 and 6.22.2-8.

The NWDAF can collect UE behavioural information per UE or group of UEs as specified in clause 6.7.5.2.

The NWDAF may use output of abnormal UE behaviour analytics and UE dispersion analytics as specified in clause 6.7.5 and clause 6.10.3 as input data for analytics on signalling storm. This information may be collected from a different NWDAF (e.g. distributed architecture).

The NWDAF may collect a subset of input data based on implementation.

Table 6.22.2-1: UE related Context Data collection

| Information | Source | Description |
| --- | --- | --- |
| UE ID |  | Identifies a single UE. |
| Signalling feature data |  | NF procedures containing signalling exchange information related to a particular UE or session from the Connection, Registration, Mobility and Session Managements procedures (NOTE 1). |
| > Request type and number from UE/RAN (0..max) | AMF | Request type of N1 or N2 interface (such as received Initial Registration Request, Mobility and Periodic Registration Request, Service Request, etc.) and the number of requests corresponding to the request type. |
| >> Time duration from receiving request from UE/RAN to response to UE/RAN | AMF | Time duration between the request from UE/RAN and response to UE/RAN. |
| >> Number of successful responses of UE/RAN | AMF | Number of successful responses associated to their initial requests, such as Registration Response, etc. |
| >> Number of failed responses of UE/RAN | AMF | Number of failed responses associated to their initial requests, such as Registration Reject, Service Reject, etc. |
| >> Reason of failed responses of UE/RAN | AMF | Reasons of failed responses associated to their initial requests, e.g. reject, no-response, etc. |
| >> A posterior Request type of UE/RAN (0..max) | AMF | A posterior Request types triggered from UE/RAN, for NF Service request, or request to UE/RAN (NOTE 4). |
| > Request type and number from NF (0..max) | NF | Request type (i.e. identification of NF service instance) received from NF, e.g. Namf_N1N2Trans, Namf_comm, etc., as well as the number of requests received from NF (NOTE 2) (NOTE 3). |
| >> Time duration from sending a request to receiving NF to response from the NF | NF | Time duration between the request from an NF and response to the NF. |
| >> Number of successful responses from NF | NF | Number of successful responses associated to their initial requests (NOTE 2). |
| >> Number of failed responses from NF | NF | Number of failed responses associated to their initial requests (NOTE 2). |
| >> Reason of failed responses from NF | NF | Reasons of failed responses associated to their initial requests, e.g. reject, no-response, etc. |
| >> Number of redundant signalling of NF | NF | Number of received redundant signalling. The redundant signalling means the signalling which is transmitted multiple times (NOTE 2) (NOTE 3). |
| >> A posterior Request type of NF (0..max) | NF | A posterior Request types triggered from NF, for NF Service request (NOTE 3) (NOTE 4). |
| > Public Warning information | AMF | Public Warning information as defined in the TS 23.041 [52], such as WRITE-REPLACE WARNING REQUEST, etc. |
| > Frequent Mobility Registration Update | AMF | The number of Mobility Registration Updates N within a period M may be an indication for abnormal ping-pong behaviour, where N and M are operator's configurable parameters. |
| > Number of receiving Session Report from UPFs | SMF | Number of receive Session Report from UPF triggered by DL packet in case of PDU Session is in 5GCM-idle state. |
| UE Context in NF |  |  |
| > State transition information | AMF, SMF | UE related state transition information such as transition type, frequency of CM state changes, etc. State transition identifier: - "Access Type change to 3GPP access"; - "Access Type change to non-3GPP access"; - "RM state change to RM-DEREGISTERED"; - "RM state change to RM-REGISTERED "; - "CM state change to CM-IDLE"; - "CM state change to CM-CONNECTED"; - "Handover"; - "Mobility Registration Update"; or - "Frequent Mobility Registration Update"(table 6.7.2.2-1). Or, PDU Session related state transition information such as transition type, frequency SM state changes, etc. State transition identifier: - "PDU Session Establishment"; - "PDU Session Release"; - "Communication failure"; or - "PLMN change". It can be reported for a group of UEs (e.g. the number of total transitions or percentage of the group UEs who have transitions). |
| > timer information | AMF, SMF | Timer information which has been set for the UE, such as timer type, duration. |
| NOTE 1: NWDAF can optionally provide transaction dispersion analytic information for MM and SM transactions. NOTE 2: This data is provided per time interval that is indicated by the NWDAF. NOTE 3: As a centric routing/proxy entity, SCP might be involved with some signalling that may not be visible to the NF sending the SCP's ingress traffic, such a signalling could be associated with actions such as selection & discovery, retransmissions, re-directions, re-selection, throttling, etc. NOTE 4: A posterior request is a request sent from NF1 to NF2 (or from NF2 to NF1) that was triggered by a previous request received by NF1. For example, in clause 4.3.2.2 of TS 23.502, the request sent by AMF in step 3 and the request sent by SMF in step 11 are both posterior requests of the request received by the AMF in step 1. These posterior requests can be collected by NF1 (the AMF in this example). |  |  |

Table 6.22.2-2: NF Context Data collection

| Information | Source | Description |
| --- | --- | --- |
| NF instance ID |  | NF instance ID |
| > NF profile | NRF | NF Profile information such as allowed NF information for the NF and NF Service(s), NF capacity, NF load, NF priority, NF service instance capacity, NF service instance load, NF service instance priority. |
| > NF load information | NRF | Load information indicates the current load of the NF and NF Service(s) (NOTE 1). |
| NF heart-beat related information | NF, OAM | NF heart-beat related information such as responding time, Number of retransmissions, heart-beat intervals. |
| > Unexpected operational status indicator | NF, OAM | The parameter indicates unexpected operational status occurs. It is an indication of deviation from the normal operations, based on thresholds or rules configured by operator at the NF or OAM (e.g. for energy consumptions, loads, etc.). |
| NOTE 1: If this information is not available in the NRF, it can be obtained from the OAM. |  |  |

Table 6.22.2-3: NF Specific Data collection

| Information | Source | Description |
| --- | --- | --- |
| NF instance ID |  | NF instance ID |
| > Usage information of UE IP address resources | SMF | Usage information of UE IP address resources (dynamic and static, V4, V6, etc.) for CP or UP allocation, such as number, usage, number of UE IPs, which prohibit allocation during certain time interval, etc. This parameter is valid for SMF only. |
| > Load information of connected UPFs | SMF | Load information of connected UPFs such as using PFCP Load Control Information. This parameter is valid for SMF only. |

Table 6.22.2-4: Application activation time information

| Information | Source | Description |
| --- | --- | --- |
| Application ID | AF | Identifies the application providing this information. |
| User activation time information (1…max) | AF | Information of activation time for the users (e.g. IoT users) per application. |
| > Number of UEs | AF | The total number of UEs |
| > Active Time | AF | The time stamp of the users per application switch to active, or the start and end time of the users activity per application. |
| > Inactive Time | AF | The time stamp of the users per application switch to inactive, or the start and end time of the users inactivity per application, if applicable. |
| > UE type ID | AF | Identifies a group of UEs, e.g. external group ID, or a list of UE IDs. |

Table 6.22.2-5: Data collection from OAM related to NRF operations as defined in clause 5.10 of TS 28.552 [8]

| Information | Source | Description |
| --- | --- | --- |
| NRF ID |  | NRF instance ID. |
| Number of NF service registration requests | OAM | Number of registration request received at the NRF. |
| > Number of successful NF service registrations | OAM | Number of successful registrations. |
| > Number of failed NF service registrations due to encoding error of NF profile | OAM | Number of failed registrations. |
| > Number of failed NF service registrations due to NRF internal error | OAM | Number of failed registrations. |
| Number of NF service update requests | OAM | Number of update request received at the NRF. |
| > Number of successful NF service updates | OAM | Number of successful updates |
| > Number of failed NF service updates due to encoding error of NF profile | OAM | Number of failed updates. |
| > Number of failed NF service updates due to NRF internal error | OAM | Number of failed updates. |
| Number of NF discovery requests | OAM | Number of discovery request received at the NRF. |
| > Number of successful NF discoveries | OAM | Number of successful discovery attempts. |
| > Number of failed NF service discoveries due to unauthorized NF Service consumer | OAM | Number of failed discovery attempts. |
| > Number of failed NF discoveries due to input errors | OAM | Number of failed discovery attempts. |
| > Number of failed NF discoveries due to NRF internal error | OAM | Number of failed discovery attempts. |

Table 6.22.2-6: Data collection from MDAF/MDAS of control plane congestion analysis as defined in clause 8.4.7.1.3 of TS 28.104 [45]

| Information | Source | Description |
| --- | --- | --- |
| affectedObject | MDAF | Indication of 5GC NFs where congestion issues occurred or potentially may occur. |
| cPCongestionIssueID | MDAF | This field holds the ID of the control plane congestion issue which is reported. |

Table 6.22.2-7: Data collection from MDAF/MDAS of fault information as defined in clause 8.4.3.1.3 of TS 28.104 [45]

| Information | Source | Description |
| --- | --- | --- |
| failurePredictionObject | MDAF | Indication of NFs where the failure related issues occurred or potentially occur. |
| potentialFailureType | MDAF | This field holds the ID of the control plane congestion issue which is reported. |
| eventTime | MDAF | This field holds the time of potential failure predicted |
| issueID | MDAF | This filed holds the ID of this failure prediction which is reported. |
| perceivedSeverity | MDAF | This field holds the value to indicate relative level of urgency for operator attention. |

Table 6.22.2-8: SCP Signalling Infomation

| Information | Source | Description |
| --- | --- | --- |
| NF instance ID |  | NF instance ID of the service producer or consumer. |
| > Number of requests sent to an NF Service instance | SCP | Number of requests received by SCP for an NF service instance, such as Nudm-sdm, Namf-loc, etc. |
| > Number of requests sent by an NF instance | SCP | Number of requests received by the SCP from an NF instance. |
| > Number of successful responses of different NF Service(s) of the NF instance | SCP | Number of successful responses received by SCP from NF service instances, such as Nudm-sdm, Namf-loc, etc. |
| > Number of failed responses of different NF Service of the NF instance | SCP | Number of failed responses received by SCP from different NF Service, such as Nudm-sdm, Namf-loc, etc. |
| > Reason of failed responses of different NF Service of the NF instance | SCP | Reasons of failed responses received by SCP from different NF Service, e.g. reject, no-response, etc. |
| > Number of failure responses SCP generated to the NF instances | SCP | Number of total failure responses generaterd by SCP. |
| > Reason of failure responses SCP generated to the NF instance | SCP | Reasons of failed responses of SCP, e.g. target nf not reachable, no-response, etc. |
| > Number of times SCP reselect NF instance that is originally to that NF instance | SCP | Number of times SCP reselect NF instance, it happens when e.g. SCP finds the original NF instance producer not reachable. |
| > Reason of SCP reselection of NF instance | SCP | Reason of SCP reselecting NF instance producer as defined in clause 6.3.1.0 of TS 23.501 [2]. |
| > Status of the connection to the NF instance | SCP | The connection status (i.e. ACTIVE or INACTIVE) between SCP and NF instance. |
| > Average time duration from sending request to the NF instance to receiving response from the NF instance. | SCP | Average time duration between SCP sending the request to the NF instance and SCP receiving the response from the NF instance. |

6.22.3	Output analytics

The output analytics of signalling storm provided by NWDAF is defined in Table 6.22.3-1, and Table 6.22.3-2. The Table 6.22.3-3 gives examples of NF/SCP actions for solving each risk. The final mitigation or prevention operations are based on operator's policy/configuration and NF/SCP implementation.

The statistics and predictions are provided with a Validity Period, as defined in clause 6.1.3.

Table 6.22.3-1: Signalling storm statistics

| Information | Description |
| --- | --- |
| Report (1..max) | List of observed signalling storm statistics. |
| > Target NF ID/SCP ID | A list of impacted NFs/SCPs of signalling storm detected by NWDAF. |
| > Cause ID of the signalling storm | The potential cause of abnormal level of signalling (i.e. massive signalling from UE or NF abnormal signalling). |
| > Source UE/NF | a UE or a group of UEs or several groups of UEs identified by internal Group ID or SUPI or SUPI range, or a list of NFs which cause the signalling storm. |
| > OPTIONAL Time slot entry (1..max) | List of time slots during the Analytics target period. |
| >> Time slot start | Time slot start within the Analytics target period. |
| >> Duration | Duration of the time slot. If a Temporal granularity size was provided in the request or subscription, the Duration is greater than or equal to the Temporal granularity size. |
| >> Received Signalling Analytics | Information of signalling received by the target NF(s). |
| >>> Total number of received signalling | Indicates the statistics on the number of signalling messages received by the target NF(s) in the time slot. |
| >>> Growth rate of received signalling | Difference between the number of signalling messages received by the NF in the time slot and the number of signalling messages received by the target NF(s) in the previous time slot. |
| >>> Signalling analytics of UE | Indicates the statistics on the number of signalling messages received from UEs within the time slot (NOTE 1). |
| > OPTIONAL Timer List | The list of timer information per source UE(s) (NOTE 1). |
| >> Type of timer | The type of timer which has been set. |
| >> Timer duration | The timer duration that has be selected for the source UE(s). |
| NOTE 1: Only available when Cause ID of signalling storm is massive signalling from UEs, and there exists Source UE(s). |  |

Table 6.22.3-2: Signalling storm predictions

| Information | Description |
| --- | --- |
| Report (1..max) | List of predicted signalling storm analytics. |
| > Target NF ID/SCP ID | A list of impacted NFs/SCPs signalling storm predicted by NWDAF. |
| > Cause ID of the signalling storm | The potential cause of abnormal level of signalling (i.e. massive signalling from UE or NF abnormal signalling). |
| > Source UE/NF | a UE or a group of UEs or several groups of UEs identified by internal Group ID or SUPI or SUPI range, or a list of NFs which cause the signalling storm. |
| > OPTIONAL Time slot entry (1..max) | List of time slots during the Analytics target period. |
| >> Time slot start | Time slot start within the Analytics target period. |
| >> Duration | Duration of the time slot. If a Temporal granularity size was provided in the request or subscription, the Duration is greater than or equal to the Temporal granularity size. |
| >> Reference point | The information of the reference point impacted. (e.g. N1, N2) |
| >> Service operation(s) | The information of the service operation(s) impacted. (e.g. Namf_Communication(UEContextTransfer), Nsmf_PDUSession(UpdateSMContext)). |
| >> Received Signalling Analytics | Information of signalling received by the target NF(s). |
| >>> Received number of signalling | Received number of signalling of the specific signalling type. |
| >>> Growth rate of received signalling | Difference between the number of signalling messages received by the NF in the time slot and the number of signalling messages received by the target NF(s) in the previous time slot. |
| >>> Signalling analytics of UE | Indicates the statistics on the number of signalling messages received from UEs within the time slot (NOTE 1). |
| > OPTIONAL Timer List | The list of timer information per Source UE(s) (NOTE 1). |
| >> Type of timer | The type of timer. |
| >> Timer duration | The timer duration that commonly is selected for the Source UE(s). |
| > Priority | Priority (relative to other NFs of the same type) of candidate NFs as defined in the TS 29.510 [18] (NOTE 2). |
| > Capacity | Candidate NF capacity information, expressed as a weight relative to other NF instances of the same type as defined in the TS 29.510 [18] (NOTE 2). |
| > Confidence | Confidence of this prediction. |
| NOTE 1: Only available when Cause ID of signalling storm is massive signalling from UEs, and there exists Source UE(s). NOTE 2: The Priority and Capacity may have different values for different Analytics subscription/request parameters in the analytics request or subscription. |  |

Table 6.22.3-3: Example mechanisms to mitigate and prevent the signalling storm

| Cause of the signalling storm | Example actions of NFs |
| --- | --- |
| massive signalling from UE | AMF sets MM NAS related timer (e.g. back-off, T3512) for a selected set of UEs. |
| massive signalling from UE | SMF sets SM NAS related timer (e.g. back-off) for a selected set of Sessions. |
| massive signalling from UE | AMF/SMF sets suggested N1/N2 interface related ingress/egress threshold, or AMF triggers RAN to initiate overload control for a selected set of UEs in specific slice or priority as defined in clause 8.7.7 of TS 38.413 [16] to start overload control. The AMF may assign a specific slice to the UEs under signalling storm conditions and initiate overload control for the slice under signalling storm conditions. |
| NF abnormal signalling | NRF configures the local policy (e.g. to prevent the source NF with abnormal signalling from being discovered or discovering others, to update the NF profile(s)(e.g. Priority, Capacity) of NF(s) based on the signalling storm statistics and/or predictions (e.g. Priority, Capacity) and send the updated NF profiles(s) to NF service consumer(s) of the NF(s)(e.g. subscriber of the NF profile as defined in clause 4.17.7 of TS 23.502 [3])). |
| NF abnormal signalling | Source NF configures to (re)select other NFs instead of NF with abnormal signalling and may unsubscribes the NF with abnormal signalling. |
| NF abnormal signalling | Source NF configures to deprioritize the NFs/Services with abnormal signalling from being selected. |
| NF abnormal signalling | Source NF triggers UE Reregistration and/or Session Reestablishments to avoid NF with abnormal signalling. |
| NF abnormal signalling | SCP blocks or throttles messages originating from an NF/service with abnormal signalling. |
| NF abnormal signalling | SCP redirects requests towards an NF service producer with abnormal signalling towards another service producer. |
| NF abnormal signalling | SCP notifies communication peers of an NF that the NF may suffer signalling storm. |

6.22.4	Procedures

The NWDAF can provide information on network signalling storm as follows (Figure 6.22.4-1).

Figure 6.22.4-1: Procedure for NWDAF-assisted Network Signalling Storm Mitigation and Prevention

1.	The consumer NF (SCP included) or other entities subscribes to or sends a request to NWDAF for the signalling storm analytics using either Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request service operation. The request additionally includes thresholds such as confidence level and accuracy of detection:

-	The Analytics ID is set to "Signalling Storm" analytics. The target for analytics reporting is set to be any NF/SCP or any UE. Analytic filters may be provided as shown in clause 6.22.1.

-	The consumer NF can request statistics or/and predictions for a given Analytics target period.

-	The consumer NF may also subscribe to "NF load information" analytics as depicted in clause 6.5 and trigger the signalling storm analytics based on output of the NF load analytics, e.g. CPU usage is over 80%.

-	The consumer NF may also subscribe to "Abnormal behaviour" analytics as depicted in clause 6.7.5 and trigger the signalling storm analytics based on the suspicion of DDoS attack.

-	The consumer NF may also subscribe to "Dispersion Analytics" analytics as depicted in clause 6.10 and trigger the signalling storm analytics when the transaction dispersion is above the expected transaction dispersion configurable threshold.

2.	The NWDAF retrieves Input data from NFs, AFs and SCPs, using Nnf_EventExposure_Subscribe, or from NRF using Nnrf_NFManagement_NFStatusSubscribe service operation, such as UE and/or NF as depicted in Table 6.22.2-1 and Table 6.22.2-2, NF specific data as depicted in Table 6.22.2-3, application data as depicted in Table 6.22.2-4, and collects input data from OAM and MDAF as depicted in Table 6.22.2-5, Table 6.22.2-6 and Table 6.22.2-7 to derive the required analytics as depicted in clause 6.22.2. The NWDAF may optionally request additional data aligned with the analysis target identified in step 1.

If the NWDAF has subscribed to receive analysis data from MDAF, NWDAF uses the analysis output from MDAF to determine source NFs to retrieve input data for generating signalling storm analytics.

The NWDAF may optionally subscribe the abnormal UE behaviour analytics and UE dispersion analytics as specified in clause 6.7.5 and 6.10.3 as input data from other NWDAFs, using Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request service operation.

3.	The NWDAF derives the required analytics with the collected data described in clause 6.22.2.

4.	The NWDAF invokes Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalyticsInfo_Request response or response to the consumer NF/SCP for the Output data analytics as depicted in clause 6.22.3.

5.	The consumer upon receiving the detection and/or prediction and/or mitigation may execute based on the example mechanisms as depicted in clause 6.22.3.

* * * End of Changes * * *