3GPP TSG-WG2 Meeting #171	S2-2508855

October 13 – 17, 2025, Wuhan, China		(revision of S2-250xxxx)

Source:	Ericsson, AT&T

Title:	Interim agreement for KI#2

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: This contribution provides agreed principles and topics for further consideration for KI#2

# 1. Discussion

This document proposes interim agreements for KI#2.

# 2. Text Proposal

It is proposed to agree the following principles to 3GPP TR 23.700-14.

>>>> Start of Changes <<<<

### 7.1.2	Agreed Principles for KI#2 on Authorization and Revocation to Support Sensing Service

In order for a 3GPP network to perform authorization of a requested sensing service, the following main principles apply:

-	NEF and SnCF – Sensing Core Function may be involved to perform authorization of the sensing service requested by a sensing service consumer (e.g., AF) from the network.

NOTE X1:	It is assumed that a sensing service requester and a sensing service consumer is the same entity.

-	In case a sensing service consumer is a third-party AF, the general NEF framework is used to authorize the AF by determining whether the AF is allowed to invoke the sensing-specific APIs. For the determination, the NEF uses an AF Identifier that the sensing service consumer shall include in the sensing service request to the network.

-	If the NEF grants the authorization to the AF, the NEF invokes the corresponding sensing service operation to the SnCF. The SnCF then performs authorization of the sensing service operation (including the parameters in the request).

-	The SnCF performs authorization of the sensing service operation, considering, for instance, the received parameters for the requested sensing service (e.g., Target Sensing Area, Sensing Service Type, Sensing service time duration), information about the sensing service consumer (e.g., AF Identifier), and sensing authorization information such as allowed/forbidden Sensing Service Area, allowed/forbidden Sensing Service types.

-	In case of authorization failure (determined either by the NEF or by the SnCF), the sensing service request is rejected, and the sensing service consumer is provided with a Result value indicating that the authorization has failed.

In order for a 3GPP network to revoke the authorized sensing service, the following main principles apply:

-	Revocation of AF authorization: NEF may revoke the AF authorization to invoke/use sensing-specific APIs at any time, for instance, due to changes in the authorization data or a subsequent sensing service request by the AF.

-	Cancellation of the authorized sensing service: SnCF may stop the ongoing sensing service at any time for which the SnCF previously granted the authorization. In such a case, the SnCF notifies the sensing service consumer that the sensing service has been cancelled, stops all relevant sensing procedures, and may sends the corresponding sensing message to the NG-RAN node to terminate the sensing procedure.

>>>> Next Change <<<<

### 7.2.2	Topics for further consideration for KI#2 on Authorization and Revocation to Support Sensing Service

The following aspects require further considerations to cover authorization aspects fully:

-	Non-allowed/restricted/forbidden/ areas for sensing: It is expected that there will be defined geographic areas/locations where sensing services cannot be performed or at least restricted to certain consumers. Therefore, it needs to be identified:

-	how the information about non-allowed/restricted/forbidden areas for sensing is provisioned,

-	where this information is stored,

-	which 5GC NF (NEF and/or SnCF in particular) retrieves the information about non-allowed/restricted/forbidden areas for sensing,

-	how compliance with non-allowed/restricted/forbidden/ areas are enforced.

The aspects above require also coordination with SA WG3.

>>>> End of Changes <<<<