SA WG2 Meeting #171	S2-2508901

13 – 17 Oct, 2025, Wuhan, China	(revision of S2-2508109)

Source:	MediaTek Inc.

Title:	[WT#1.1, all topics] Work task #1.1 Support of 6G Control signaling

Document for:	Approval

Agenda Item:	20.6.1.1

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes Scope of Work for task 1.1 and associated KI.

# 1. Introduction

Per 3GPP TS 21.905, NAS (Non-Access Stratum) is defined as "Protocols between UE and the core network that are not terminated in the UTRAN." This definition is of course outdated by now (re: UTRAN) but otherwise accurate. 3GPP TS 24.501 defines the NAS protocol for 5GS. The figure below illustrates what NAS basically refers to.

Figure 1-1: NAS protocols

In other words, NAS encompasses all control plane protocols between the UE and the Core Network such as mobility management, session management comprising control plane procedures such as registration, service request, authentication, session management procedures, etc. as specified in 3GPP TS 24.501 for 5GS.

On top of those, some MNO services (e.g. SMS, LCS) can use the NAS as a transport in 5GS today (see NAS transport procedures in TS 24.501). In 6G, discussion needs to be had on how a given service (MNO, OTT) is transported e.g. whether over user plane, control plane (aka NAS) or otherwise, but confusion must be avoided between NAS protocols (#1 priority) and NAS transport offered by NAS.

Figure 1-2: User service transport (examples only)

The Work Task 1.1 was scoped using the generic terms

-	"control signaling" – which encompasses NAS and any other signaling a given service may require.

-	"non-access stratum functionality" – which encompasses everything (in MNO domain) that is not access stratum.

Such generic terminology, helpful in drafting the Work Task, is effectively triggering a lot of confusion. It is critical that a common understanding be reached on terminology before going any further.

Our recommendations are as follows:

I)	to continue using NAS as control plane protocols (and related signaling) and to avoid generalizing NAS beyond what it stands for today

II) While NAS is control signaling between the UE and the Core Network, control signaling is not necessarily only NAS. For example: IMS uses SIP which is not NAS, SMS uses a number of protocols that are not NAS, LCS uses LPP which is not NAS. At the same time there is also NAS signaling that enables the use of IMS, SMS, LCS services. In other words, a distinction need to be made between:

a)	NAS Functionality: A UE and Core Network control plane functionality offered by NAS control plane protocols between peer UE and Core Network entities, noting the functionality itself may of course differ between the UE and the Core Network (e.g. slicing functionality in the UE is not slicing functionality in the Core Network but both operate in concert to enable network slicing)

b)	NAS Signaling: signaling pertaining to NAS procedures offered by NAS control plane protocols between peer UE and Core Network entities. This also includes NAS signaling that enables a given service (e.g. specific policies, service support indication, etc.). NAS Signaling is not User Service Signaling.

c)	NAS Transport: NAS procedures and related signaling enabling the transport of payload between the UE and the Core Network (e.g. 5GS enables NAS Transport, between the UE and AMF, of LPP, SMS, SOR container etc.)

d)	User Service Signaling: signaling pertaining to user service protocols between the UE and an AF/NF (e.g. SIP, LPP, SMS, etc.) for delivering a given user service. User Service Signaling is not NAS Signaling.

Reference: Work Task 1.1 scope as agreed in 6G System Study item S2-2506096: .1. Study the support for control signaling for 6G for connectivity services and/or beyond connectivity services, including at least the following: a. Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities. b. Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities. c. Whether and how to develop generic mechanisms for UE-Core Network interaction to support operator services.

**** First Change ****

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

NAS Functionality: A UE and Core Network control plane functionality offered by NAS control plane protocols between peer UE and Core Network entities.

NAS Signaling: signaling pertaining to NAS procedures offered by NAS control plane protocols between peer UE and Core Network entities. This also includes NAS signaling that enables a given service (e.g. specific policies, service support indication, etc.). NAS Signaling is not User Service Signaling.

NAS Transport: NAS procedures and related signaling enabling the transport of payload between the UE and the Core Network (e.g. 5GS enables NAS Transport, between the UE and AMF, of LPP, SMS, SOR container, etc.)

User Service Signaling: signaling pertaining to user service protocols between the UE and an AF/NF (e.g. SIP, LPP, SMS, etc.) for delivering a given user service. User Service Signaling is not NAS Signaling.

**** Second Change (all text new) ****

# Annex A.X. WT#1.1

WT#1.1 Study the support for control signaling for 6G System for NAS functionalities and user services, including the following:

a.	Whether and how to enable the introduction of a new non-access stratum functionality with minimal or no impact to other non-access stratum functionalities.

b. Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities.

c.	Whether and how to develop generic mechanisms for UE-Core Network interaction to support operator services.

NOTE 1: 	It is assumed that this WT covers impacts to 6G System procedures for NAS functionalities such as mobility management, session management and UE NAS identifiers.

**** Third Change (all text new) ****

# 5.X. Key Issue #X: Support for NAS and User Service Signaling

This Key issue studies the following aspects:

1)	Whether and how to enable the introduction of a new non-access stratum (NAS) functionality with minimal or no impact to other NAS functionalities.

NOTE 1: Coordination with CT1 could be required.

2)	NAS termination and NAS messages routing

3)	Transport of user services over NAS

4)	Whether and how to identify a minimum, common set of NAS functionalities

5)	Study system procedures for NAS functionality, NAS signaling, including impact to RAN-CN interface and 6G CN interfaces related to these system procedures.

6)	Study whether and how to define generic mechanism including architecture, procedures and UE-CN interactions to support user services

-	a. Study system procedures to support generic mechanism for supporting user services, including impact to NAS Signaling procedures (if applicable and with minimum impact), User Service Signaling, RAN-CN interface and 6G CN interfaces related to these system procedures.

-	b. Study how to support transfer and routing of User Service Signaling

NOTE 2: Any impact on the UE and RAN interaction aspects will be coordinated with RAN3 and the decision for RAN-Core interface will be aligned with RAN3.

NOTE 3: Any impact on the RAN and core network interaction aspects will be coordinated with RAN2.

NOTE 4: Interworking and migration aspects, especially inter-PLMN aspects (e.g. roaming), where applicable, are covered by WT#2.

NOTE 5: Potential security and privacy impacts require coordination with SA3.

NOTE 6: 	It is assumed that this key issue covers impacts to 6G System procedures for functionalities such as mobility management, session management and UE NAS identifiers.

NOTE 7: Coordination with KIs related to WT1.2, WT2, WT#3, WT#4, WT#5, WT#6 is required.

**** End of Changes ****