SA WG2 Meeting #S2-171	S2-2508972

13 - 17 October, 2025, Wuhan, China

Title:	LS Response on Exposure of "UE Reachable for DL Traffic" event for a UE in CM-CONNECTED state with RRC-Inactive

Response to:	S2-2508178/C4-253434

Release:	Rel-19

Work Item:	TEI19

Source:	SA2

To:	CT4

Cc:

Contact Person:

Name:	Jinguo Zhu

E-mail Address:	zhu.jinguo@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

SA2 thanks CT4 for the LS on Exposure of "UE Reachable for DL Traffic" event for a UE in CM-CONNECTED state with RRC-Inactive. In the LS CT4 asked the following question:

3GPP TS 23.502 defines the following conditions for the AMF to report a "UE Reachable for DL Traffic" event in Table 4.15.3.1-1:

Detected when the UE transitions to CM-CONNECTED state or when the UE will become reachable for paging, e.g. Periodic Registration Update timer. It indicates when the UE becomes reachable for sending downlink data to the UE.

When the AMF receives a subscription for the above event for a UE in CM-CONNECTED state with RRC-Inactive (assuming the AMF receives N2 notifications with the RRC state), should the AMF report immediately that the UE is reachable for DL traffic or should it do so only when the UE becomes available for paging (e.g., when the UE wakes up from eDRX)?

SA2 answer: If the AMF has received N2 notification (i.e. N2 MT Communication Handling request message) indicating that the UE is transitioning to RRC_INACTIVE state and the determined eDRX values for RRC_INACTIVE, the AMF can determine when the UE is reachable based on the eDRX value. Therefore when the AMF receives a subscription for the "UE Reachable for DL Traffic" event, the AMF reports the UE is reachable for DL traffic only when the UE becomes reachable for paging (e.g., when the UE wakes up from eDRX).

2. Actions:

To CT4 group.

ACTION: 	SA2 kindly asks CT4 to take the answer above into account.

3. Date of Next SA2 Meetings:

TSG SA WG2 Meeting #172		17th Nov – 21th Nov 2025		Dallas, U.S.

TSG SA WG2 Meeting #173		9th Feb – 13th Feb 2026			India, India