3GPP TSG-WG SA2 Meeting #171	S2-2508986

13th-17th October, 2025, Wuhan, China	(revision of S2-250xxxx)

Source:	Apple

Title:	[WT#1.3] Support for untrusted non-3GPP access and multi-access data connections between 3GPP access and non-3GPP access in 6G

Document for:	Agreement

Agenda Item:	20.6.1.3

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution provides detailed scope for WT#1.3 for supporting non-3GPP access and multi-access data connections and proposes corresponding WT wording enhancements and a Key Issue.

# 1. Justifications

The 6G SID approved in TSG SA#108 has the following text for WT#1.3 corresponding to non-3GPP access and multi-access data connections:

1.3.	Study whether and how to support and/or enhance different non-3GPP access (e.g. Wi-Fi, wireline) in 6G and support multi-access data connections between 3GPP access and non-3GPP access

Architectural Motivation:

In EPS, VoWi-Fi has been the driving force for deploying ePDG based solution for connecting untrusted Wi-Fi to EPC. S2b allows the UE to connect from anywhere with any Wi-Fi. Many operators use VoWi-Fi today to provide operator-managed voice and SMS services, mainly focusing on indoor coverage where cellular coverage is inferior to Wi-Fi coverage. The ePDG's integration with the core network facilitates interworking procedures, including seamless handovers between 3GPP and non-3GPP networks.

In 5GS, 3GPP designed an access-agnostic architecture and for non-3GPP access N3IWF provides tight coupling based connectivity to the 5GC through the N2 and N3 interfaces.. Due to this tight integration of non-3GPP access, the UE can make use of the same 5GS features over non-3GPP access (e.g. N1 NAS signalling including Registration and PDU session connectivity, slicing, edge computing, enhanced security, etc.) as over 3GPP access. Despite N3WIF’s advantages over the ePDG architecture, N3IWF has not been deployed in the industry so far. This is largely because initial 5GS deployments started with NSA (non-standalone) option, wherein the new NR radio deployment leveraged existing EPC deployments and with VoWi-Fi still being the main driving application for Wi-Fi, there was no motivation to deploy anything new (N3IWF) as existing S2b deployments using ePDG already supported VoWi-Fi in EPC. Now that 5G SA deployments are gaining momentum, questions are being asked as to how to support non-3GPP access with 6G.

Gap Analysis:

It is expected that supporting untrusted non-3GPP access remains an essential feature for 6G, especially to provide service continuity between 3GPP access network and non-3GPP access network.

For the support of non-3GPP access in 6G, the existing architecture specified in 5GS using N3IWF can be re-used, largely as is. This could support the full set of features for non-3GPP access that are available in 5GS including multi-access data connections that are required for ATSSS. Supporting N3IWF based architecture however requires significant development effort both for UE and network due to the need to support N1 NAS signalling.

Alternatively, 3GPP could also evolve the ePDG based solution in EPS to support untrusted non-3GPP access in 6G. There would be minimal development effort required for the UE to support such an architecture and enable key applications such as VoWi-Fi. On the other hand, evolving the ePDG based solution from EPS and incorporating support of newer security framework and service-based interfaces of 5GS would require some specification and development work (mostly on the network side). Adding multi-access data connections to this architecture would also require significant additional specification and development effort.

Technical Impact:

In order to specify the preferred architecture to support untrusted non-3GPP access in 6G, the necessary features required for the non-3GPP access need to be analyzed and decided based on the requirements set by the key applications (e.g. VoWiFi, multi-access data connections) foreseen to make use of the access. Considering the amount of specification and development effort required both on UE and network side, it is essential to enable cost effective deployments that are also forward looking to meet requirements of any compelling future use cases.

# 2. Proposal for the TR

It is proposed to include the following text in the TR 23.801-01.

**** First Change ****

# Annex A.X. WT#1.3 Scope on non-3GPP access

1.3.	Study whether and how to support and/or enhance different non-3GPP access in 6G and support multi-access data connections between 3GPP access and non-3GPP access.

For non-3GPP access:

-	Study how to support untrusted non-3GPP access in 6G System architecture.

-	How to support service continuity between 3GPP access and untrusted non-3GPP access.

NOTE 1:	Interworking aspects related to non-3GPP access will be studied as part of WT#2.

NOTE 2:  WT#1.2 aspects related to QoS, Policy, and UP applicable to non-3GPP access are studied under this WT with a checkpoint to align with WT#1.2 at TBD.

-	Whether and how to support Non-Seamless WLAN Offload (NSWO).

For the multi-access:

-	Study whether and how to support the multi-access data connectivity (e.g. traffic steering, switching and splitting) between 3GPP access and non-3GPP access for different type of traffic (e.g. IPv4/v6, UDP, TCP) in 6G.

**** Second Change ****

# 5.X. Key Issue #X: Support of untrusted non-3GPP access and multi-access data connections between 3GPP access and non-3GPP access

Supporting untrusted non-3GPP access is an essential feature for 6G, especially to provide service continuity between 3GPP access network and non-3GPP access network.

This Key Issue aims to study:

-	How to support and/or enhance connectivity using untrusted non-3GPP access architecture. Enhancements should consider the amount of specification and development efforts necessary both for UE and network to enable cost effective deployments;

-	How to support service continuity between 3GPP access and untrusted non-3GPP access; and

-	Whether and how to support Non-Seamless WLAN Offload (NSWO).

For UEs that can be simultaneously connected to both 3GPP access and non-3GPP access, this key issue will also investigate whether and how to support access traffic steering, switching and splitting via multi-access data connections between 3GPP access and non-3GPP access.

**** End of Changes ****