SA WG2 Meeting #171	S2-2509085

13- 18 October, 2025, Wuhan, China	revision of S2-2507352

Source:	Xiaomi

Title:	Update Solution #20 UE data collection over UP

Document for: 	Approval

Agenda Item: 	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2/Rel-20

Abstract of the contribution: Update Solution #20 UE data collection over UP.

1. Introduction

There are following ENs in Solution #20 UE data collection over UP

Editor's note: Details on the data collection triggering is FFS, which may take the data collection configuration solution decided by RAN into consideration.

Editor's note: Details on the visibility control information is FFS, based on what visibility handling means.

- For the 1st EN, the triggering entity can be UE itself and the details are not covered in this solution, so the related clarification is provided.

- For the 2nd EN, it related to the visibility requirement from RAN.

a) In RP-243316, the following is stated:

From RAN perspective, transfer of data over UP for Solution 2 in SA2 should be studied and provide feedback on fulfillment of visibility and controllability requirements.

TSG RAN kindly asks TSG SA to start studying the transfer of data over UP for solution 2 in WG SA2

b) In RP-242389, the following is stated:

RAN2 has discussed the topic of UE side data collection for UE-side model training and agreed on a TP for TR 38.843 captured in the annex of this LS.

7.2.1.3.2	Data collection for UE-side model training

The following options were discussed in RAN2:

1a.	UE collects and directly transfers training data to the data collection entity outside the MNO (e.g. Over-The-Top (OTT) server) for UE-side model training. No 3GPP specification impact is expected.

1b.	UE collects training data and transfers it to the server for data collection for UE-side model training (inside the MNO) and then optionally from the server for data collection for UE-side model training to the OTT server (outside the MNO).

2.	UE collects training data and transfers it to Core Network. Core Network transfers the training data to the server for data collection for UE-side model training/OTT server.

3.	UE collects training data and transfers it to OAM. OAM transfers the training data to the server for data collection for UE-side model training/OTT server.

The options listed above were analysed to understand potential specification impact. The analysis included aspects such as termination entities, data transfer path, whether control plane (CP) or user plane (UP) should be used to transfer the data, protocol layers involved, MNO controllability and visibility over the collected data, etc.

…

Table 7.2.1.3.2-1. Analysis of different data collection options for UE-side model training.

| Option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| First termination entity | Training entity (e.g., Over-The-Top (OTT) server) | Server for data collection for UE-side model training | Inside the CN | Inside OAM domain |
| AI/ML-specific Data Transfer Path | UE to OTT server via either 3GPP or non-3GPP network | UE ->Server for data collection for UE-side model training/OTT server (Note 4) | UE-> CN -> Server for data collection for UE-side model training/OTT server (Note 4) | UE-> gNB->OAM -> Server for data collection for UE-side model training/OTT server (Note 4) |
| UP/CP tunnel | UP tunnel (for the case of data transfer from UE to OTT server via 3GPP network) | UP tunnel | CP tunnel (provided that the data volume remains within the NAS signalling capacity) FFS: UP tunnel (Note 7) | CP tunnel (provided that the data volume remains within the RRC signalling capacity) FFS: UP tunnel (Note 7) |
| Protocol layer for data transfer | Application layer | Application layer | NAS layer for CP tunnel FFS: the protocol layer for UP tunnel | RRC layer for CP tunnel FFS: the protocol layer for UP tunnel |
| Controllability of MNO on data transfer | No AI/ML specific controllability | FFS: level of controllability (Note 5) (Note 1) | Full controllability (Note 1) | Full controllability (Note 1) |
| Solution for network controllability | N/A (the OTT server can directly request data from the UE) | Example: per PDU sessions | Via NAS procedure, FFS other procedures | Via RRC procedure |
| Possible Options for data content visibility in MNO (Note 2, Note 3) | No standardized visibility | FFS on level of visibility (Note 5) | Opt A) Full visibility for standardized data contents. Opt B) Partial visibility for partially standardized data contents. (Note 6) Opt C) No standardized visibility. (Note 6) | Opt A) Full visibility for standardized data contents. Opt B) Partial visibility for partially standardized data contents. (Note 6) Opt C) No standardized visibility. (Note 6) |
| Impacted WGs | N/A | SA2, SA3, RAN2, RAN3, CT1 | SA2, SA3, RAN3, RAN2, CT1 and CT3 | RAN2, RAN3, SA3, SA5, SA2 |
| - Note 1: Full controllability: The MNO can manage data transfer to the server for UE-side data collection, without the need of SLA. This includes initiating, terminating, and fully managing data transfer. - Note 2: Visibility of data content signifies that the MNO can, at least, be aware of, access, and comprehend the data without the need of SLA. - Note 3: The following options are identified to realize the different levels of data content visibility to the MNO: - Full visibility for standardized data content. - Partial visibility for partially standardized data content (e.g. UE proprietary information can be included transparently together with the standardized data message). - No standardized visibility (e.g. UE proprietary information can only be included transparently). - Note 4: The potential involvement of NF or other higher layers entities/functionalities should be discussed in other WGs. Impact on the OTT server is not in the scope of RAN2 discussion. - Note 5: RAN2 cannot reach consensus on the level of possible MNO controllability and visibility via Option 1b without input from SA groups. - Note 6: RAN2 has not concluded on the need for partial and no visibility options. - Note 7: RAN2 cannot reach consensus on the feasibility of UP tunnel in Options 2 and 3 |  |  |  |  |

- In RP-243316, the following is clarified from RAN:

As stated in the LS sent from RAN, visibility of data content signifies that the MNO will be able to be aware of, access, and comprehend the content of the collected/reported data without the need of SLA. Thus, full visibility allows the MNO to verify/match the data specified/configured to be collected and the data that is being reported.

Based on the above, the ENs are proposed to be solved as following:

- For the 1st EN, the related clarification is provided to clarify that the triggering entity can be UE itself and the details for data collection triggering are not covered in this solution,

- For the 2nd EN, visibility control information is clarified to allows the MNO to verify/match the data specified/configured to be collected and the data that is being reported.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-04 v102.

* * * Start of Change * * * *

## 6.20	Solution #20: UE data collection over UP

### 6.20.1	High level principles

The solution is for Key Issue #1: Transfer of data over UP for UE data collection.

The principles of the solution including:

-	The mechanism of PDU session with user plane connectivity between UE and the UE model training entity server with UPF performing visibility handling is used, which is totally standardised mechanism which is pretty mature in 5G.

-	The UPF or an dedicated NF (e.g. a new NF as DCF(Data Collection Function) in 5GC performs the full visibility of standardized UE data contents based on configured data packets handling rules.

-	The PDU session management mechanisms of QoS flow management or service data flow management support the controllability of standardized UE data transfer including initiating, terminating data transfer and also support to differentiate the traffic for UE data collection from the UE regular traffic in the 5GC.

-	The UE knows about what the standardized data to be transferred are based on what data is standardised.

### 6.20.2	Description

The data to be transferred is the UE collected data for UE-side model training. And within the scope of KI#1, the data transfer over UP refers to the sending/transfer of the collected data from the UE to the UE model training entity Server via UP with 5GC as intermediary node of the data transfer path between UE and the UE model training entity server. The UE model training entity server can be the server for data collection for UE-side model training or OTT server.

The mechanism of PDU session with user plane connectivity between UE and the server with UPF in 5GC performing visibility handling is used, which is totally standardised mechanism which is pretty mature in 5G. The controllability of standardized UE data transfer including initiating, terminating data transfer and differentiating the traffic for UE data collection from the UE regular traffic in the 5GC can be fulfilled using QoS flow management or service data flow management for the PDU session.

### 6.20.3	Procedures

#### 6.20.3.1	UE data collection over UP

Figure 6.20.3.1-1: UE data collection over UP

0.	UE collects the data based related configuration procedure defined by RAN. The data collection can be triggered by AF, NF within core network,  RAN or UE.

Note:	Details on the data collection triggering is based on which entities triggers the data collection, which may take the data collection configuration solution decided by RAN into consideration. The details on the data collection triggering is not covered by this solution.

1.	The PDU session to manage the data communication path(s) between User Equipment (UE) and the data network where the UE model training entity server is hosted is established. The address information of the UE model training entity server for UE data collection can be obtained by the UE via application layer, or configuration or URSP rule.

2.	The UE sends PDU session modification request to the SMF to QoS rule management indicating the specific user data is expected to be exchanged between UE and the UE model training entity server requiring visibility handling, e.g. via one of the following:

-	addition or update QoS rule with specific packet filter(s), e.g. flow label, remote address info, local address info, Type of service/Traffic class;

-	creating a new Segregation QoS rule by setting the rule operation code to "Create new QoS rule" and set the segregation bit to "Segregation requested".

3.	The SMF may request the PCC rule update from PCF based on the information received via step 2 if the rule for visibility handling is configured in PCF. And the PCC rule to provide visibility for specific service flow(s) or QoS flow(s) is sent to SMF.

4.	The SMF decides the information to apply visibility control based on information received in step 3, or information received in step 2 and/or location policy configuration.

5.	The visibility control information on the specific service flow(s) or QoS flow(s) is configured by the SMF.

If it is the UPF in 5GC performing the visibility of standardized UE data contents, the SMF configures the UPF to perform the visibility check and the data packets are further forwarded after the visibility check.

If it is the dedicated NF such as a new NF as DCF(Data Collection Function) in 5GC performing the visibility check of standardized UE data contents, the SMF decides the DCF, configures the UPF to forwards the related packets to be forwarded to the selected DCF and configures the DCF to perform the visibility check. The data packets are further forwarded after the visibility check.

The PDU session management mechanisms of QoS flow management or service data flow management support the controllability of standardized UE data transfer including initiating, terminating data transfer and also support to differentiate the traffic for UE data collection from the UE regular traffic in the 5GC. The UE or AF or the core network can trigger to initiate, modify or terminate the data transfer of the service data flow or QoS flow for the UE data collection via UP.

The visibility control information includes the data handling action as verifying/matching the received user data with configured information. The configured information can be configured onto the UPF or DCF, or preconfigured in SMF or PCF and sent by the SMF via including configured information in the visibility control information.

6.	The PDU Session Modification Ack is sent to UE.

7.	The standardised data requiring visibility handling is transferred from UE to core network via user plane and the UPF or DCF in 5GC performs the full visibility of standardized UE data contents, after the visibility check, the collected data is sent to UE model training entity server. The data can be transport to UE model training entity server from the NF performing visibility handling or via UPF.

### 6.20.4	Impacts on services, entities and interfaces

SMF/PCF:

-	Decides the PCC rule for visibility control for specific service flow(s) or QoS flow(s) and configures related rule to UPF.

UPF:

-	Enforce the PCC rule for visibility handling for specific service flow(s) or QoS flow(s).

-	Forward the data packets requiring visibility handling to DCF.

DCF:

-	Support visibility handling for the collected data.

-	Forward the data packets to UE model training entity server after visibility handling.

* * * End of Change * * * *