3GPP TSG-SA WG2#171	S2-2509295

Wuhan, China, 13-17 October 2025

Source:	Qualcomm Incorporated

Title:	TR 23.700-04: Update agreement for KI#1

Document for:	Discussion / Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2

Abstract of the contribution: This contribution proposed interim conclusion for KI#1 Transfer of data over UP for UE data.

# 1.	Discussion

This contribution proposed to update the agreed principles for KI#1 Transfer of data over UP for UE data.

# 2.	Text proposal

It is proposed to agree the following changes vs. TR 23.700-04:

# >>>>BEGINNING OF CHANGES<<<<

### 7.1.1	Agreed Principles for KI#1

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#1. This may be populated directly or e.g. also when a topic in clause 7.2.1 gets resolved and a principle is agreed.

The interim agreements on principles for KI#1 are as follows:

-	A new 5GC entity called DCF is defined as the  UP path termination entity to support the transfer of data over UP for UE data. The DCF will further transfer the received data to the AF for UE side model training. The DCF can be standalone or co-located with an existing function in 5GC.

-	A UP path is used between the UE and a 5GC NF, for transferring standardized collected data from the UE using PDU connectivity service provided by a PDU session.

-	Traffic Differentiation

-	Dedicated S-NSSAI/DNN is used to differentiate the traffic of collected UE data from the UE regular traffic per PDU Session level. The UE establishes a new PDU Session, or modifies an existing PDU Session which has been established for UE data transfer, to transfer the standardized UE data, based on URSP rules, or

-	Optionally, The packet filter set (e.g., IP address / IPv6 prefix or port number of the new 5GC entity) is used in the QoS rule and the PDR to identify the packet flow used for UE data transfer.

-	DCF discovery

-	NEF discovers the DCF based on NF discovery procedure from NRF or local configuration.

-	UP path information

-

The FQDN / IP address of the DCF is self-constructed by the UE according to the URSP rule for UP path establishment. The trigger for UE establishing a UP path to the DCF is based on AF’s request or UE side implementation, or

-	Optionally, The DCF is discovered from NRF to support the UE data transfer to different AFs that need to receive data for UE side model training. DCF sends the FQND / IP address to UE.

-	DCF performs the user consent check from UDM by reusing the existing use consent check purpose.

-	The UE is allowed to negotiate with the 5GC about transfer data (e.g., accept / reject / cancel data transfer request)

# >>>>NEXT CHANGE<<<<

### 7.2.1	Topics for further consideration for KI#1

Editor's note:	This clause will include the topics for further consideration as work progresses for the specific KI#1. Eventually this clause should only contain topics for further consideration that did not result in agreements (i.e. in agreed principle(s) in a clause 7.1.1) and can either be then marked as not pursued or postponed to a future Release.

The following topics/principles are for further consideration:

-	Whether and how the UE controls UE data transfer.

-

-	How the configuration information for UE data transfer is sent to the UE.

-

-	How the 5GC NF checks the reported UE data to support the full visibility of standardized UE data contents.

-	Whether and how to select UEs for UE data collection.

-	Whether the UE needs to be authorized for UE data collection.

-	Whether and how to align the UE data measurement with the requirement of UE data transfer provided by the 5GC NF.

-	How does DCF sends UP path information to UE is FFS.

-	Whether DCF performs further processing on the transferred data from the UE (e.g., data aggregation, data anonymisation) as per the configuration provided by the AF (UE side model training server) is FFS.

# >>>>NEXT CHANGE<<<<

# >>>>NEXT CHANGE<<<<

# >>>>END OF CHANGES<<<<