3GPP TSG-SA2 Meeting #171	S2-2509302

# Wuhan, China, 13th Oct 2025 - 17th Oct 2025                                 (Revision of S2-2508278)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1494 | rev | 2 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on VFL preparation and VFL inference procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-09-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the previous meeting, it is already agreed for the VFL inference procedure, if the NWDAF containing AnLF cannot generate the analytics output, the NWDAF containing AnLF will only determine the VFL Server AF for the requested analytics. However, in the Figure 6.2H.2.4.2-1 and also in the clause 6.2H.2.4.1 step 7, there are still some wordings related to the NWDAF as the VFL server case which need to be updated. Remove the parameters in the clause 6.2H.2.4.1 step 0 by referring to the 23.288 clause 6.1.3. Some typo for the VFL service operation in the VFL preparation procedure and VFL inference procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the Figure 6.2H.2.4.2-1 to remove the NWDAF as the server case. Update the description for the step 7 in clause 6.2H.2.4.1. Remove the parameters in the clause 6.2H.2.4.1 step 0 by referring to the 23.288 clause 6.1.3. Correction for the VFL service operation in the VFL preparation procedure and VFL inference procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The VFL inference procedure is not fully correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2H.2.2.1, 6.2H.2.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.2H.2.2.1	Preparation procedure for Vertical Federated Learning when NWDAF/trusted AF is the VFL Server

Figure 6.2H.2.2-1: Preparation procedure for Vertical Federated Learning when NWDAF/Trusted AF is the VFL Server

1.	An NWDAF as VFL Server may send a Vertical Federated Learning preparation request including the Analytics ID to each of the NWDAF VFL Client(s), using Nnwdaf_VFLTraining_Request and to each of the AF VFL Clients(s), using Naf_VFLTraining_Request possibly via NEF when the VFL Client is an untrusted AF. An AF as VFL Server may send a Vertical Federated Learning preparation request including the Analytics ID to each of the NWDAF VFL Client(s), using Nnwdaf_VFLTraining_Request. The NWDAF or trusted AF as a VFL Server may also provide, the suggested VFL Interoperability Information to negotiate the intermediate results that will be used in VFL process (e.g, VFL training and/or VFL inference), the suggested list of sample IDs (e.g, SUPI(s), GPSI(s)) expected to be used in the VFL process. Optionally the feature ID(s) together with VFL Interoperability indicator expected to be used in VFL process for each VFL Client, and as additional criteria for sample alignment, time window of the data samples, and required minimum sample size. When a Trusted AF is acting as a VFL Server, the VFL Client can only be an NWDAF.

NOTE 1:	In step 1, the feature ID(s) will only be provided to the VFL Client(s) when the VFL Server received the supported feature ID(s) of VFL Client(s) during the VFL Client(s) discovery procedure as defined in clause 6.2H.2.1.

NOTE 2:	For UE as samples, UEs can be registered to the network and NWDAF as a VFL server can check with UDM whether the UEs are registered to the network.

2.	Each VFL Client checks if it can meet the ML Model training requirement. Each VFL Client checks the list of sample IDs and required criteria for sample alignment suggested by the VFL Server and then provides to the VFL Server the list of sample IDs that it can accept within the sample IDs suggested by the VFL Server and satisfying the required criteria for sample alignment. If the suggested list of sample IDs is not provided by the VFL Server, the VFL Client provides the list of supported sample IDs to the VFL Server. Each VFL Client checks the VFL Interoperability Information and determines which VFL Interoperability information that the VFL Client supports. The VFL Client determines whether it supports the feature ID(s) that will be used in VFL process when the feature ID(s) was provided by the VFL Server; alternatively, the VFL Clients may provide the list of supported feature ID(s), which is associated to the VFL Interoperability information, to the VFL Server, or VFL Server may know the supported featureID(s) for a VFL Client based on configuration.

3.	Each NWDAF VFL Client invokes Nnwdaf_VFLTraining_Request Response or and each AF VFL Client invokes Naf_VFLTraining_Request Response, possibly via NEF when the AF is untrusted, to indicate to the VFL Server whether it accepts the ML Model training requirements, the VFL Client can also indicate that it cannot join the FL process. When NEF is involved in the procedure, if the NEF receives the SUPI as sample ID(s) from NWDAF, the NEF shall map the SUPI(s) to GPSI(s) before sending to the AF.

4.	The VFL Server determines the final list of samples ID(s) that all selected VFL Client(s) support as the samples to be used in the VFL process. If the VFL Server is NWDAF, it may map the sample ID(s) in different types (e.g, SUPI, GPSI) from the VFL Client(s) into same format (i.e SUPI) as described in clause 6.2.8.2.4.4 before determining the final list of sample ID(s). The VFL Server also determines the feature ID(s) per VFL Client and VFL Interoperability Information to be used in VFL process and provides them to the selected VFL Client(s) at the start of the training phase, as described in clause 6.2H.2.3.1.

# * * * Next change * * * *

6.2H.2.4.1	Inference procedure for vertical federated learning when NWDAF or Trusted AF is acting as VFL server

Figure 6.2H.2.4.2-1: Inference procedure for vertical federated learning when NWDAF or Trusted AF is acting as VFL server

The inference procedure when trusted AF is acting as VFL server may be triggered by a request or subscription from a 5GC consumer NF or internal service logic of the AF acting as VFL server. If triggered by internal service logic of the AF acting as VFL server, the steps 0, 1,7 and 8 are skipped.

0.	The analytics consumer NF sends an Analytics request/subscribe (providing parameters as defined in clause 6.1.3) to NWDAF containing AnLF by invoking a Nnwdaf_AnalyticsInfo_Request or a Nnwdaf_AnalyticsSubscription_Subscribe..

1.	If the NWDAF containing AnLF can be the VFL server to generate the VFL inference results for the requested analytics ID, then step 1 is skipped.

If the NWDAF containing AnLF cannot generate the analytics output, the NWDAF containing AnLF determines the VFL Server AF for the requested analytics.

If the VFL server is trusted AF, the NWDAF containing AnLF sends a request to trusted AF VFL server using Naf_Inference_subscribe/request including Analytics ID, Target of Analytics Reporting = e.g. UE IDs and optionally Analytics Reporting Information=e.g. Analytics target period, Analytics Filter Information and Analytics Accuracy Request.

2.	Based on the information received in the step 0 or 1, VFL server decides to initiate the VFL inference procedure with the VFL clients. Before the VFL server initiate the VFL inference procedure, the VFL server may initiate the VFL training procedure if no VFL model is already trained as described in the clause 6.2H.2.3 based on the information received in the step 0 or 1 or VFL server local configuration. VFL Server selects clients(s) using information stored in the VFL training process. The VFL server may select some or no clients, e.g. depending on the accuracy of the VFL model, the contribution to the training result and the current status of the VFL clients.

When no VFL Clients are selected, the VFL server may generates the VFL inference results based only on its local trained ML model associated with the determined VFL correlation ID, skipping the steps 2 - 6 (and 7 if step 1 was also skipped).

NOTE 1:	If the server does not select VFL clients for all features, it can internally store information about the features considered when deriving the analytics results together with a timestamp to identify the analytics results, to enable to subsequently trace the origin of analytics results and to interpret possible feedback on analytics accuracy.

When a VFL client is not available for inference, e.g. depending on the contribution to the training result, the accuracy of the VFL model, the contribution to the training result and the current status of the VFL clients, the VFL server may select another available VFL client with a well-trained model not yet selected for inference, from the original training process, or the VFL server may select a new VFL client and start a new training procedure.

VFL server NWDAF or trusted AF determines and sends a VFL Inference request/subscription to each VFL client including the Target of VFL inference = e.g. UE IDs, VFL correlation ID to indicate the VFL client which previously well-trained VFL local model associated with this ID will be used and optionally VFL inference filter, Data time window, Time when intermediate local result is needed, Dataset Statistical Properties, Analytics Metadata Request, as defined in clause 6.2H.2.4.3.

2a.	For each NWDAF VFL client, the VFL Server NWDAF or trusted AF sends an Nnwdaf_VFLInference_Subscribe or Nnwdaf_VFLInference_Request to the VFL client.

2b.	For each trusted AF VFL client, the VFL Server NWDAF sends an Naf_VFLInference_Subscribe or Naf_VFLInference_Request to the VFL client.

2c.	For each untrusted AF VFL client, the VFL Server NWDAF sends an Nnef_VFLInference_Subscribe or Nnef_VFLInference_Request to the NEF serving the AF.

2d.	For each untrusted AF VFL client, the NEF converts any internal identifiers to external identifiers and sends an Naf_VFLInference_Subscribe or Naf_VFLInference_Request to the untrusted AF VFL client.

NOTE 2:	For the trusted AF is acting as VFL server, the VFL client can only be the NWDAF.

3.	Each VFL Client collects its local data by using mechanism as specified in clause 6.2 and considering a possibly received data time window to ensure that different VFL clients use the inference data collected in the same time period if the VFL Client does not have local data available already.

4.	Based on the VFL correlation ID, each VFL Client determines the VFL local model to generate the intermediate local inference results.

NOTE 3:	In this Release, it is assumed that local intermediate inference is shared between VFL server and VFL client.

5.	VFL Client sends the client intermediate local results to the VFL server. If requested in step 2, it includes Analytics Metadata, as defined in clause 6.2H.2.4.3. Alternatively, the VFL client may report their status and other cause value for rejecting the VFL inference process (e.g. overload, target UE moved out of NWDAF serving area) in the VFL inference response service.

The intermediate local results, which are sent from the VFL Client to the VFL Server during the VFL inference process, are the information for the VFL Server to combine and generate the VFL inference results.

If the VFL server used an inference subscription in step 2, step 5 may be repeated.

5a.	Each NWDAF VFL client sends an Nnwdaf_VFLInference_Notify or Nnwdaf_VFLInference_Request response to the VFL Server NWDAF or trusted AF.

5b.	Each trusted AF VFL client sends a Naf_VFLInference_Notify or Naf_VFLInference_Request response to the VFL Server NWDAF.

5c.	Each untrusted AF VFL client sends a Naf_VFLInference_Notify or Naf_VFLInference_Request response to the NEF.

5d.	For each untrusted AF VFL client, the NEF converts any external to internal identifiers and sends an Nnef_VFLInference_Notify or Nnef_VFLInference_Request response to the NWDAF VFL server.

6.	The VFL server may collect its local data and generate the intermediate local inference results. When the VFL Server selected VFL clients to participate in the VFL Inference process, it combines all the intermediate local results to generate the combined inference results based on the VFL correlation ID. The VFL server takes into account the participation of each VFL client during the ML training process and the importance of the intermediate local results when it generates the combined inference results.

The VFL server may compute the VFL accuracy based on all the intermediate local results received from VFL clients and the label, if it receives Analytics accuracy request in step 0. The server also aggregates possibly received analytics metadata.

7.	If the trusted AF as the VFL server is determined as described in the step 1, the trusted AF as VFL server sends Naf_Inference_Request Response/Naf_Inference_Notify to the consumer (i.e. NWDAF containing AnLF) including the VFL inference results and optionally the VFL accuracy.

8.	The NWDAF containing AnLF provides the analytics output to the analytics consumer NF based on the VFL inference results by means of either Nnwdaf_AnalyticsInfo_Request Response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 0.

The NWDAF containing AnLF may provide VFL accuracy if the consumer NF provides Analytics Accuracy request in step 0.

NOTE 4:	There may be some time delay between the time when the AnLF provides the analytics output to consumer NF and the time when the AnLF provides VFL accuracy, as the labels are collected after making predictions. The analytics result and VFL accuracy may be send in different response messages.

# * * * * End of changes * * * *