3GPP TSG-WG SA2 Meeting #171	S2-2509323

# Wuhan, CN, 13th Oct – 17th Oct, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.401 | CR | 3935 | rev | 2 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Avoid transfer of user plane PDN connection to S&F Mode network with split MME |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3-ARC |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 1) Data transfer over UP is not supported in split MME case as stated in 23.401: NOTE 3: Some deployments cannot support all system features, for example user plane establishment and user plane data transfer cannot be supported, when a satellite is operating in S&F Mode with the split MME deployment described in Annex O. The UE may be on a terrestrial network (TN) and then moves to a network which is operating in S&F mode. However, the UE does not know if the S&F is deployed with split MME functionality. A PDN connection with UP data transfer from the TN cannot be moved to the NTN S&F network. Such scenario is not addressed in the specification. 2) Incorrect term “S&F mode” exists in the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) When the target MME in the S&F network fetches the context from the source MME, the target MME checks for any PDN connection with S1-U data transfer in the UE context. If any is present, then the target MME removes the PDN connection from the UE context. 2) Change “S&F mode” to “S&F Mode”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1) A PDN connection which cannot be supported in the S&F network with split functionality is included in the UE context for which there is currently no recovery mechanism. 2) Terminology misalignment across specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.13.9.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 4.13.9.2	Transition between S&F Mode and non S&F Mode

If a UE is attached to a PLMN using satellite operating in S&F Mode, the UE follows the existing AS and NAS procedures to access a satellite that is not operating in S&F Mode (either TN or NTN eNodeB) and vice versa.

In order to ensure smooth idle mode transition from S&F Mode to non S&F Mode, if the old (S&F capable) MME in inter MME mobility has the knowledge that the UE context is not up to date (e.g. based on implementation) then the old (S&F capable) MME rejects the MM context transfer.

When a target MME with split MME deployment fetches a UE context from a source MME, the target MME verifies if there is any PDN connection in the UE context which supports S1-U data transfer and removes any such PDN connection from the UE context.

If a satellite has no feeder link connectivity, it can only operate in S&F Mode. When a satellite supporting S&F Mode obtains a feeder link, then, the satellite may cease to operate in S&F Mode depending on operator policy and regulatory requirement.

# * * * * End of changes * * * *