3GPP TSG-SA WG2 Meeting #171	S2-2509453

Wuhan, China, October 13 - 17, 2025	(revision of S2-2509390, S2-2508661)

Source:	Ericsson

Title:	New Solution for DO-A Delivery

Document for:	Approval

Agenda Item:	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC / Rel-20

Abstract of the contribution: The contribution discusses and proposes a new solution for DO-A Delivery.

1. Introduction

In TS 23.700-30, KI#2 was agreed in SA2#170 to support DO-A capable AIoT Devices:

## 5.2	Key Issue #2: Support of DO-A Capable AIoT Devices

This key issue will study the system architecture to support DO-A capable Ambient IoT Devices in Topology 1 and Topology 2.

The following aspects will be studied:

-	How the AIoT Device informs the network of its presence autonomously (e.g., an AIoT Device initiated registration-like procedure) and what are the triggers for the DO-A capable device to inform the network of its presence.

-	Whether and how to consider power consumption of DO-A Capable AIoT Devices.

-	How an AIoT Device sends data to the AIOTF autonomously.

-	Support for routing the data received by AIOTF from an AIoT Device to an AF.

-	Whether and how to enhance the Inventory and Command procedures defined in TS 23.369 [3] to support DO-A capable AIoT Devices.

-	Naiotf, Namf and Nnef interface enhancements to support DO-A capable AIoT Device.

NOTE 1:	The conclusions from Key Issue #1 are the basis for supporting DO-A capable AIoT Devices in topology 2 in this key issue.

NOTE 2:	Coordination with RAN WGs is required.

This solution discusses and focuses on the following aspects of KI#2:

-	How an AIoT Device sends data to the AIOTF autonomously.

-	Support for routing the data received by AIOTF from an AIoT Device to an AF.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-30:

* * * Start of Change * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |
| --- | --- | --- |
| Solutions | Key Issue #1 | <Key Issue #2> |
| #1 |  | X |
| #2 |  | X |
| #3 |  | X |
| #4 |  | X |
| #X |  | X |

* * * Next Change * * * *

## 6.X	Solution #X: DO-A Data Delivery and Routing

### 6.X.0	High-level Solution Principles

The solution is based on the following general principles to support DO-A delivery from DO-A capable AIoT devices:

-	The target address(es) of the AIoT device are stored in the AIoT Device Profile data, which are provided in the subscribe request from the AF.

-	The DO-A capable AIoT devices sends DO-A data over AIoT NAS between the AIoT devices and the AIOTF.

-	The AIOTF forwards the DO-A data to the target AF address(es) obtained from AIoT Device Profile data.

### 6.X.1	Description

This solution addresses KI#2 and focuses on the following aspects:

-	How an AIoT Device sends data to the AIOTF autonomously.

-	Support for routing the data received by AIOTF from an AIoT Device to an AF.

This solution assumes the AIoT device has registered towards the network (i.e., the AIOTF), and have the security contexts established between between the AIoT devices and the AIOTF.

The AIOTF updates its supported domain information or AIoT Device Permanent ID ranges in the NF Profile to the NRF. The NRF supports the AIOTF discovery based on target device information.

The AF subscribes to the DO-A Data Delivery by sending a Subscribe Request to the AIOTF (optioally via the NEF). After receiving the Subscribe Request to the DO-A Data Delivery, the AIOTF performs AF authorization by checking the AF authorization data. If the AF request is authorized, the AIOTF updates the target address(es) provided in the Subscribe Request into the AIoT Device Profile Data in ADM.

The DO-A capable AIoT devices sends DO-A data over AIoT NAS between the AIoT devices and the AIOTF. For example, the AIoT Device sends NAS DO-A Delivery Request to the AIOTF containing the AIoT Device ID and DO-A data. The AIOTF acknowledges the AIoT Device by sending NAS DO-A Delivery Response. The NAS DO-A Delivery Requeust and DO-A Delivery Response are security protected.

NOTE:	The form of AIoT Device ID and the security protection of the NAS DO-A Delivery Request/Response are to be addressed by SA3.

The AIOTF obtains the target address(es) from the AIoT Device Profile Data in ADM, and forwards the DO-A data to the target address(es) together with the AIoT Device Permanent ID.

### 6.X.2	Procedures

#### 6.X.2.1	Subscribe to DO-A Data Delivery

Figure 6.X.2.1-1 Subscribe to DO-A Data Delivery

Figure 6.X.2.1-1 depicts how the AF subscribes to the DO-A data delivery:

1.	The AF sends Subscribe Request to the NEF for DO-A data delivery with the AF ID, target device information (AIoT Device Permanent IDs or Filtering Information) and target address (i.e., notification endpoint of the AF). For AFs in trusted domain, the AF can send Subscribe Request to the AIOTF directly without involving the NEF.

2.	The NEF discovers the AIOTF by providing target device information to the NRF or based on local configuration. The NEF sends the Subscribe Request to the AIOTF for DO-A data delivery with the AF ID, target device information and target address (i.e., notification endpoint of the NEF).

3.	The AIOTF authorizes the AF request to check whether the AF is allowed to subscribe to DO-A delivery, based on the AF authorization data stored in the ADM.

4.	If authorized, the AIOTF updates AIoT Device Profile data with the target address for the target devices. If the target device information from the AF is Filtering Information, the AIOTF needs to update the AIoT device profile data for each device in  the group. Alternatively, the AIOTF may send the Filtering Information to the ADM and the ADM is responsible for the updating for each device in the group.

The target address is the notification endpoint of the NEF or the notification endpoint of the AF, depends on which NF sends the Subscribe Request.

6.	The AIOTF sends Subscribe Response towards the NEF indicating the result of the subscribe. In case of failure, failure cause is included. In case of succeed, subscription correlation ID is included.

7.	The NEF sends Subscribe Response to the AF indicating the result of the subscribe. In case of failure, failure cause is included. In case of succeed, subscription correlation ID is included.

#### 6.X.2.2	DO-A Data Delivery

Figure 6.X.2.2-1 DO-A Data Delivery

Figure 6.X.2.2-1 depicts how DO-A data are delivered from the AIoT device towards the AF:

- The AIoT Device sends AIoT NAS DO-A Delivery Request to the AIOTF with the Device ID and DO-A data.

For topology 1, the AIoT NAS message is delivered via the RAN reader inside NG-RAN node. The NG-RAN node delivers the NAS message to the AIOTF directly or via the AMF.

For topology 2, the AIoT NAS message is delivered via the UE reader. The UE reader delivers the message to the NG-RAN node, and the NG-RAN node delivers the NAS message to the AIOTF via the AMF.

NOTE 1:	The details of AIoT Device ID and the security protection of the NAS DO-A Delivery Request/Response are to be addressed by SA3.

NOTE 2:	The NG-RAN and the AMF are assumed to route the AIoT NAS DO-A Delivery Request to the AIOTF which holds the AIoT device contexts in the Registration Procedure.

Editor’s Note: 	If the request is routed to a new AIOTF, how the new AIOTF fetches the AIoT device contexts from the old AIOTF is FFS.

- The AIOTF authenticates the AIoT device and sends the NAS DO-A Delivery Response to the AIoT Device. The result is included in the response message. In case of failure, the failure cause is included.

NOTE 3:	The authentication is to be addressed by SA3.

3.	The AIOTF gets the target address(es) of the AIoT device from the AIoT Device Profile data. For each target address, step 4 to step 7 are executed individually.

4.	The AIOTF sends Notify message to the NEF with the AIoT Device Permanent ID and DO-A data, if the target address is the notification endpoint of the NEF. If the target address is the notification endpoint of the AF, the AIOTF sends Notify Request to the AF directly without involving the NEF.

5.	The NEF sends Notify message to the AF with the AIoT Device Permanent ID and DO-A data.

### 6.X.3	Impacts on Services, Entities and Interfaces

The following NFs are impacted::

-	AIOTF:

-	AIoT NAS layer support for DO-A Data Delivery.

-	Notify DO-A data to the target address(es) obtained from AIoT Device Profile Data in ADM.

-	Support subscribe request towards the DO-A data delivery.

-	Perform AF authorization for the subscribe request. If authorized, the AIOTF update the AIoT Device Profile data to include the target address provided in the subscribe request.

-	ADM:

-	Enhance AF authorization data for the subscribe request towards DO-A data delivery.

-	Enhance AIoT Device Profile data for the target address(es).

-	NEF:

-	Support subscribe request towards the DO-A data delivery.

-	Notify DO-A data to the AF.

-	NRF:

-	Support domain information or AIoT Device Permanent ID ranges in the NF Profile of the AIOTF.

-	Support AIOTF discovery based on target device information.

-	AIoT Device:

-	AIoT NAS layer support for DO-A Data Delivery.

* * * End of Change * * * *