3GPP SA WG2 Meeting #171	S2-2509611

# Wuhan, China, October 13 – 17, 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.401 | CR | 3943 | rev | 3 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TAI list allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, NEC, MediaTek |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3-ARC |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The SA2 CR which introduced below text did not indicate UE impacts: “If the UE subscription in HSS indicates Store and Forward Satellite operation is not allowed for the UE, then network shall reject the UE request with appropriate cause code.” Thus CT1 did not agree for a new UE behaviour for this case in previous CT1 meeting and during discussion delegates requested for more SA2 inputs. We have two options: - We can clarify that UE does not have S&F subscription then till power cycle it should not attempt on the S&F supporting cell similar to other subscription services. This implies there will be UE impact and CT1 will have to introduce the new cause code. - MME based on implementation does not mix up S&F TAI and non S&F TAI in TAI list, so that when UE moves to S&F TAI, MME gives a reject so that respective S&F TAI can be populated as Forbidden TAI. We propose to go with option 2 because same handling can solve more problems of: - legacy UEs not supporting S&F mode, - Exposure parameters(Estimated delivery time) related to AF as described in 23.682 if that has to work accurately then MME should detect UE moving from S&F mode to not in S&F mode and viceversa. Based on discussion during the meeting, it was concluded that satellite can have either feeder link or will not have feeder link at any given time and mixture of this is not possible. As a consequence of this the cells or TAIs or TAI list are always either S&F or not S&F and they cannot be mixed for the eNB onboard or MME onboard the satellite. |  |  |  |  |  |  |  |  |
| Summary of change: |  | MME or eNB will not mix up TAIs of S&F and non S&F mode |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specs do not clarify that TAIs or TAI list are always homogeneously deployed in either S&F mode or non S&F at a given time. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.5.3s |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************************** Start of Changes ***************************

#### 4.3.5.3	Tracking Area list management

Tracking Area list management comprises the functions to allocate and reallocate a Tracking Area Identity list to the UE. All the tracking areas in a Tracking Area List to which a UE is registered are served by the same serving MME.

The "tracking area list concept" is used with E-UTRAN. With this concept, when the UE registers with the network, the MME allocates a set (a "list") of tracking areas to the UE. By making the centre of this set of tracking areas close to the UE's current location, the chance of a UE rapidly making another tracking area update can be reduced.

If SIPTO at local network with stand-alone GW, Serving GW relocation without mobility and ISR are supported in the core network the Tracking Area list should only contain either Tracking Areas inside one local network or inside the macro network. If the tracking area list covers both local network and macro network, the ISR shall not be activated if the UE is allowed to use SIPTO at local network.

The MME determines the RAT type the UE is camping on, i.e. NB-IoT or WB-E-UTRAN for terrestrial access and WB-E-UTRAN or NB-IoT RAT types for satellite access, based on the Tracking Area indicated in the INITIAL UE MESSAGE by the eNodeB.

To ensure a UE initiates tracking area updating procedure when performing inter-RAT mobility between NB-IoT and WB-E-UTRAN, the E-UTRAN shall be configured such that a Tracking Area does not contain both WB-E-UTRAN and NB-IoT cells, and, the MME shall not allocate a Tracking Area Identity list that contains both NB-IoT and WB-E-UTRAN Tracking Areas.

NOTE X: It is assumed that a satellite either operates in S&F Mode or does not operate in S&F Mode at any given time, so that all the E-UTRAN cell(s), Tracking Area(s) or TAI list served by the onboard MME and onboard eNB are either operating in S&F mode or non S&F mode.The S&F cells and non S&F cells need to be using different Tracking Areas in order to trigger the UEs to perform Tracking Area Update.

To ensure the UE initiates tracking area update procedure when it moves to and from an MME that supports 15 EPS bearers per UE as defined in clause 4.12 to an MME that does not support 15 EPS bearers per MME and vice versa, the MME shall allocate a Tracking Area Identity list that provides homogenous support for 15 EPS bearers per UE.

Other features (e.g. User Plane CIoT EPS Optimisation, Supporting up to 15 EPS bearers per UE, satellite access) may require the MME to adapt how it creates the "list" of TAIs.

NOTE:	This TAI list functionality is different to the SGSN behaviour in GERAN and UTRAN systems. In GERAN/UTRAN the UE is only registered in one Routeing Area at a time.

*************************** End of Changes ****************************