3GPP TSG-WG SA2 Meeting #172	                                   S2-2510089

Dallas, USA, 17th – 21st November, 2025

Source:	Ericsson

Title:	KI#1, Adding Principles: NEF is enhanced to support sensing result exposure

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-19

Abstract: This contribution discusses introduction of a new Gateway Sensing Function (GSF) vs. enhancing NEF to accommodate the required functionality, and proposes a new principle for KI#1 agreements.

# 1. Introduction

This paper discusses principles related to KI#1.

# 2. Discussion

This pCR discusses whether a Sensing Gateway Function (GSF) needs to be introduced between NEF and Sensing Function (SF) for sensing result exposure or the NEF functionality shall be enhanced to accommodate functionalities attributed to a GSF in some solutions.

Observation 1: Solution#2, #4, #5, #6, #7, #8, #9, #10, #11, #12, #13, #14, #15, #20, #21, #23, #24, #26, #28, #29, #30, #31, #32, #36, #37 and #38 propose to enhance the NEF functionalities.

Observation 2: Solution#16, #17, #18, # 23, # 25, #35 and #39 do not have information related to sensing result exposure.

Observation 3: Solution #1 describes a Gateway Function for service authorization, Sensing Function discovery and selection and sensing result exposure, and NEF is not mentioned. Solution #33 and #34 consider the following functionalities of a Gateway Sensing Function:

-	SF discovery and selection;

-	Sensing service authorization;

-	Sensing Result exposure;

-	Sensing request Routing, charging.

Figure 1-1: High level Sensing Architecture with a GSF as shown in Solution#34.

In the solution, the NEF is still required when a third-party AF invokes a sensing request to the network and when the network exposed the determined sensing result.

Observation 4: Solution #19 describes that an Exposure Function is used for Sensing Function discovery and selection and sensing result exposure.

Observation 5: Solution #21 describes NEF/GW-function can receive the sensing results. It does not describe how the NEF and GW shall work with each other.

Except solution#19, which describes Exposure Function providing similar exposure function as NEF, all solutions clearly describe that the NEF shall be used for sensing result exposure to third-party AFs.

Several solutions propose adding a new Gateway Sensing Function to handle sensing service-level authorization as well as the discovery and selection of SFs, for example, based on the target sensing service area. The GSF takes over parts of the functionalities that are handled by the SF in majority of the proposed solutions.

Essentially, the GSF acts as the initial point of access to a SF, conducting some SF’s functionalities along with the discovery and selection of SF(s). In the similar way, the GSF could provide these functionalities to AFs within the operator domain for accessing sensing services and SF selection. However, the same can also be done via NEF.

Therefore, these limited functionalities attributed to GSF do not justify the introduction of a new standalone network function and defining a new reference point and new interface. Therefore, it is proposed these sensing-related functionalities can be supported by enhancing the NEF functionalities, since such approach offers the reuse of the existing NEF common function parts.

The NEF is enhanced to support the following sensing-related functionalities for instance,

-	make use of the existing NEF for authorization of sensing service consumers,

-	perform SF discovery and selection,

-	make use of existing NEF exposure services for sensing result exposure to both third-party and operator’s AFs.

-	route sensing service request to SFs.

An NEF with only the sensing service functionalities can be deployed to support the sensing service specific features/APIs and the NEF features/APIs that can specified for exposure towards sensing service consumers.

# 3. Proposal

It is proposed to include following changes in the TR 23.700-14.

>>>> Start of Changes <<<<

### 7.1.1	Agreed Principles for KI#1: System Architecture to Support Sensing

To support KI#1 System Architecture to Support Sensing, the following principles are concluded:

Principle 1: one new Network Function (i.e. Sensing Function, SF) is defined to support Sensing Service. The SF may contain Sensing control functionality (SCF) and Sensing Processing functionality (SPF).

NOTE 1:	There is no standardized interface between the SCF and SPF in Rel-20 5G-A.

NOTE 2:	How to capture the deployment options of SPF(s) functionality and SCF functionality of the same SF, e.g. co-located or separated, can be discussed during the normative phase.

NOTE 3:	The final name and Acronym for Sensing Function may need further update if necessary.

Principle 2: NO dedicated storage NF is needed to store the Sensing data and Sensing result.

Principle 3: In this study, the gNB is the only entity that acts as the Sensing Entity (SE).

NOTE  4:	Privacy protection and other security aspects will be coordinated with SA WG3 and the related impact to architecture enhancement will be based on SA WG3 during the normative phase.

NOTE 5:	Other KI's interim conclusions will be aligned with KI#1 conclusions.

Principle 5: The NEF is enhanced to support: sensing result exposure to both third-party and operator’s AFs, SF discovery and selection, and routing service requests to SFs.

>>>> Next Change <<<<

# 8	Conclusions

Editor's note:			This clause will capture conclusions for the study.

Where there is consensus, interim agreements (e.g. solution principles descriptions) should be documented in the TR as soon as possible during a study.

These can be documented in the TR as "7.1.Y Agreed Principles for KI#Y" in the "Interim Agreements" clause. If the interim agreement has impacts on another clause in the TR and if there is consensus, that TR clause can be updated.

By consensus interim agreements can become part of the final conclusions of the study.

The Overall Evaluation clause previously used in TR skeletons should not be used.

There should be a Topics for further consideration clause per Key Issue. It is recommended that this is used e.g. to capture common issues that need to be resolved for multiple solutions.

## 8.1	Conclusions for KI#1 on System Architecture to Support Sensing

TBD

>>>> End of Changes <<<<