3GPP TSG SA WG2 Meeting #80	S2-103516

# 30 August – 03 September, 2010, Brunstad, Norway

| CR-Form-v9.6 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.060 | CR | 1149 | rev | - | Current version: | 10.0.0 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LIPA permissions and CSG information for LIPA-able APNs in the HSS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics, Alcatel-Lucent, Panasonic, Verizon Wireless |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LIPA_SIPTO |  |  |  |  |  |  | Date: |  | 24/08/2010 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) |  |
| Reason for change: |  | In relation to LIPA, the user’s subscribed APN stored in the HSS can fall into one of the following three categories: - APN pointing to a PDN that can be accessed (in the user plane) through the Packet Core (EPC or GPRS) only (e.g. the operator’s IMS); - APN pointing to a PDN (or PDNs) that can be accessed in LIPA mode only (e.g. access to the “local” network); - APN pointing to a PDN that can be accessed either via the Packet Core (EPC or GPRS) or via LIPA (e.g. enterprise network). These three APN categories are referred to respectively as: LIPA-prohibited, LIPA-only and LIPA-conditional. In the latter two cases the access in LIPA mode may be authorised only from certain CSG cells. When access to a PDN is attempted from a non-authorised CSG cell, the overall PDN connection establishment either fails (APN with LIPA-only permission) or is completed with user plane access via the Packet Core (APN with LIPA-conditional permission). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 13.1: - Adds “LIPA permissions” in the HSS to reflect the three APN categories described in the Reason for change of the present CR. The LIPA permissions can take one of the following values: LIPA-prohibited, LIPA-only and LIPA-conditional. - For a CSG IDs that can be used to access specific PDNs via Local IP Access, the CSG ID entry is associated with the APN(s) of the corresponding PDN(s). Clause 13.2.3: - The same changes are made in the SGSN storage table. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SGSN will make erroneous decision for PDN connection in LIPA mode that leads to either procedure failure or acces to a wrong PDN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.1; 13.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  | 23.401 CR1644 |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
|  |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of change

## 13.1	HLR/HSS

IMSI is the prime key to the subscription data stored in the HLR/HSS. There may be several sets of PS subscription data per IMSI. This is illustrated in Figure 93.

Figure 93: Subscription Data

As Figure 93 indicates, the PS subscription data is at the same level as basic services. Each PDP subscription is seen as a basic service. Supplementary services are provisioned as part of the overall subscription. Activation of SSs is either at the basic service level (SS1) or at the overall subscription level (SS2).

Table 5 shows the GPRS/EPS subscription data contained in the HLR/HSS.

Table 5: HLR/HSS GPRS/EPS Subscription Data

| Field | Description |
| --- | --- |
| IMSI | IMSI is the main reference key. |
| MSISDN | The basic MSISDN of the MS. |
| SGSN Number | The SS7 number of the SGSN currently serving this MS. |
| SGSN Address | The IP address of the SGSN currently serving this MS. |
| Subscribed Charging Characteristics | The charging characteristics for the MS, e.g. normal, prepaid, flat-rate, and/or hot billing subscription. |
| Trace Reference | Identifies a record or a collection of records for a particular trace. |
| Trace Type | Indicates the type of trace, e.g. MSC/BSS trace, HLR trace, and/or SGSN/GGSN/BSS trace. |
| OMC Identity | Identifies the OMC that shall receive the trace record(s). |
| SMS Parameters | SMS-related parameters, e.g. operator-determined barring. |
| MS PS Purged for GPRS | Indicates that the MM and PDP contexts of the MS are deleted from the SGSN. |
| MNRG | Indicates that the MS is not reachable through an SGSN, and that the MS is marked as not reachable at the SGSN and possibly at the GGSN. |
| GGSN-list | The GSN number and optional IP address pair related to the GGSN that shall be contacted when activity from the MS is detected and MNRG is set. The GSN number shall be either the number of the GGSN or the protocol-converting GSN as described in the clauses "MAP-based GGSN-HLR Signalling" and "GTP and MAP-based GGSN -HLR Signalling". |
| GPRS-CSI | Optional GPRS CAMEL subscription information, see TS 23.016 [5b] |
| MG-CSI | Optional Mobility Management for GPRS CAMEL subscription information, see TS 23.016 [5b]. |
| APN-OI Replacement | Indicates the domain name to replace the APN-OI when constructing the GGSN/PDN GW FQDN upon which to perform a DNS resolution. This replacement applies for all the APNs in the subscriber's profile. See TS 23.003 [4] clause 9.1.2 for more information on the format of domain names that are allowed in this field. |
| ODB for PS parameters | Indicates that the status of the operator determined barring for packet oriented services. |
| Access Restriction | Indicates the access restriction subscription information. (Note, the access restriction applies to both packet and circuit oriented services). |
| IMEI | International Mobile Equipment Identity |
| SVN | Software Version Number |
| RFSP Index | An index to specific RRM configuration in the UTRAN/GERAN |
| CSG Subscription Data | The CSG Subscription Data is a list of CSG IDs per PLMN and for each CSG ID optionally an associated expiration date which indicates the point in time when the subscription to the CSG ID expires; an absent expiration date indicates unlimited subscription. For a CSG ID that can be used to access specific PDNs via Local IP Access, the CSG ID entry is associated with the APN(s) of the corresponding PDN(s). |
| Subscribed UE-AMBR | The Maximum Aggregated uplink and downlink MBRs to be shared across all Non-GBR PDP contexts according to the subscription of the user. |
| URRP-SGSN UE | Reachability Request Parameter indicating that UE activity notification from SGSN has been requested by the HLR/HSS. |
| Homogenous Support of IMS Over PS Sessions for SGSN | Indicates whether or not "IMS Voice over PS Sessions" is supported homogeneously in all RAs in the serving SGSN. |
| Each subscription profile may also contain one or more APN configurations: |  |
| PDP/EPS Bearer Context Identifier | Index of the PDP/EPS Bearer context. |
| PDP Type | PDP type, e.g. PPP or IP (IPv4, IPv6, IPv4v6). |
| PDP Address | PDP address, e.g., an IP address. This field shall be empty if dynamic addressing is allowed. |
| APN-OI Replacement | APN level APN-OI Replacement which has the same role as the UE level APN-OI Replacement but with higher priority than UE level APN-OI Replacement. This is an optional parameter. When available, it shall be used to construct the GGSN/PDN GW FQDN instead of UE level APNOI Replacement. See TS 23.003 [4] clause 9.1.2 for more information on the format of domain names that are allowed in this field. |
| Access Point Name | A label according to DNS naming conventions describing the access point to the packet data network. For S4-SGSN the APN to be used as default APN is indicated. |
| SIPTO permissions | Indicates whether the traffic associated with this APN is allowed or prohibited for SIPTO |
| LIPA permissions | Indicates whether the PDN can be accessed via Local IP Access (LIPA). Possible values are: LIPA-prohibited, LIPA-only and LIPA-conditional. |
| QoS Profile Subscribed | The quality of service profile subscribed. QoS Profile Subscribed is the default level if a particular QoS profile is not requested. . QoS Profile Subscribed is also the maximum QoS per PDP context to the associated APN. |
| Subscribed Evolved ARP | The Subscribed Evolved ARP for PDP contexts associated with the APN. |
| VPLMN Address Allowed | Specifies whether the MS is allowed to use the APN in the domain of the HPLMN only, or additionally the APN in the domain of the VPLMN. |
| PDP/EPS Bearer context Charging Characteristics | The charging characteristics of this PDP/EPS Bearer context, e.g. normal, prepaid, flat-rate, and/or hot billing. |
| EPS subscribed QoS profile | The EPS bearer level QoS parameter values for that APN's default bearer (QCI and ARP) |
| APN-AMBR | The maximum aggregated uplink and downlink MBR values to be shared across all Non-GBR EPS bearers, which are established for this APN. |
| P-GW/GGSN address | The address currently used for the P-GW/GGSN supporting this APN |

NOTE:	IMEI and SVN are stored in HLR/HSS when the Automatic Device Detection feature is supported, see clause 15.5.

An expired CSG subscription should not be removed from the HLR/HSS subscription data before it is removed from the MS's Allowed CSG list or Operator CSG list. When a CSG subscription is cancelled it should be handled as an expired subscription in HLR/HSS subscription data to allow for removing it from MS's Allowed CSG list or Operator CSG list first.

Next change

### 13.2.3	Context fields for one MS

Table 6: SGSN MM and PDP/EPS Bearer Contexts

| Field | Description | A/Gb mode | Iu mode |
| --- | --- | --- | --- |
| IMSI | IMSI is the main reference key. | X | X |
| IMSI-unauthenticated-indicator | This is an IMSI indicator to show the IMSI is unauthenticated. |  | X |
| MM State | Mobility management state, IDLE, STANDBY, READY, PMM-DETACHED, PMM-IDLE, or PMM-CONNECTED. | X | X |
| P-TMSI | Packet Temporary Mobile Subscriber Identity. | X | X |
| P-TMSI Signature | A signature used for identification checking purposes. | X | X |
| IMEI | International Mobile Equipment Identity | X | X |
| SVN | Software Version Number (stored by SGSNs supporting the "Provision of UE Specific Behaviour Information to Network Entities" feature as defined in TS 23.195 [76].) or the "Automatic Device Detection" feature, see clause 15.5. | 3) | X |
| MSISDN | The basic MSISDN of the MS. | X | X |
| Routeing Area | Current routeing area. | X | X |
| Cell Identity | Current cell in READY state, last known cell in STANDBY or IDLE state. | X |  |
| Cell Identity Age | Time elapsed since the last LLC PDU was received from the MS at the SGSN. | X |  |
| Service Area Code | Last known SAC when initial UE message was received or Location Reporting procedure was executed. |  | X |
| Service Area Code Age | Time elapsed since the last SAC was received at the 3G-SGSN. |  | X |
| CSG ID | Last known CSG ID when the UE was active. |  | X |
| CSG membership | Last known CSG membership of the MS when the UE was active. |  | X |
| Access mode | Access mode of last known Cell Identity when the UE was active. |  | X |
| VLR Number | The VLR number of the MSC/VLR currently serving this MS. | X | X |
| New SGSN Address | The IP address of the new SGSN where buffered and not sent N-PDUs should be forwarded to. | X | X |
| Authentication Vectors | Authentication and ciphering parameters (authentication triplets or quintets). | X | X |
| Kc | Currently used A/Gb mode ciphering key. | X | 2) |
| CKSN | Ciphering key sequence number of Kc. | X | 2) |
| Ciphering algorithm | Selected ciphering algorithm. | X | X |
| CK | Currently used Iu mode ciphering key. | 1) | X |
| IK | Currently used Iu mode integrity key. | 1) | X |
| KSI | Key Set Identifier. | 1) | X |
| MS Radio Access Capability | MS, mostly GERAN, radio access capabilities. | X |  |
| MS Radio Access Capabilities for other RATs | MS's Radio access capabilities for UTRAN, E-UTRAN, etc (needed only for PS handover support) | X |  |
| MS Classmark 2 | GERAN/UTRAN CS domain core network Classmark (used if the MS supports SRVCC to GERAN or UTRAN) | X | X |
| MS Classmark 3 | GERAN CS domain radio network Classmark (used if the MS supports SRVCC to GERAN) | X | X |
| Supported Codecs | List of codecs supported in the CS domain (used if the MS supports SRVCC to GERAN or UTRAN, see TS 23.216 [101]) | X | X |
| MS Network Capability | MS network capabilities. | X | X |
| UE Network Capability | Core network capabilities (stored if the UE supports E-UTRAN) | X | X |
| DRX Parameters | Discontinuous reception parameters. | X | X |
| MNRG | Indicates whether activity from the MS shall be reported to the HLR. | X | X |
| NGAF | Indicates whether activity from the MS shall be reported to the MSC/VLR. | X | X |
| PPF | Indicates whether paging for PS and CS services can be initiated. | X | X |
| Subscribed Charging Characteristics | The charging characteristics for the MS, e.g. normal, prepaid, flat-rate, and/or hot billing subscription. | X | X |
| Selected CN operator id | Selected core network operator identity (to support network sharing as defined in TS 23.251 [83]). |  | 4) |
| Trace Reference | Identifies a record or a collection of records for a particular trace. | X | X |
| Trace Type | Indicates the type of trace. | X | X |
| Trigger Id | Identifies the entity that initiated the trace. | X | X |
| OMC Identity | Identifies the OMC that shall receive the trace record(s). | X | X |
| SMS Parameters | SMS-related parameters, e.g. operator-determined barring. | X | X |
| Recovery | Indicates if HLR/HSS or VLR is performing database recovery. | X | X |
| Radio Priority SMS | The RLC/MAC radio priority level for uplink SMS transmission. | X |  |
| Access Restriction | The access restriction subscription information. | X | X |
| GPRS-CSI | Optional GPRS CAMEL subscription information, see TS 23.016 [5b] | X | X |
| MG-CSI | Optional Mobility Management for GPRS CAMEL subscription information, see TS 23.016 [5b]. | X | X |
| ODB for PS parameters | Indicates that the status of the operator determined barring for packet oriented services. | X | X |
| APN-OI Replacement | Indicates the domain name to replace the APN-OI when constructing the GGSN/PDN GW FQDN upon which to perform a DNS resolution. See TS 23.003 [4] clause 9.1.2 for more information on the format of domain names that are allowed in this field. | X | X |
| Subscribed UE-AMBR | The Maximum Aggregated uplink and downlink MBR values to be shared across all Non-GBR PDP contexts/bearers according to the subscription of the user. | X | X |
| UE-AMBR | The currently used Maximum Aggregated uplink and downlink MBR values to be shared across all Non-GBR PDP context/bearers. | X | X |
| APN Subscribed | The APN received from the HLR/HSS. For S4-SGSN the APN to be used as default APN is indicated. | X | X |
| CSG Subscription Data | The CSG Subscription Data is a list of CSG IDs for the visiting PLMN and for each CSG ID optionally an associated expiration date which indicates the point in time when the subscription to the CSG ID expires; an absent expiration date indicates unlimited subscription. For a CSG ID that can be used to access specific PDNs via Local IP Access, the CSG ID entry is associated with the APN(s) of the corresponding PDN(s). |  | X |
| MME IP Address for S3 | IP address of the MME for the MS when connected via E-UTRAN (used if ISR is activated for the MS). | X | X |
| MME TEID for S3 | Tunnel Endpoint Identifier of the MME for the MS when connected via E-UTRAN (used if ISR is activated for the MS). | X | X |
| URRP-SGSN | URRP-SGSN indicating that the HSS has requested the SGSN to notify the HSS regarding UE reachability at the SGSN | X | X |
| Subscribed RFSP Index | An index to specific RRM configuration in the UTRAN/GERAN that is received from the HSS. | X | 5) |
| RFSP Index in Use | An index to specific RRM configuration in the UTRAN/GERAN that is currently in use. | X | 5) |
| > For each active PDN connection with GGSN (using Gn/Gp) or with S-GW (using S4): |  |  |  |
| APN in Use | The APN currently used. This APN shall be composed of the APN Network Identifier and the default APN Operator Identifier as specified in TS 23.003 [4], clause 9.1.2. Any received value in the APN-OI Replacement field in the subscription data is not applied here. | X | X |
| APN Restriction | Denotes the restriction on the combination of types of APN for the APN associated with this PDP Context. (See Note) | X | X |
| APN-OI Replacement | APN level APN-OI Replacement which has the same role as the UE level APN-OI Replacement but with higher priority than UE level APN-OI Replacement. This is an optional parameter. When available, it shall be used to construct the GGSN/PDN GW FQDN instead of UE level APN-OI Replacement. See TS 23.003 [4] clause 9.1.2 for more information on the format of domain names that are allowed in this field. | X | X |
| SIPTO permissions | Indicates whether the traffic associated with this APN is allowed or prohibited for SIPTO |  | X |
| LIPA permissions | Indicates whether the PDN can be accessed via Local IP Access (LIPA). Possible values are: LIPA-prohibited, LIPA-only and LIPA-conditional. |  | X |
| Subscribed APN-AMBR | The maximum aggregated uplink and downlink MBRs to be shared across all Non-GBR PDP contexts/EPS Bearers established for this APN according to the subscription of the user. | X | X |
| Negotiated APN-AMBR | The maximum aggregated uplink and downlink MBRs to be shared across all Non-GBR PDP contexts/EPS Bearers established for this APN according to the GGSN/PGW decision. | X | X |
| VPLMN Address Allowed | Specifies whether the MS is allowed to use the APN in the domain of the HPLMN only, or additionally the APN in the domain of the VPLMN. | X | X |
| EPS subscribed QoS profile | The bearer level QoS parameter values for that APN's default bearer | X | X |
| Subscribed Evolved ARP | The Evolved ARP for this PDN connection according to the subscription of the user | X | X |
| GGSN or P-GW TEID on Gn/Gp or S5/S8 (control plane) | Tunnel Endpoint Identifier at the GGSN or P-GW on the Gn/Gp or S5/S8 interfaces for control plane signalling | X | X |
| GGSN or P-GW IP Address in Use (control plane) | The IP address of the GGSN (using Gn/Gp) or P-GW (when using S4) for control plane signalling | X | X |
| SGSN IP address for Gn/Gp or S4 (control plane) | SGSN IP address for the Gn/Gp (used by GGSN) or S4 (used by S-GW) for control plane signalling | X | X |
| SGSN TEID for Gn/Gp or S4 (control plane) | SGSN Tunnel Endpoint Identifier for the Gn/Gp used by GGSN) or S4 (used by S-GW) for control plane signalling. | X | X |
| S-GW IP address for S11/S4 (control plane) | S-GW IP address for the S11 and S4 interfaces | X | X |
| S-GW TEID for S11/S4 (control plane) | S-GW Tunnel Endpoint Identifier for the S11 and S4 interfaces. | X | X |
| > For each active PDN connection with S-GW (using S4): |  |  |  |
| Default bearer | Identifies the NSAPI of the default bearer, corresponding to the PDP context which was first established within the given PDN connection. | X | X |
| PDN GW GRE Key for uplink traffic (user plane) | PDN GW assigned GRE Key for the S5/S8 interface for the user plane for uplink traffic. (For PMIP-based S5/S8 only) | X | X |
| Each MM context contains zero or more of the following PDP/EPS bearer contexts within the PDN connection:: |  |  |  |
| PDP/EPS bearer Context Identifier | Index of the PDP/EPS bearer context. | X | X |
| PDP State | Packet data protocol state, INACTIVE or ACTIVE. | X | X |
| PDP/PDN Type | PDP type (e.g., PPP) or PDN type (e.g., IPv4, IPv6, or IPv4v6) | X | X |
| PDP/PDN Address | PDP/PDN address, e.g., an IP address. | X | X |
| NSAPI/EPS bearer ID | Network layer Service Access Point Identifier. There is a 1:1 mapping between the NSAPI and the EPS bearer ID. | X | X |
| TI | Transaction Identifier. | X | X |
| TEID for Iu | Tunnel Endpoint Identifier for the Iu interface. |  | X |
| GGSN or P-GW TEID (user plane) | Tunnel Endpoint Identifier of the GGSN or P-GW for user-plane traffic. (NOTE 4) | X | X |
| GGSN or P-GW IP address (user plane) | The IP address of the GGSN or P-GW for user-plane traffic. (NOTE 5) | X | X |
| S-GW IP address (user plane) | S-GW IP address for user plane. | X | X |
| S-GW TEID (user plane) | S-GW Tunnel Endpoint Identifier for user plane. | X | X |
| SGSN IP address (user plane) | SGSN IP address used by GGSN for Gn/Gp or by S-GW for S4 for user plane downlink traffic. | X | X |
| SGSN TEID (user plane) | SGSN Tunnel Endpoint Identifier used by GGSN for Gn/Gp or by S-GW for S4 for user plane downlink traffic. | X | X |
| QoS Profile Subscribed | The quality of service profile subscribed. | X | X |
| QoS Profile Requested | The quality of service profile requested. | X | X |
| QoS Profile Negotiated | The quality of service profile negotiated. This also represents the EPS Bearer QoS parameters QCI and ARP and optionally GBR and MBR in case of GBR bearer | X | X |
| Negotiated Evolved ARP | The evolved ARP to be used for this PDP context as authorized by the GGSN | X | X |
| Radio Priority | The RLC/MAC radio priority level for uplink user data transmission. | X |  |
| Packet Flow Id | Packet flow identifier. | X |  |
| Aggregate BSS QoS Profile Negotiated | The aggregate BSS quality of service profile negotiated for the packet flow that this PDP context belongs to. | X |  |
| Send N-PDU Number | SNDCP sequence number of the next downlink N-PDU to be sent to the MS. | X |  |
| Receive N-PDU Number | SNDCP sequence number of the next uplink N-PDU expected from the MS. | X |  |
| GTP-SND | GTP-U sequence number of the next downlink N-PDU to be sent to the MS. | X | X |
| GTP-SNU | GTP-U sequence number of the next uplink N-PDU to be sent to the GGSN. | X | X |
| PDCP-SND | Sequence number of the next downlink in-sequence PDCP-PDU to be sent to the MS. |  | X |
| PDCP-SNU | Sequence number of the next uplink in-sequence PDCP-PDU expected from the MS. |  | X |
| Charging Id | Charging identifier, identifies charging records generated by SGSN, S-GW and P-GW/GGSN. (NOTE 3) | X | X |
| PDP/EPS bearer Context Charging Characteristics | The charging characteristics of this PDP/EPS bearer context, e.g. normal, prepaid, flat-rate, and/or hot billing. | X | X |
| RNC Address in Use | The IP address of the RNC/BSC currently used. |  | X |
| Prohibit Payload Compression | Indicates that the SGSN should negotiate no data compression for this PDP context. | X |  |
| DTI | Indicates whether SGSN has established direct user plane tunnel between RNC and GGSN (using Gn/Gp) or between RNC and S-GW (using S12) for this PDP context. |  | X |
| MS Info Change Reporting Action | Denotes the need to send changes in CGI/SAI or RAI to the GGSN (using Gn/Gp) or S-GW (using S4) associated with this PDP Context. (See Note 2). | X | X |
| CSG Information Reporting Action | Denotes the need to send changes in User CSG Information to the GGSN (using Gn/Gp) or S-GW (using S4) associated with this PDP Context. (See Note 2). This field denotes separately whether the SGSN is requested to send changes in User CSG Information for (a) CSG cells, (b) hybrid cells in which the subscriber is a CSG member, and (c) hybrid cells in which the subscriber is not a CSG member, or any combination of the above. | X | X |
| CGI/SAI/RAI change support indication | Denotes the indicated level of support to the GGSN (using Gn/Gp) or S-GW (using S4) associated with this PDP Context (See Note 2). | X | X |
| BCM | The negotiated BCM. | X | X |
| > For each PDP/EPS bearer context with S-GW (using S4): |  |  |  |
| DL TFT | Downlink Traffic Flow Template. (For PMIP-based S5/S8 only) | X | X |
| UL TFT | Uplink Traffic Flow Template. (For PMIP-based S5/S8 only) | X | X |
| NOTE 1: The SGSN might not have information on the allocated IPv4 address. Alternatively, following mobility involving a pre-Release 8 SGSN, this IPv4 address might not be the one allocated to the MS. NOTE 2: APN Restriction, CGI/SAI/RAI change support indication, MS Info Change Reporting Action and CSG Information Reporting Action shall not be transferred between SGSNs during mobility management, unless the connectivity between the SGSNs is based on S16.. NOTE 3: When the SGSN is connected with a S-GW through S4, then the Charging Id is only applicable for online charging with CAMEL as specified in clause 15.1.0. NOTE 4: The PDN GW TEID is needed in SGSN context as S-GW relocation is triggered without interaction with the source S-GW. The Target S GW requires this Information Element, so it must be stored by the SGSN. NOTE 5: The PDN GW IP address is needed in SGSN context as S-GW relocation is triggered without interaction with the source. The Target S-GW requires this Information Element, so it must be stored by the SGSN. |  |  |  |

The information marked with a "1)" in table 6 may be maintained if authentication is performed by the UMTS authentication procedure.

The information marked with a "2)" in table 6 may be maintained if authentication is performed by the GSM authentication procedure.

The information marked with a "3)" in table 6 is optional. It can be sent to a new SGSN at RA update.

The information marked with a "4)" in table 6 is used in networks that support network sharing as defined in TS 23.251 [83].

The information marked with a "5)" in table 6 is used in UTRAN only.