3GPP TSG-SA3 Meeting #104-e 	S3-212759

# e-meeting, 16 - 27 August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 1180 | rev | - | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NF to always insert PLMN-ID enabling roaming scenario |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2021-08-09 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In a roaming scenario where one SEPP serves a given PLMN, and such PLMN has multiple PLMN-IDs but uses the same N32 connection for all PLMN-IDs, the current specification does not describe the solution how to identify and verify the source PLMN-ID of a message. Please check S3-212755 DP for more information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | NFs insert 3gpp-Sbi-Asserted-Plmn-Id header in NF service/subscription request and notification request. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Roaming scenarios where one SEPP serves a given PLMN, and such PLMN has multiple PLMN-IDs but uses the same N32 connection for all PLMN-IDs, will not work. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.9.3.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Start of Change ***

#### 5.9.3.x	Requirements for Network Function (NF)

NFs involved in interconnect shall provide PLMN ID (3gpp-Sbi-Asserted-Plmn-Id header) in service/subcription request and notification messages.

NOTE: 	In a roaming scenario where one sending SEPP serves a given PLMN, and such PLMN has multiple PLMN-IDs but uses the same N32 connection for all PLMN-IDs, the receiving SEPPs need to get the correct PLMN ID for each message.

*** End of Change ***