3GPP TSG-SA3 Meeting #107-e 	S3-220694

# e-meeting, 16 - 20 May 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.535 | CR | 0126 | rev | - | Current version: | 17.05.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on anonymization api |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2022-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | At the last meeting, CR S3-220569 was agreed to define the anonymized API at the AAnF. This API can be invoked by AF directly when AF is located in the operator domain. It is not clear if the same API can be used by NEF as well when AF outside the operator domain wants to access the AAnF service for anonymized users. NEF usually needs SUPI for charging purposes. Therefore, even if AF (non-trusted ) invokes the NEF api for an anonymized user and if NEF needs SUPI, then NEF invokes the AAnF api where AAnF can provide SUPI to NEF. Then it is an NEF responsibility to not pass SUPI to AF. Secondly, if AF is not trusted and does not want to know UEID, (anonymization use case) should be able to invoke the NEF api with anonymization. However, the NEF API is not enhanced; therefore, AF will get UEID even if AF is not interested. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added: Informative note is added to clarify NEF behavior Added: Similar to AAnF , Nef_AKMA_ApplicationKey_ AnonUser_Getservice api is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The missing clarification causes NEF to always invoke the anonymized AAnF API if AF is requesting the same. Non trusted AF (behind NEF) will always get UEID even if it is not interested (anonymization case). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2, 7.3.1,7.3.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************ START OF CHANGES

### 6.2.2	AAnF response without UE Identity

In some scenarios, anonymous user access to the AF is desirable (e.g., UE identification is not required at the AF). For allowing such anonymous user access to the AF, the procedure detailed in clause 6.2.1 of the present document is used with the following changes:

-	in step 2, instead of Naanf_AKMA_ApplicationKey_Get request,  Naanf_AKMA_ApplicationKey_AnonUser_Get request is used by the AF; and

-	in step 4, the AAnF sends Naanf_AKMA_ApplicationKey_AnonUser_Get response to the AF with KAF and the KAF expiration time.

The A-KID functions as a temporary user identifier.

NOTE:	If NEF needs SUPI for charging, then the NEF will not use this API. I.e., if a non-trusted AF invokes NEF for an anonymous user and if the NEF requires SUPI for charging, then the NEF retrieves SUPI from the AAnF but does not pass it to the AF.

**** Next change****

### 7.3.1	General

The NEF exposes AKMA Application Key derivation service to the requester NF.

The following table shows the NEF Services and NEF Service Operations related to AKMA service.

Table 7.3.1-1: List of NEF Services

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_AKMA | ApplicationKey_Get | Request/Response | AF |
|  | ApplicationKey_AnonUser_Get | Request/Response | AF |

**** Next change****

### 7.3.x	Nnef_AKMA_ApplicationKey_ AnonUser_Getservice operation

Service operation name: Nnef_AKMA_ApplicationKey_AnonUser_Get.

Description: The NF consumer requests the NEF to provide AF related key material. This service is for allowing anonymous user access to the AF based on A-KID (i.e., UE identification is not required at the AF). The A-KID functions as a temporary user identifier.

Input, Required: A-KID, AF_ID

Input, Optional: None.

Output, Required: KAF, KAF expiration time.

Output, Optional: None.

************ END OF CHANGES