3GPP TSG-SA3 Meeting #107-e 	S3-221135

# e-meeting, 16th May 2022 - 20th May 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 1423 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MBS capability exchange and delivery method |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2022-05-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Due to optionality to support CP procedure and UP procedure, it is required for the network to know the capability of the UE to determine whether the network also supports the security procedure. Further, if some UEs does not support MBS security procedure, then the network may need to provide the MBS session without protection as to provide service to all subscribed UE. Exploiting this situation by the malicious UE will lead to bidding down attack. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - UE includes its MBS security capability in the N2 message. - Network either reject the request or prepares for individual MBS traffic delivery method of the UEs that does not support the procedure selected/supported by the network, instead of shared MBS traffic delivery. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Possibility of providing MBS service without security protection, if malicious UEs indicate it does not support MBS security capability. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | W.4.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*****Start of Change*****

### W.4.1.1	General

For security protection of MBS traffic, control-plane procedure and user-plane procedure are optionally supported in service layer. The UE includes the MBS security capability in the PDU Session Modification Request message. The MBS security capability indicates whether UE supports service layer protection or not and if supported then, indicates whether UE supports control-plane procedure and/or user-plane procedure. If the MBS security capability of the UE does not support the network selected MBS security procedure, then the SMF shall either reject the request or prepare for 5GC individual MBS traffic delivery for the UE without service layer protection instead of deactivating the security protection for the MBS session delivery via 5GC shared MBS traffic delivery method.

For 5GC Individual MBS traffic delivery method, the SMF shall determine and provide UP security policy for the associated PDU session to the gNB as specified in TS 23.502 [8].

The user plane security between UE and RAN shall be deactivated when 5GC shared MBS traffic delivery method for MBS data transmission is used to avoid redundant protection.

*****End of Change*****