3GPP TSG-SA3 Meeting #120	S3-250443

Athens, Greece, 17 - 21 February 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.256 | CR | DraftCR | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding USS changeover with multi-USS support |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Interdigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph3_Sec |  |  |  |  |  | Date: |  |  | 2025-02-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Adding support for USS changeover for the support of multi-USS functionality |  |  |  |  |  |  |  |  |
| Summary of change: |  | the UAS NF is informed about the new serving USS info during a USS initiated re-authentication/authorization procedure. The payload information from USS is forwarded to the UAV includes the new serving USS info that the UAV is authorized to communicate with. The UAS NF verifies that the new USS is authorized to request UAV re-authorization based on list of authorized serving USSs info in the UAV UUAA context and authorization token provided by the new USS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | USS changeover cannot be supported for the support of multi-USS functionality |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1.4, 5.2.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF CHANGES ***

#### 5.2.1.4	UUAA re-authentication procedure (5G)

As described in clause 5.2.1.1, the USS or the AMF (if support UUAA during registration) may initiate the Re-authentication procedure for the UAV at any time.

This clause describes the USS initiated Re-authentication procedure (the AMF initiated Re-authentication procedure is described in the clause 5.2.1.2). The below description considers only the security related parameters (for full details of the flows see TS 23.256 [3]).

Figure 5.2.1.4-1: UUAA re-authentication in 5GS

1. The USS sends a re-authentication request for the UAV to UAS-NF that includes GPSI, CAA-Level UAV ID, and an authentication message. It may contain the PDU Session IP address if available. The USS shall use the UAS NF Routing information received during the previous successful UUAA related to GPSI for sending the re-authentication request. If the request is to perform a USS changeover, the request from the target USS shall include an authorization token to allow the changeover and an Aviation Payload including the target USS identifier to be forwarded to the UAV.

2. The UAS NF retrieves the UAV UE's context. The UE's context contains identity mapping between the GPSI and the USS identifier that performed UAA. The UAS-NF verifies the USS re-authentication request by checking whether the GPSI and the USS identifier match of the USS requesting the re-authentication the stored mapping of GPSI and USS identifier. The UAS-NF shall only continue the re-authentication procedures if match.

If the request is to perform a USS changeover, the UAS-NF verifies that that the USS changeover is authorized by verifying the authorization token and whether the USS identifier matches one of the USS identifiers in the list of authorized serving USSs in the UAV UE's context. The UAS-NF shall only continue the re-authentication procedures if match.

NOTE 1:	The USS identifier is based on the security link on the interface between UAS NF and USS (e.g. the identity mapped during link establishment or the identity in certificate).

The UAS NF determines whether the target NF is an AMF or an SMF.

-	If the target NF is an AMF, the UAS NF further determines the target AMF for re-authentication and continues step 3a.

-	If the target NF is an SMF, the UAS NF further determines the target SMF for re-authentication and continues step 3b.

3a or 3b. The UAS NF sends to either the target AMF or the target SMF the UAA re-authentication request for the UE identified by the GPSI and for the SMF only the PDU Session IP address if available. If the request is to perform a USS changeover, the request includes a transparent container including the target USS identifier.

4. The UAS NF responses the USS that the UAA Re-authentication has been initiated.

5a. If the target NF is an AMF, the AMF initiates re-authentication of the UAV as UUAA described in the clause 5.2.1.2 (step 2 to step 9). If the UUAA is performed for a USS changeover, the transparent container from AMF sent to UAV over NAS MM transport includes the target USS identifier with whom UAV performs UUAA.

5b. If the target NF is an SMF, the SMF initiates re-authentication of the UAV as UUAA described in the clause 5.2.1.3 (step 2 to step 7). If the UUAA is performed for a USS changeover, the transparent container from SMF sent to UAV over NAS MM transport includes the target USS identifier with whom UAV performs UUAA.

NOTE 2:	If the procedure is performed for a USS changeover, the new USS (i.e. the target USS) notifies the old USS (i.e., former serving USS) about the changeover. This notification is out of 3GPP scope.

*** NEXT CHANGES ***

#### 5.2.2.3	UUAA re-authentication procedure (EPC)

The USS the Re-authentication procedure for the UAV at any time. The below description considers only the security related parameters (for full details of the flows see TS 23.256 [3]).

Figure 5.2.2.3-1: UUAA re-authentication in EPS

1. The USS sends a re-authentication request for the UAV to UAS-NF that includes GPSI, CAA-Level UAV ID, and an Authentication message. It may contain the PDU Session IP address if available. The USS shall use the UAS NF Routing information received during the previous successful UUAA related to GPSI for sending the re-authentication request. If the request is to perform a USS changeover, the request from the target USS shall include an authorization token to allow the changeover and an Aviation Payload including the target USS identifier to be forwarded to the UAV.

2. The UAS NF retrieves the UAV UE's context. The UE's context contains identity mapping between the GPSI and the USS identifier that performed UAA. The UAS-NF verifies the USS re-authentication request by checking whether the GPSI and the USS identifier of the USS requesting the re-authentication match the stored mapping of GPSI and USS identifier. The UAS-NF shall only continue the re-authentication procedures if match.

If the request is to perform a USS changeover, the UAS-NF verifies that that the USS changeover is authorized by verifying the authorization token and whether the USS identifier matches one of the USS identifiers in the list of authorized serving USSs in the UAV UE's context. The UAS-NF shall only continue the re-authentication procedures if match.

NOTE 1:	The USS identifier is based on the security link on the interface between UAS NF and USS (e.g. the identity mapped during link establishment or the identity in certificate).

3. The UAS NF sends to the target SMF+PGW-C the UAA re-authentication request for the UE identified by the GPSI. If the request is to perform a USS changeover, the request includes a transparent container including the target USS identifier.

4. The UAS NF responses the USS that the UAA Re-authentication has been initiated.

5. The SMF+PGW-C initiates re-authentication of the UAV as UUAA described in the clause 5.2.2.2 (step 4c to step 7). If the UUAA is performed for a USS changeover, the transparent container from SMF+PGW-C sent to UAV over NAS MM transport includes the target USS identifier with whom UAV performs UUAA.

NOTE 2:	If the procedure is performed for a USS changeover, the new USS (i.e. the target USS) notifies the old USS (i.e., former serving USS) about the changeover. This notification is out of 3GPP scope.