3GPP TSG-SA3 Meeting #120	S3-250767

# Athens, Greece, 17 - 21 February 2025

Source:	Samsung

Title:	Security procedure for inter-CU LTM

Document for:	Approval

Agenda item:	4.12

Spec:	3GPP TS 33.501

Version:	19.1.0

Work Item:	NR_Mob_Ph4_Sec

Comments

This pCR proposes normative text to the draftCR for security procedure for inter-CU LTM.

* * * First Change * * * *

#### 6.X.Y	Security mechanism and procedures for L1/L2 Triggered Mobility

Editor’s Notes: This clause contains the security procedure for inter-CU LTM in 5G.

##### 6.X.Y.1	When DC is not configured

During LTM execution phase, for the case where CU is acting as MN and DC is not configured as specified in TS 38.300 [52], the source gNB generates the KNG-RAN* from target PCI, its frequency ARFCN-DL/EARFCN-DL, and either from currently active KgNB or from the NH during as described in Annex A.11 or A.12. The source gNB performs either a vertical key derivation if an unused {NH, NCC} pair is available or horizontal key derivation. The source gNB shall send the generated {KNG-RAN*, NCC} pair to the target gNB in CELL SWITCH NOTIFICATION message.

The source gNB includes the NCC used for the derivation of KNG-RAN* in the Cell Switch Command MAC CE to the UE to avoid any de-synchronization issue due to NCC mismatch. Upon receiving the cell switch command, if the NCC of the current KgNB is less than the received NCC value, then the UE synchronizes the locally kept NH parameter by computing the function defined in Annex A.10 iteratively and computes the KNG-RAN* from the synchronized NH parameter. If the received NCC value is equal to the NCC value associated with the current KgNB, then the UE uses the active KgNB to derive the KNG-RAN*.

For the gNBs with split DU-CU implementations, the DU receives the NCC value from its CU. The CU provides the NCC of either the unused NH, if present or that of the current KgNB to the DU. The DU includes the received NCC value in the cell switch command to the UE. To avoid any de-synchronization issue due to NCC mismatch, the DU shall include the NCC value that is provided to the UE in the DU-CU Cell Switch Notification message to the CU. The CU shall derive the KNG-RAN* based on the received NCC value in the DU-CU Cell Switch Notification message.

The target gNB shall initiate an intra-gNB-CU handover, if there is mismatch in the UE security context received in the PATH SWITCH REQUEST ACKNOWLEDGE message from the AMF and the UE security context received from the source gNB. The target gNB shall indicate to the UE to change the current KgNB, to reconfigure the DRBs to activate or de-activate the UP integrity/confidentiality as per the received policy from SMF and selected algorithms in the intra-cell handover procedure. Further, the target gNB updates the UE’s 5G security capabilities and/or UE’s UP security policy for the active PDU session to all candidate gNBs.

##### 6.X.Y.2	When DC is configured

In the case where CU is acting as SN and MN is unchanged as specified in TS 37.340 [51], the security mechanism and procedures as specified in clause 6.10.2.4 of this specification for SCPAC shall be applied.

* * * End of Changes * * * *