3GPP TSG-SA3 Meeting #120	S3-251050

# Athens, Greece, 17 - 21 February 2025

Source:	Xiaomi

Title:	Pseudo-CR on 33.721: CAPIF-based Solution for Key Issue #4

Document for:	Approval

Agenda item:	5.18

Spec:	3GPP TR 33.721

Version:	0.6.0

Work Item:	FS_Metaverse_Sec

Comments

This pCR proposes a new solution based on CAPIF framework to address the requirements in key issue #4 in TR 33.721.

* * * First Change * * * *

6.Y	Solution #Y: Avatar authentication and authorization supported by CAPIF

6.Y.1	Introduction

This solution addresses KI #4 on authentication and authorization of digital representation.

When CAPIF is used to support metaverse enablement services, it serves as the framework for north bound API of the DA service provided by the DA server. In this case, the DA server can be mapped to the AEF and the API invoker in VAL UE can be mapped to the API invoker in CAPIF functional model.

For authenticating the avatar and authorizing its usage by the user, the API invoker in the VAL UE invokes the DA service API for retrieving DA profile from the AEF (i.e. DA server). Based on validation of avatar authenticity and the retrieved DA profile containing the association between avatar and user, the VAL UE is able to determine whether the avatar can represent the user in the application and whether the user is allowed to use the avatar in the application.

6.Y.2	Solution details

1.	The metaverse application triggers the VAL UE for avatar authentication and authorization by passing at least the avatar ID and corresponding avatar (media), VAL user ID, app ID.

As the avatar is created and signed by the DA server, the VAL UE first verifies the authenticity of the avatar by verifying the signature of the avatar using the certificate of the DA server if locally available. Only when the avatar is verified as authentic, the VAL UE will then proceed with the following steps.

Figure 6.Y.2-1: Avatar authentication and authorization

2.	The API invoker in the VAL UE sends a DA service API invocation request to the DA server, including at least invoker ID, identifier of DA service API to be invoked (i.e. SS_DAProfileManagement_Retrieve in TS 23.438 [8]), avatar ID and optionally app ID. If the VAL UE is not able to verify the authenticity of the avatar (e.g. no DA server certificate), it also sends the received avatar to the DA server for verification.

3.	If avatar is included in the request, the DA server first verifies the authenticity of the avatar, and then checks the app ID if received. Only when the avatar is verified as authentic and allowed to be used in the application, the DA server will then accept the DA service API invocation request. Otherwise, the DA server returns a failure response to the VAL UE. If needed, the DA server may obtain more authorization information from the CCF before responding.

4.	The DA server sends the DA service API invocation response to the VAL UE, which contains either the requested DA profile or a result.

5.	If the requested DA profile or a result is received, it implies that the authenticity of the avatar is successfully verified, which means that avatar is implicitly authenticated. The VAL UE then determines whether the authentic avatar can be used to represent the VAL user or whether the VAL user is allowed to use the avatar, by checking the association between avatar and VAL user in the DA profile.

6.	The metaverse application in the UE proceeds only if the avatar authentication and authorization is successful.

6.Y.3	Evaluation

This solution addresses KI #4 on authentication and authorization of digital representation by applying the CAPIF framework.

From the perspective of the avatar, the combination of successful verification of avatar authenticity and identified association between the avatar and user can ensure that the avatar is authenticated to represent the user in a mobile metaverse application. From the perspective of the user, this combination also ensures that the user is authorized to use the avatar in a mobile metaverse application. Hence, both requirements in key issue #4 are fulfilled.

This solution reuses the procedures in CAPIF without impact on CAPIF or DA service architecture. The DA server or the VAL UE is required to verify the authenticity of the avatar.

This solution addresses the case where the VAL user is the UE subscriber.

* * * End of Change * * * *