3GPP TSG-SA3 Meeting #120	S3-250471

# Athens, Greece,  17 - 21 February 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.558 | CR | draftCR | rev |  | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Solutions for secure retrieval of 5G system UE Ids and privacy related information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph3 |  |  |  |  |  | Date: |  |  | 2024-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The KI#2.1 of TR 33.749 (Secure retrieval of 5G system UE Ids and privacy related information) was concluded by a collection of solution design principles to be followed in normative text, and by indicating that alternative solutions with those design principles would be captured in an informative annex during normative work. This CR compiles those solutions in a new informative annex in TS 33.558. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New annex which compiles alternative solutions for secure retrieval of 5G syterm UE Ids and privacy related information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There would not be illustrative examples of implemenation of the security principles agreed for security retrieval of 5G system UE ids and privacy related information. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New annex X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 33.210: "3G security; Network Domain Security (NDS); IP network layer security".

[3]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[4]	Void

[5]	3GPP TS 23.558: "Architecture for enabling Edge Applications."

[6]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

[7]	3GPP TS 33.122: "Security aspects of Common API Framework (CAPIF) for 3GPP northbound APIs"

[8]	Void

[9]	Void

[10]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[11]	3GPP TS 33.535: "Authentication and Key Management for Applications (AKMA) based on 3GPP credentials in the 5G System (5GS)".

[12]	3GPP TS 33.222: "Generic Authentication Architecture (GAA); Access to network application functions using Hypertext Transfer Protocol over Transport Layer Security (HTTPS)".

[13]	Void

[14]	Void

[15]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[16]	IETF RFC 6750: "The OAuth 2.0 Authorization Framework: Bearer Token Usage".

[17]	IETF RFC 7519: "JSON Web Token (JWT)".

[18]	IETF RFC 7515: "JSON Web Signature (JWS)".

[19]	IETF RFC 9113: "HTTP/2".

[20]	IETF RFC 9110: "HTTP Semantics".

[21]	3GPP TS 23.502: "Procedures for the 5G System".

* * * Second change * * * *

# Annex X (informative): Alternative solutions for secure retrieval of 5G system UE Ids and privacy related information

## X.1	Introduction

The clause 5.1.X of the present document defines the security requirements for secure retrieval of 5G system UE Ids and privacy related information. The following clauses present different alternative implementations that comply with the previously defined security requirements.

## X.2	AKMA based solution

Figure X.2-1 illustrates the procedure to fetch the 5G system UE privay related information based on AKMA. The 5G UE Id is not required to be fetched by the AF, since 5G Core through AKMA feature provides already the authentication service, and the trust between UE, 5G Core and AF.

Figure X.2-1: Procedure to fetch the 5G system UE privacy information based on AKMA

Preliminary steps:

- Primary authentication
- AKMA Key derivation (A-KID) as per procedure in TS 33.535 [11].
- Creation of AKMA context in AAnF (SUPI, KAKMA, A-KID) as per procedure in [11].

Step 1. The application client in the UE (EEC) requests a service to the AF, i.e., EDGE application server (EAS, EES), including the A-KID (AKMA Key Identifier) in the request.

Step 2-5. AKMA procedure intended to provide KAF to the AF from the AAnF via NEF as specified in clause 6.3 of [11].

NOTE: AF specific UE Id does not need to be part of the of the incoming request (step 2), since the NEF will provide the GPSI (external ID) to AF.

Step 6. The AF requires to know privacy information of the UE available in the 5G Core, such as the the location of UE, and makes the request to the 5G Core via NEF, using the given GPSI as UE identifier.

Step 7. The NEF checks the GPSI and if the request is authorized, the corresponding information, in this example location of the UE, is provided to the AF.

Step 8. The AF responds to the UE (EEC).

## X.3 	Temporary ID based solution

Figure X.3-1 illustrates the procedure to fetch the 5G system UE Id and private related information based on the utilization of a temporary identifier (ID) as UE identity, generated by the Core and associated to the PDU session.

Figure X.3-1: Procedure to fetch the 5G system UE Id and privacy related information based on a temporary ID

Step 0. The EEC requests a Temporary ID to UE through interface provided by UE module.

Step 1. PDU session establishment / modification request is sent from the UE to the 5G system.

Step 2. The 5G Core (e.g., UDM) generates (per UE) an internal temporary ID (with a ramdomly generated part) for PDU session.

NOTE 1: The UE could request multiple temporary IDs during the same PDU session (PDU Session modification). The UE can use this to invalidate previous IDs which might have been compromised and/or at fix intervals without initializing another PDU session establishment/modification.

NOTE 2: The Temporary ID consists of a random part and an identifier of the UDM (UDM group ID) that generated the Temporary ID.

NOTE 3: The Temporary ID could also be generated from SMF and sent to UDM/UDR for storage.

Step 3. The 5G Core returns the internal temporary ID to the UE as part of the NAS PDU session establishment/modification response.

Step 4. The EEC uses the allocated temporary ID, received from the UE, inside the Edge application layer communication with the application function AF (e.g., EAS).

Step 5. The AF uses the received temporary ID to invoke the Nnef_UEId API intended to fetch the 5G system UE identifier, i.e., AF specific UE Id, as per the procedure described in TS 23.502 figure 4.15.10-1 [X].

Step 6. The NEF verifies the temporary ID, authorizes the request coming from the AF and accordingly retrieves from the 5G Core the AF specific UE Id corresponding to the temporary ID.

NOTE 4: NEF retrieves the correct UDM instance from the UDM identifier in the Temporary ID, and NEF queries UDM to obtain the UE ID from the temporary ID.

Step 7. The NEF replies to the AF with the corresponding AF specific UE Id.

Step 8-9 (Optional). Assuming the application logic requires to fetch the UE location, a location request is sent to the NEF using the AF specific UE Id, and the corresponding information is provided by the 5G system through the NEF.

Step 10. The AF responds to the UE (EEC).

## X.4 	UE ID Token based solution

Figure X.3-1 illustrates the procedure to fetch the 5G system UE Id and private related information based on the usage of a UE ID token.

Figure X.4-1: Procedure to fetch the 5G system UE Id and privacy related information based on a UE ID token

Step 1: The client obtains a UE ID token from the UE ID server which is located inside the operator domain. UE ID server can identify the UE by using the private IP address of the PDU session. The UE ID Server can fetch the IP address from the IP packets received from the Client. The UE ID Server can use the Nbsf_Management_Discovery service operation with UE address and IP domain to retrieve the session binding information of the UE. For the UE ID Server service consumption by the client, the UE where the client is running on needs to execute the primary authentication successfully and have a PDU session. The client can authenticate the UE ID Server by using server certificate. The Client needs to be configured with the root certificate or certificate of the UE ID Server, or the Internet (public) PKI mechanism can be used. The client does not need to authenticate with the server since the server issues the UE ID token, which does not include cleartext sensitive information, to the client running on the UE who has already executed the primary authentication.

NOTE:	Since the issuer and resolver of the UE ID token is the UE ID Server, the details of the UE ID token (e.g., whether it is a signed token or a random number) can be left to implementation.

Step 2: The client forwards the token to the AF.

Step 3: The AF invokes a NEF service and use the UE ID token as UE identifier.

Step 4: The NEF interacts with the UE ID server to resolve the UE ID token into UE information (e.g., private IP address, MSISDN, SUPI) needed for the operation.

Step 5: The UE ID server returns the requested information.

Step 6: The NEF interacts with other NFs as per existing NEF service procedures as per TS 23.502 [X].

Step 7: The NEF returns the result of the NEF service execution.

Note that if the AF is a trusted AF, it can directly interact with the UE ID Server to identify the UE and the obtain required information such as IP address, and then invoke the NF/NEF directly with the required inputs.

## X.5 	Hash based solution

Figure X.3-1 illustrates the procedure to fetch the 5G system UE Id and private related information based on the usage of a fresh hash value with count, which is a changeable value shared between the UE and the 5GC.

Figure X.5-1: Procedure to fetch the 5G system UE Id and privacy related information based on a shared hash value

Step 0.	During the PDU session establishment procedure, SMF generates the salt, which is a random value, and then sends the salt to both UE and BSF. SMF can send the salt to the BSF along with SM policy Association Establishment or SM policy Association Modification message in clauses 4.16.4 or 4.16.5 of TS 23.502 [X]. If the SM policy Association Establishment and SM policy Association Modification procedures are not performed, SMF will not send the salt to the UE.

Step 1.	UE receives the UE ID request.

Step 2.	The UE generates a hash value based on the SUPI, IP address, salt, and count, and other values that is only known to both the EEC and BSF (e.g., DNN of the PDU Session, and S-NSSAI of the PDU Session). After that, the UE increases the count by 1, which is used for the next hash value generation procedure. Note that the count can be configured to start from 0 in each PDU session.

Note1: In order to mitigate the rainbow table attack, the following solutions should be adopted:

(1) using strong hash algorithms, e.g., SHA-256.

(2) increasing the length of the hash algorithm's input.

- The length of the salt can be set as 128bits to make the rainbow table attack infeasible [15].

- The input space of the SUPI is large enough to mitigate the rainbow table attack, considering that the length of IMSI is 15-16 digits, and NAI is 1-759 bits, as specified in TS 23.003 [16].

- Other inputs, including DNN of the PDU Session, and S-NSSAI of the PDU Session, can also help defend against the rainbow table attack.

(3) key stretching. The inputs can be run through the hash function multiple times to increase the time to build a precomputed rainbow table.

Note2: The UE can send the hash value and the count together to avoid the out-of-synchronization between the UE and BSF.

Step 3.	The UE sends the generated hash value and count to the AF.

Step 4.	The AF invokes the Nnef_UEId_Get request with the parameter of hash value, count, and IP address to get the UE ID.

NOTE 1: If private IP address is sent in step 3, then the same private IP address will be sent in step 4.

Steps 5-8.	This solution reuses Steps 3-6 in Clause 4.15.10, TS 23.502 [X].

NOTE 2: Steps 5-8 are workable only if NAT is in the UPF.

Step 9.	When the NEF invokes the Nbsf_Management_Discovery service operation, the NEF provides the hash value and IP address to the BSF.

Step 10.	The BSF determines the SUPI, salt and count based on the IP address. The BSF then generates the hash' value, and then compares the hash' value with the received hash value.

Step 11.	If the hash' value is equal to the received hash value, the BSF will respond to the NEF with a SUPI. Otherwise, the BSF will respond to the NEF with an indication that request is denied with the reason that the IP address is not associated with the UE.

Steps 12-14.	This solution reuse Steps 9-11 in Clause 4.15.10, TS 23.502 [X].

* * * End of Changes * * * *