3GPP TSG-SA3 Meeting #120	S3-250479

Athens, GREECE, 17-21 February 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.122 | CR |  | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Security solution for revocation at CAPIF-8 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3_sec |  |  |  |  |  | Date: |  |  | 2025-02-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Security details for CAPIF-8 were out of scope in Rel-18 and need to be addressed in Rel-19. |  |  |  |  |  |  |  |  |
| Summary of change: |  | CAPIF-8 solution on revocation in line with TR 33.700-22 conclusion. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No security solution on revocation for CAPIF-8. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.Y (new), 6.Y.1 (new), 6.Y.2 (new), 6.Y.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

******* START OF CHANGES

## 6.Y	Revocation of the API invoker authorization initiated by the RO

Pre-conditions:

1.	The API invoker is authenticated and authorized to use the service API.

2.	The AEF in the CAPIF is configured with the access policy to be applied to the service API invocation corresponding to the API invoker and the service API.

The following flow describes the procedure for revoking API invoker authorization initated by the RO.

1.  The resource owner may trigger token revocation through the ROF. The ROF sends an authorization revocation request message to the CCF. The message shall include the service API ID, the GPSI, and other information related to the revoked token (e.g., the scope info).

NOTE: The GPSI is the identifier of the resource owner. It is stored at the ROF through configuration or the authorization procedure.

2.	The CAPIF core function sends a revoke API invoker authorization request to the AEF with the details of the API invoker, the service API, and the GPSI of the UE on which the ROF resides.

3.  Upon receiving the information to revoke the API invoker's authorization for service API invocation, the AEF invalidates the API invoker authorization corresponding to the service API. The AEF may additionally determine whether to update the resource due to revocation, e.g., the API invoker is using the resource (i.e., QoS) that should be revoked after token revocation for the QoS service API, the AEF may inform PCF/SMF to modify the QoS level of corresponding PDU sessions after revocation.

4.  The AEF sends a revoke API invoker authorization response to the CAPIF core function.

5.  The CAPIF core function invalidates the API invoker authorization for the resource owner/UE corresponding to the GPSI.

6.  The CAPIF core function sends a revoke API invoker authorization notify to the ROF whose authorization to access the service API has been revoked.

### Figure 6.Y.3-1: Revocation procedure initiated by RO through ROF

******* END OF CHANGES