3GPP TSG-WG SA3 Meeting #120	S3-250525

# Athens, Greece,  17 Feb – 21 Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.401 | CR | 0723 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Security Considerations for Store and Forward (S&F) Satellite Operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_SEC |  |  |  |  |  | Date: |  |  | 2025-02-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduce new (sub)clause(s) or Annex in [TS 33.401] describing security requirements and procedures for Store and Forward Satellite Operation: |  |  |  |  |  |  |  |  |
| Summary of change: |  | State that normal EPS security procedures apply when S&F is used. Emphasize that communication links between satellite and ground proxies are outside 3GPP's scope. Clarify that the onboard MME obtains and stores EPS authentication vectors, ensuring the latest NAS security context is used. Highlight that enhancement to mitigate DoS attacks before full security establishment are FFS in normative work. Add clarifications on the timestamp mechanism used by the HSS to process location updates in S&F Mode: If an HSS does not support timestamps, the network shall reject the UE request in S&F Mode. Ensure the CR aligns with SA2-defined concepts (e.g., S&F Wait Timer, S&F Monitoring List) from a security perspective and references them where relevant. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent security handling for S&F satellite deployments, potentially leading to vulnerabilities or confusion about whether standard EPS security applies. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New Clause X.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First Change * * * *

## XX.1	Security of Store and Forward Satellite Operation

### XX.1.1	General

The present subclause defines the security mechanisms and procedures for E-UTRAN satellite deployments operating in Store and Forward (S&F) Mode. The requirements in this subclause are applicable when on-board network elements (e.g., on-board HSS, on-board MME) manage EPS security locally due to intermittent feeder link availability. In S&F Mode, the normal EPS security procedures specified in the present document shall apply. This includes EPS AKA, NAS Security Mode Command, and NAS security contexts.

### XX.1.2 On-Board HSS Deployment

In deployments where the HSS is placed onboard the satellite, the onboard HSS shall store the security credentials required for performing authentication procedures with UEs. The onboard HSS shall use standard EPS security mechanisms (e.g., EPS AKA) consistent with the present document. Operators may deploy IOPS-based keying (e.g., as described in Annex F of TS 33.401) to limit the exposure of long-term security credentials on the satellite. Implementations of IOPS-based keying remain outside the scope of this specification.

NOTE ZZ: Communications between satellite-based and ground-based proxies for application-level traffic (e.g., for MT/MO data or SMS delivery) are outside the scope of 3GPP security specifications.

### XX.1.2 S&F Operation with Split MME Architecture

In deployments where the MME is implemented in a split architecture, the on-board MME and the ground MME shall each fulfill their respective functional and security requirements as defined in the present specification. The ground network provides the on-board MME with the relevant EPS authentication vectors and subscription information. The NAS security shall terminate on the on-board MME when the UE is served via S&F Mode. The on-board MME shall store the latest NAS security context for each UE. Any distribution or synchronization of NAS security context between the onboard MME and the ground MME is outside the scope of this specification. When multiple satellites support S&F Mode, normal EPS security procedures shall be used for each satellite. In cases where a UE’s mutual authentication involves more than one on-board MME, the network shall ensure that each on-board MME obtains the necessary security credentials.

In deployments where the full core network resides onboard the satellite, the existing EPS security procedures defined in the present document shall apply without requiring additional normative enhancements for S&F Mode.

Editor’s Note: Enhancements to provide additional protection against DoS attacks before security establishment are FFS

Editor’s Note: Privacy enhancements that may be specific to split MME deployments are FFS

# * * * * End of changes * * * *