3GPP TSG-SA3 Meeting #120	S3-250531

# Athens, Greece, 17 -21 February 2025

Source:	OPPO

Title:	Pseudo-CR on Update conclusion on AIOT KI#5

Document for:	Approval

Agenda item:	5.9

Spec:	3GPP TR 33.713

Version:	0.6.0

Work Item:	FS_Ambient_IoT_Sec

Comments

SA2#166-Ad Hoc-e meeting sent LS S2-251242 to SA3 to include the following information:

“SA2 discussed about where to store the AIoT device profile data which contains device permanent ID and may also contain device credentials subject to SA3 decision. SA2 agreed on that ADM (Ambient IoT Data Management) is used to store such data.

SA2 kindly asks SA3 to take the above information into account and keep SA2 updated on the decision on whether to store the credential in the AIoT device profile data.”

It’s thus proposed to update the KI#5 conclusion about using ADM to store the device credential in the AIoT device profile.

Proposed Changes

* * * First Change * * * *

## 7.5	Conclusion on KI #5

1. 	The credential (including device ID and authentication credential) for AIoT device authentication can be owned by an operator or by a third party. The principles for device ID management are concluded in 8.2.1 in TR 23.700-13 [4].

2. 	For operator managed AIoT devices, the following principles are taken as the conclusion for KI#5:

Inventory-only Procedure:

- Authentication (i.e., at least one-way authentication; the 5G network authenticates the AIoT device) shall be supported.

NOTE 1: Details of the authentication procedure (e.g., one-way , parameter(s) used,  etc.)  are to be resolved during normative phase.

Inventory and Command Procedure:

- Mutual authentication between the AIoT device and the 5G network shall be supported.

- The authentication procedure is based on a challenge-response mechanism.

NOTE 2: Details of the mutual authentication procedure and details of the challenge-response mechanism(e.g., parameter(s) used for the 5G network to authenticate the AIoT device and for the AIoT device to authenticate the network) are to be resolved during normative phase.

Network Storage of AIoT device information:

ADM (Ambient IoT Data Management) shall be used to store the AIoT device credentials and related policy information. The AIoT device credentials and related policy information contains at least pre-shared long term key.

3. 	Additional considerations during normative phase:

NOTE 3:	Detail of the AIoT device profile data and AIoT device credentials and related policy information will be based on SA2 and SA3 coordination.

* * * End of Changes * * * *