3GPP TSG-SA3 Meeting #120	S3-250592

Athens, Greece, 17 - 21 February 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 2105 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE ID Token based mechanism for 33501 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph3 |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Implements TR 33.749 KI#2.1 conclusion into TS 33.501. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Based on TR 33.749 KI#2.1 conclusion, proposes to add the principles and the UE ID token based solution as an alternative mechanism in a new annex in TS 33.501. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TR 33.749 KI#2.1 conclusion will not be applied. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex ZZZ (new), Annex ZZZ.0 (new), Annex ZZZ.Y (new), Annex ZZZ.Y.1 (new), Annex ZZZ.Y.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Start of the change ***

# Annex ZZZ (informative): Mechanisms for secure retrieval of 5G system UE Ids and privacy related information

## ZZZ.0	General

This clause describes the alternative mechanisms for secure retrieval of 5G system UE Ids and privacy related information. They align with the priciples that decribed below.

-	The information used in Nnef_UEId API call invocation and UE Identifier API call invocation needs to prevent the use of permanent or semi-permanent UE identifiers subject to be abused/compromised by malicious applications.

-	The entity responsible for the generation of the information needs to be part of the 5G system in HPLMN and needs to be verified by the 5G system in HPLMN. Alternatively, the information can be generated by both the UE and the 5G system.

-	The information can be used by the AF invoking the NEF APIs, so that the NEF APIs can identify the UE by using the information.

NOTE 1:	The API invoker does not need to obtain the permanent 5G UE ID, which improves the privacy.

NOTE 2:	The principles above do not prevent usage of existing mechanisms of Nnef_UEId  API and UE Identifier API.

## ZZZ.Y	UE ID Token based mechanism

### ZZZ.Y.1	Overview

With the usage of UE ID token, there will be no need to share private IP address with EEC and other entities in the EC architecture, which increases privacy protection. In addition, the mechanism prevents IP spoofing attacks. The mechanism works also for the case that the NAT operation is performed after the UPF. In section ZZZ.Y.2, the generic view of the mechanism is presented and how the mechanism can work for the following special cases of NEF API invocation by the EAS, NEF API invocation by the EES, and EES API invocation by the EAS is explained.

### ZZZ.Y.2	Details

Generic representation of the mechanism is presented in Figure ZZZ.Y.2-1.

Figure ZZZ.Y.2-1: Generic view of UE ID token based mechanism

Step 1: The client obtains a UE ID token from the UE ID server which is located inside the operator domain. UE ID server can identify the UE by using the private IP address of the PDU session. The UE ID Server can fetch the IP address from the IP packets received from the Client. The UE ID Server can use the Nbsf_Management_Discovery service operation with UE address and IP domain to retrieve the session binding information of the UE. For the UE ID Server service consumption by the client, the UE where the client is running on needs to execute the primary authentication successfully and have a PDU session. The client can authenticate the UE ID Server by using server certificate. The Client needs to be configured with the root certificate or certificate of the UE ID Server, or the Internet (public) PKI mechanism can be used. The client does not need to authenticate with the server since the server issues the UE ID token, which does not include cleartext sensitive information, to the client running on the UE who has already executed the primary authentication.

NOTE 1:	Since the issuer and resolver of the UE ID token is the UE ID Server, the details of the UE ID token (e.g., whether it is a signed token or a random number) can be left to implementation.

Step 2: The client forwards the token to the AF.

Step 3: The AF invokes a NEF service and use the UE ID token as UE identifier.

NOTE 2:	The UE ID token can be provided as input in any NEF Service that takes GPSI as input (as NAI/External Id format).

Step 4: The NEF interacts with the UE ID server to resolve the UE ID token into UE information (e.g., private IP address, MSISDN, SUPI) needed for the operation.

Step 5: The UE ID server returns the requested information.

Step 6: The NEF interacts with other NFs as per existing NEF service procedures as per TS 23.502.

Step 7: The NEF returns the result of the NEF service execution.

Note that if the AF is a trusted AF, it can directly interact with the UE ID Server to identify the UE and the obtain required information such as IP address, and then invoke the NF/NEF directly with the required inputs.

Application of the mechanism to the case of NEF API invocation by the EAS/EES is presented in Figure 6.9.2-2.

Figure ZZZ.Y.2-2: The EAS/EES invokes NEF API

Step 1: The AC/EEC obtains a UE ID token from the UE ID server.

Step 2: The AC or the EEC respectively forward the token to the EAS or the EES.

Step 3: The EAS/EES invokes a NEF service and use the UE ID token as UE identifier.

Step 4: The NEF interacts with the UE ID server to resolve the UE ID token into UE information (e.g., private IP address, MSISDN, SUPI) needed for the operation.

Step 5: The UE ID server returns the requested information.

Step 6: The NEF interacts with other NFs as per existing NEF service procedures as per TS 23.502.

Step 7: The NEF returns the result of the NEF service execution.

Application of the mechanism to the case of EES API invocation by the EAS is presented in Figure 6.9.2-3.

Figure ZZZ.Y.2-3: The EAS invokes the EES API

Step 1: The AC obtains a UE ID token from the UE ID server.

Step 2: The AC forwards the token to the EAS.

Step 3: The EAS invokes a EES service and use the UE ID token as UE identifier.

Step 4: The EES invokes the NEF API by sending the UE ID token.

Step 5: The NEF interacts with the UE ID server to resolve the UE ID token into UE information (e.g., private IP address, MSISDN, SUPI) needed for the operation.

Step 6: The UE ID server returns the requested information.

Step 7: The NEF sends the UE ID to the EES.

Step 8: The EES returns the result of the EES service execution.

*** End of the change ***