3GPP TSG-SA3 Meeting #120	S3-250623

# Athens, Greece, 17 - 21 February 2025

Source:	China Mobile

Title:	Pseudo-CR on New solution on AIoT privacy

Document for:	Approval

Agenda item:	5.9

Spec:	3GPP TR 33.713

Version:	0.6.0

Work Item:	FS_Ambient_IoT_Sec

Comments

This contribution proposes a new Temp ID based privacy protection solution. This solution proposes to generate Temp ID by AIoT device. Compared to the solution with Temp Id generated by 5GC, this solution can avoid recurring of writing signaling due to failure of writing operation or AIoT device out of power. Frequent writing operations shall be avoided based on LS from RAN1(R1-2405796)

Proposed Changes

* * * First Change * * * *

6.Y	Solution #Y: Temp ID based privacy protection for Ambient IoT device identifier

6.Y.1	Introduction

This solution addresses key issue #3 on privacy by protecting AIoT device identifiers. It is based on a temp ID generated by Ambient IoT device. Compared to the solutions with temp ID assigned by 5GC, this solution can avoid frequent sync-up procedures (writing operations) between 5GC and Ambient IoT Device due to failure of writing operation or AIoT device out of power. The main points of this solution are as follows:

1)  The ambient IoT device assigns the temp ID, not 5GC.

2)  The Ambient IoT temp ID is synced between the ambient IoT device and 5GC by standard inventory procedure.  Thus, there is no writing signaling between 5GC and the Ambient IoT Device.

3)  5GC will obtain and maintain the binding relationship between temp ID and permanent device ID via standard inventory procedure.

4)  5GC will add an “ID response indication” parameter in the standard inventory message to instruct ambient IoT devices to respond with only Temp ID or both Temp ID and permanent device ID to realize uplink privacy protection

5) 5GC can use Temp ID to replace permanent device ID as the target ambient device filter info in inventory message from 5GC to ambient IoT devices to realize downlink privacy protection

6.Y.2	Solution details

Temp ID generation and sync up procedure between device and 5GC

Figure x: Procedure for Temp ID generation and sync up between device and 5GC

Downlink and Uplink Privacy protection based on Temp ID

Figure y: Procedure for Downlink and Uplink Privacy protection based on Temp ID

Temp ID update and sync up procedure

Figure z: Procedure for Temp ID update and sync up

The figures are self-explanatory, therefore, the steps are not explained step-by-step.

Highlights on Solution:

  - Don’t need additional registration procedure to trigger Temp ID allocation

  - Don’t need write signaling to sync up Temp ID between 5GC and ambient IoT Devices

  - Temp ID is synced up between ambient IoT device and 5GC by standard inventory procedure (read operation)

  - Improve data transportation efficiency by using Temp ID (maybe 20-30 bits) to replace Permanent device ID (average ID length 96bits – 496bits)

  - Temp ID solution can be enabled based on AF ID

  - Overhead: Need to add an additional parameter, ID response indication, to instruct the ambient IoT device how to respond to the inventory.

6.Y.3	Evaluation

Editor’s Note: Further evaluation is FFS

* * * End of Changes * * * *