3GPP TSG-SA3 Meeting #120	S3-250642

# Athens, Greece, 17 - 21 February 2025

Source:	Sony

Title:	KI#4, Conclusions

Document for:	Approval

Agenda item:	5.9

Spec:	3GPP TR 33.713

Version:	0.6.0

Work Item:	FS_AIOT_Sec

Comments

The Ambient IoT Check point at TSG SA#106 resulted in Way Forward in SP-241980.

The Way Forward includes important aspects on how to focus the work in Q1 to reach conclusions that can be implemented during the normative phase during Q2 and Q3, 2025.

The most important guidance is to focus the work on device type 1 (the most constrained device type) and topology 1 (base station reader), as this is what RAN will specify in Rel-19 according to approved WID in RP-243326.

Proposal 1: As device type 1 is very constrained it can be concluded that these devices cannot use an USIM. Solution 11 and 12 cannot be considered.

Observation: In rel-19 it is important to minimize the computational requirements as only device type 1 will be supported.

There are two types of Ambient IoT services specified in rel-19: Inventory and Command. The “information” to protect for the Inventory service is the identity of the AIoT device. In KI#3 the protection of the device identity will be concluded. Therefore, the focus for KI#4 shall be on how to protect the information shared during the Command service.

As the information in the command message may be limited due to the type of AIoT device (device type 1) and its capabilities, the most important for the service is that both the AIoT device and the network must be able to authenticate the message received. The content of the command may not be that sensitive, but it must not be possible to tamper with the content e.g. an adversary shall not be able to change a command in the request to a permanently disable command.

In TR 33.713 v.0.6.0 there are 10 solutions (incl 11 and 12) documented related to KI#4 Protection of information during AIoT service communication.

Solution Investigation (mapped to KI#4 in table 6.1-1 excluding solution 11 and 12)

Solution 13: A Nonce, SQN and MAC added to the Inventory request message (initial steps of the procedure in this solution, the steps before the command request), this is not in line with RAN 2 assumption that only an ID is part of that message. However, the rest of the procedure in 6.13.2 can be considered.

Solution 14: For Command protection, MAC is proposed to be used.

Solution 15: Propose to encrypt the Command and if device can decrypt the command message it can also verify the message.

Solution 32: Use L1 radio channel measurements to generate RAND used for authentication. Several ENs are captured indicating that this concept needs more work especially in RAN1.

Solution 33: Use L1 radio channel measurements to generate L1-key used for generating session keys Kiot which is then used to protect the command message. Several ENs are captured indicating that this concept needs more work especially in RAN1.

Solution 35: A Nonce is added to the paging message, which is not in line with RAN 2 assumption. The rest of the procedure in 6.35.2 can be considered.

Solution 40: Propose that a network NONCE is included in the command request message. The AIoT device generate a Device NONCE and use both NONCEs to derive two session keys (one for encryption and one for integrity protection). Several operations for a device type 1 AIoT device.

Solution 42: The AIoT device selects a preconfigured Nonce from a list of Nonce values and derives a session key. The session key is used to encrypts the device ID and the data. Furthermore, a HASH generated of the data. Some security concerns are captured in ENs related to the preconfigured list of Nonce values.

Proposal 2: Using L1 radio channel measurements for RAND/key generation as proposed in solution 32 and 33 would require significant investigation and involve other WG groups. As this is the last meeting of the study it is proposed that solutions 32 and 33 cannot be considered.

Observation: The principles of the remaining solutions that are quite similar, but there are differences in certain aspects e.g. MAC or HASH, freshness parameter provided over-the-air or not, Session key or shared key is used.

Solution 13 could provide a good baseline for protecting the Command service.

Figure: Capture steps 18 – 20 from solution 13 figure 6.13.2-1 shows the steps related to the Command request and Command response.

The network provides a set of parameters to the AIoT device, that the AIoT device uses to validate the MAC and generate an encryption key (session Key). The device shall also generate a new Temp ID to be used in the next service operation. A possibility to reduce the complexity could be to use Temp ID#2 as key when protecting the content instead of a Encryption key, that would require the device to have one less derivation to perform. Such simplification on key management could be considered as the amount of traffic and data generated over the radio interface would be limited, making the use of brute force to crack the security not possible as the data available for that approach is limited.

Temp ID#1

Temp ID#2

Temp ID#1

Figure: Capture from solution 13 figure 6.13.2-5 and 6.13.2-6, with the modification discussed

Details of the MAC generations and the protection of the payload can be detailed during the normative phase.

Proposal 3: Use solution 13 as baseline but consider some simplifications for device type 1.

Proposed Changes

* * * First Change * * * *

## 7.4	Conclusion on KI #4

For key issue #4 (Protection of information during AIoT service communication) described in clause 5.4, it is concluded that the normative work is based the following principles on how to protect the Inventory and Command services:

- For the Inventory service, the “information” to be protected is the identity of the AIoT device and the protection of the device identity shall follow the principles concluded for KI#3.

- For the Command service the “information protection” is based on the following:

  - The Command request message (sent after the Inventory request and response) includes parameters used to validate the MAC in the Command request message.

  - The AIoT device may have or is able to derive the Session Key used by the network (AIOTF) to protect the payload in the command request message. The same key is used by the AIoT device to protect the payload of the Command Response message.

  - Parameters in the Command request or updated parameters e.g., increase of sequence number, is used by the AIoT device to generate a MAC for the Command Response message.

  - The Command response message may include parameters e.g., new sequence number that the network (AIOTF) uses to validate the MAC.

NOTE:	The details on the payload protection, MAC generation and parameters used will be specified during the normative work. Figure 6.13.2-5 and 6.13.2-6 could be used as basis. Simplifications of the solutions in clause 6.13.2, if possible, shall be considered for device type 1.

* * * End of Changes * * * *