3GPP TSG-SA3 Meeting #120	S3-250737
Athens, Greece, 17 - 21 February 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 2107 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Protecting XRM Media related information in QUIC aware forwarding |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | DUMMY |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For Extended Reality and Media service (XRM), data streams are end-to-end (e2e) encrypted when sent from an external Application Server (AS) to the UE via the UPF. To be able to select the correct QoS setting, the UPF needs access to Media related information sent together with the e2e encrypted data. The Media related information is encrypted between the AS and the UPF. The details on how this encryption is performed and what security context to use is not specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding general clasue and details on how Media related information is encrypted and integrity protected betewen AS and UPF in case the UPF deploys a proxy with QUIC aware forwarding. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Protection of Media related information is left unspecified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, X (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF CHANGES ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System".

[3]	3GPP TS 33.210: "3G security; Network Domain Security (NDS); IP network layer security".

[4]	IETF RFC 4303: "IP Encapsulating Security Payload (ESP)".

[5]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[6]	IETF RFC 4301: "Security Architecture for the Internet Protocol".

[7]	3GPP TS 22.261: "Service requirements for next generation new services and markets".

[8]	3GPP TS 23.502: "Procedures for the 5G System".

[9]	3GPP TS 33.102: "3G security; Security architecture".

[10]	3GPP TS 33.401: "3GPP System Architecture Evolution (SAE); Security architecture".

[11]	3GPP TS 33.402: "3GPP System Architecture Evolution (SAE); Security aspects of non-3GPP accesses".

[12]	IETF RFC 5448: " Improved Extensible Authentication Protocol Method for 3rd Generation Authentication and Key Agreement (EAP-AKA')".

[13]	3GPP TS 24.301: " Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[14]	3GPP TS 35.215: " Specification of the 3GPP Confidentiality and Integrity Algorithms UEA2 & UIA2; Document 1: UEA2 and UIA2 specifications".

[15]	NIST: "Advanced Encryption Standard (AES) (FIPS PUB 197)".

[16]	NIST Special Publication 800-38A (2001): "Recommendation for Block Cipher Modes of Operation".

[17]	NIST Special Publication 800-38B (2001): "Recommendation for Block Cipher Modes of Operation: The CMAC Mode for Authentication".

[18]	3GPP TS 35.221: " Specification of the 3GPP Confidentiality and Integrity Algorithms EEA3 & EIA3; Document 1: EEA3 and EIA3 specifications".

[19]	3GPP TS 23.003: "Numbering, addressing and identification".

[20]	3GPP TS 22.101: "Service aspects; Service principles".

[21]	IETF RFC 4187: "Extensible Authentication Protocol Method for 3rd Generation Authentication and Key Agreement (EAP-AKA)".

[22]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[23]	3GPP TS 38.323: "NR; Packet Data Convergence Protocol (PDCP) specification".

[24]	3GPP TS 33.117: "Catalogue of general security assurance requirements".

[25]	IETF RFC 7296: "Internet Key Exchange Protocol Version 2 (IKEv2)"

[26]	Void

[27]	IETF RFC 3748: "Extensible Authentication Protocol (EAP)".

[28]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)".

[29]	SECG SEC 1: Recommended Elliptic Curve Cryptography, Version 2.0, 2009. Available http://www.secg.org/sec1-v2.pdf

[30]	SECG SEC 2: Recommended Elliptic Curve Domain Parameters, Version 2.0, 2010. Available at http://www.secg.org/sec2-v2.pdf

[31]	3GPP TS 38.470: "NG-RAN; F1 General aspects and principles".

[32]	3GPP TS 38.472: "NG-RAN; F1 signalling transport".

[33] 	3GPP TS 38.474: "NG-RAN; F1 data transport".

[34]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)"

[35]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[36] 	3GPP TS 35.217: "Specification of the 3GPP Confidentiality and Integrity Algorithms UEA2 & UIA2; Document 3: Implementors' test data".

[37] 	3GPP TS 35.223: "Specification of the 3GPP Confidentiality and Integrity Algorithms EEA3 & EIA3; Document 3: Implementors' test data".

[38]	IETF RFC 5216: "The EAP-TLS Authentication Protocol".

[39]	Void

[40]	IETF RFC 5246: "The Transport Layer Security (TLS) Protocol Version 1.2".

[41]	3GPP TS 38.460: "NG-RAN; E1 general aspects and principles".

[42]	Void.

[43]	IETF RFC 6749: "OAuth2.0 Authorization Framework".

[44]	IETF RFC 7519: "JSON Web Token (JWT)".

[45]	IETF RFC 7515: "JSON Web Signature (JWS)".

[46]	IETF RFC 7748: "Elliptic Curves for Security".

[47]	IETF RFC 9113: "HTTP/2".

[48]	IETF RFC 5280: "Internet X.509 Public Key Infrastructure Certificate and Certificate Revocation List (CRL) Profile".

[49]	IETF RFC 6960: "X.509 Internet Public Key Infrastructure Online Certificate Status Protocol - OCSP".

[50]	IETF RFC 6066: "Transport Layer Security (TLS) Extensions: Extension Definitions".

[51]	3GPP TS 37.340: "Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; Stage 2".

[52]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[53]	3GPP TS 33.122: "Security Aspects of Common API Framework for 3GPP Northbound APIs".

[54]	3GPP TS28.533: " Management and orchestration; Architecture framework".

[55]	3GPP TS28.531: "Management and orchestration of networks and network slicing; Provisioning".

[56]	Void

[57]	IETF RFC 7542: "The Network Access Identifier".

[58]	IETF RFC 6083: " Datagram Transport Layer Security (DTLS) for Stream Control Transmission Protocol (SCTP)".

[59]	IETF RFC 7516: "JSON Web Encryption (JWE)".

[60]	IETF RFC 8446: "The Transport Layer Security (TLS) Protocol Version 1.3".

[61]	IETF RFC 5705,"Keying Material Exporters for Transport Layer Security (TLS)".

[62]	IETF RFC 5869 "HMAC-based Extract-and-Expand Key Derivation Function (HKDF)".

[63]	NIST Special Publication 800-38D: "Recommendation for Block Cipher Modes of Operation: Galois Counter Mode (GCM) and GMAC".

[64]	IETF RFC 6902: "JavaScript Object Notation (JSON) Patch".

[65]	3GPP TS 31.115: "Secured packet structure for (Universal) Subscriber Identity Module (U)SIM Toolkit applications.

[66]	3GPP TS 31.111: "Universal Subscriber Identity Module (USIM), Application Toolkit (USAT)".

[67]	IETF RFC 9048: "Improved Extensible Authentication Protocol Method for 3GPP Mobile Network  Authentication and Key Agreement (EAP-AKA')".

[68]	3GPP TS 29.510: "5G System; Network function repository services".

[69]	3GPP TS 36.331: "Radio Resource Control (RRC); Protocol specification".

[70]	3GPP TS 29.505: "5G System; Usage of the Unified Data Repository services for Subscription Data; Stage 3".

[71]	3GPP TS 24.302: "Access to the 3GPP Evolved Packet Core (EPC) via non-3GPP access networks; Stage 3".

[72]	3GPP TS 23.216: "Single Radio Voice Call Continuity (SRVCC)".

[73]	3GPP TS 29.573: " Public Land Mobile Network (PLMN) Interconnection; Stage 3".

[74]	3GP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[75]	IEEE TSN network aspects: see 3GPP TS 23.501 [2] references [95], [96], [97], [98], [104], and [107].

[76]	IETF RFC 9190: "EAP-TLS 1.3: Using the Extensible Authentication Protocol with TLS 1.3".

[77]	IETF RFC 8446: "The Transport Layer Security (TLS) Protocol Version 1.3".

[78]	3GPP TS 38.401: "NG-RAN; Architecture description".

[79]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)"

[80]	IEEE Std 802.11-2016 (Revision of IEEE Std 802.11-2012) - IEEE Standard for Information technology—Telecommunications and information exchange between systems Local and metropolitan area networks—Specific requirements - Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications.

[81]	IETF RFC 2410 "The NULL Encryption Algorithm and Its Use With IPsec".

[82]	Void

[83]	RFC 7858: "Specification for DNS over Transport Layer Security (TLS)".

[84]	RFC 8310: "Usage Profiles for DNS over TLS and DNS over DTLS".

[85]	RFC 4890: "Recommendations for Filtering ICMPv6 Messages in Firewalls".

[86]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[87]	3GPP TS 38.305: "Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[88]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRAN); Overall description; Stage 2".

[89]	IANA: "Transport Layer Security (TLS) Parameters".

[90]	Void

[91]	3GPP TS 33.535: "Authentication and key management for applications based on 3GPP credentials in the 5G System (5GS)".

[92]	3GP TS 29.573: "5G System; Public Land Mobile Network (PLMN) Interconnection".

[93]	3GPP TS 29.503: "5G System; Unified Data Management Services".

[94]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition".

[95]	3GPP TS 29.502: "5G System; Session Management Services".

[96]	3GPP TS 29.526: "5G System; Network Slice-Specific Authentication and Authorization (NSSAA) services".

[97]	3GPP TS 23.402: "Authentication enhancements for non-3GPP accesses".

[98]	3GPP TS 23.548: "5G System Enhancements for Edge Computing; Stage 2".

[99]	RFC 5281: "Extensible Authentication Protocol Tunneled Transport Layer Security              Authenticated Protocol Version 0 (EAP-TTLSv0)".

[100]	RFC 6678: "Requirements for a Tunnel-Based Extensible Authentication Protocol (EAP) Method".

[101]	General Data Protection Regulation, https://eur-lex.europa.eu/legal-content/EN/TXT/HTML/?uri=CELEX:02016R0679-20160504&from=EN.

[102]	3GPP TS 33.246: "Security of Multimedia Broadcast/Multicast Service (MBMS)".

[103] 	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services".

[104]	3GPP TS 33.535: "Authentication and Key Management for Applications (AKMA) based on 3GPP credentials in the 5G System (5GS)".

[105]	3GPP TS 23.288: "Architecture enhancements for 5G System(5GS) to support network data analytics services".

[106]	3GPP TS 23.554 Application architecture for MSGin5G Service; Stage 2.

[107]	3GPP TS 22.262 Message service with the 5G System (5GS); Stage 1.

[108]	3GPP TS 26.502: "5G multicast–broadcast services; User Service architecture".

[109]	3GPP TS 33.503: "Security Aspects of Proximity based Services (ProSe) in the 5G System (5GS)".

[110]	NIST Special Publication 800-90A (2015): "Recommendation for Random Number Generation Using Deterministic Random Bit Generators".

[111]	IETF RFC 4555 (2006-06): "RFC IKEv2 Mobility and Multihoming Protocol (MOBIKE)".

[112]	3GPP TS 24.008: "Mobile radio interface Layer 3 specification; Core network protocols; Stage 3".

[113]	RFC 9110: "HTTP Semantics".

[114]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[115]	IETF RFC 9000: "QUIC: A UDP-Based Multiplexed and Secure Transport".

[116]	IETF RFC 9001: "Using TLS to Secure QUIC".

[117]	draft-ietf-quic-multipath: "Multipath Extension for QUIC".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[XX1]	IETF RFC 9298: "Proxying UDP in HTTP"

[XX2]	draft-ietf-masque-quic-proxy

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[XX3]	https://www.iana.org/assignments/tls-parameters/tls-parameters.xhtml#exporter-labels

[XX4]	SP 800-38C The CCM Mode for Authentication and Confidentiality

[XX5]	IETF RFC 9297: "HTTP Datagrams and the Capsule Protocol"

[XX6]		IETF RFC 9525: "Service Identity in TLS"

[XX7]	IETF RFC 9114: "HTTP/3"

*** NEXT CHANGE ***

# X		Protection of XRM Media related information

## X.1		General

This clause describes how Media related information is protected when transferred from an external Application Server to the UPF. For more information, see clause 5.37.9 of TS 23.501 [2].

The Media related information is transferred together with the e2e encrypted data sent between AS and the UE. The Media related information is encrypted and integrity protected between the AS and the UPF.

Sub clause X.Y describes the procedures when QUIC aware forwarding is used.

Editor’s note: Security details for the other agreed methods are to be added in separate sub clauses.

## X.Y 	Protection of Media related information when using QUIC aware forwarding

### X.Y.1	Architecture and protocols

End-to-end encrypted XRM application data is sent between an Application Server (AS) and a UE. The application data is sent using QUIC over UDP.

Media related information (containing for example PDU Set Information), is sent by the AS together with downlink packets containing XRM application data. The media related information is consumed by the UPF. In order to both encode and protect the media related information, an HTTP/3 connection [XX7] is established between the UPF and AS, where individual end-to-end connections can be tunnelled by use of the HTTP CONNECT method with the ‘connect-udp’ upgrade token [XX1]. With this type of tunnelling, XRM packets are encoded as HTTP datagrams within the tunnel, the HTTP datagrams can further contain the XRM media related information.

When the end-to-end traffic is based on QUIC, the Forward Mode in draft-ietf-masque-quic-proxy [XX2] can be used as an optimisation. In forward mode, end-to-end QUIC packets can be sent in “parallel” to the tunnel, which has the benefit of not having to encrypt the e2e encrypted data twice, see Figure X.Y.1-1. Forwarded packets can be transformed with packet transform mechanisms that are negotiated in the CONNECT request. For XRM data, a packet transform is used where Media related information is appended to the packet and protected. The AS appends and encrypts the media related information, the UPF decrypts and consumes the media related information and removes it before forwarding the end-to-end packet towards the UE.

Figure X.Y.1-1: Overview of the different connections. XRM application data is e2e encrypted between the AS and the UE. The protected Media related information is appended to the XRM packets and transferred secured by the security context setup between the UPF and AS.

### X.Y.2	Security of the QUIC connection between UPF and AS proxy

The UPF acts as HTTP/3 client and establishes a QUIC connection to the AS Proxy (HTTP Server). The necessary parameters such as AS proxy connect address and certificate are received as part of the Forwarding Action Rules from the PCF (via SMF). The connect address is the URL for the HTTP/3 tunnel server. The UPF shall match the host component of the URL against the identifier of the server certificate as described in RFC 9114 [XX7] and RFC 9525 [XX6]. The certificates shall adhere to the certificate profiles described in clause 6.1.3a of TS 33.310 [5]. The TLS 1.3 handshake and negotiated cipher suites shall adhere to the TLS 1.3 profile described in clause 6.2.2 of TS 33.210 [3].

### X.Y.3	Transformed packet

The QUIC aware forwarding mode [XX2] negotiates a packet transform that modifies the packet in addition to the minimal necessary to make the forwarding work. The QUIC-aware forwarding replaces the Connection ID (CID) field with the virtual CID (VCID) field and rewrites the IP/UDP header so that the packet arrives at the tunnel endpoint (UPF), rather than the end-to-end target (UE). An overview of the transformed packet is provided in Figure X.Y.3-1.

The packet transform adds the following data (see also Figure X.Y.3-1):

- The Media related information, protected using AES-CCM [XX4]

- Counter LSB

- A length field indicating the length of the protected Media related information

The added data is placed at the end of the packet for ease of adding and removing it. A length field indicates the length in bytes of the protected media related information. The Counter field includes the sixteen least significant bits of a 64-bit counter used to construct the nonce for the authenticated encryption. Note that only the necessary parts of the counter are transferred to minimize data overhead. For further details on the nonce construct, see clause X.Y.6.

This specific transformation is labelled xrm-aesccm128_64 in the extended connect HTTP request. The Label shall be an IANA registered label.

NOTE X: The transform has a dedicated encryption mode. Cipher algorithm agility is handled by defining a new transform version with a new transform label indicating a different cipher algorithm.

If there is no Media related information, the transform shall set the length field to zero (0), and no other additional fields (Counter LSB or Media related information) are inserted into those packets.

The fields in order from the end of the packet towards the beginning:

Media related information (MRI) Length field: An 8-bit unsigned integer indicating the number of bytes of the protected Media related information (including integrity tag).

Counter LSB field: Contains the 16-bit least significant bits of a 64-bit counter used to construct the nonce to the AEAD protection. The 64-bit counter is incremented with one for each payload protected by the transform for the current VCID. The 64-bit counter is initialized to zero (0) for each VCID. For more information about the counter, see clause X.Y.6.

Protected Media related information: The N bytes of AEAD output after protection operation using the below defined AEAD algorithm.

Figure X.Y.3-1: Diagram over original and transformed end-to-end packet

### X.Y.4	Encryption mode

Media related information is encrypted and integrity protected using an AES in CCM [XX4] mode with 128-bit key, a 96-bit nonce, and a truncated 64-bit authentication tag. CCM provides authenticated encryption with the option of having additional data. No additional data is used.

### X.Y.5	Key export

The VCID shall be unique per direction, see clause X.Y.7. Unique keys shall be established for each VCID value used between the UPF and AS proxy.

The keys are extracted from the QUIC connection between UPF and AS proxy using the TLS 1.3 key exporter defined in RFC 8446 [77].

The following input shall be used for the TLS 1.3 key exporter:

Label: ‘EXPORTER-MASQUE-Transform-xrm-AESCCM128_64’ (To be registered with IANA, see [XX3])

Context_value: The VCID value || The forty (40) most significant bits of the counter

Key_length: 128 bits

The produced key is the key that is used for all packets with media related information with a full 64-bit counter within the range of 2^24 of packets that has the same forty most significant bits.

To ensure that the key invocation limit for the XRM meta data protection is not reached, a new key is exported every 2^24 invocations. This is well within the recommended limit of 2^61 blocks [XX4].

NOTE X: Since multiple keys may be exported for each VCID, the higher (most significant) 40 bits from the counter is used as input to the key export function to ensure uniqueness of the input to the key export.

### X.Y.6	Nonce and counter values

For each invocation of AES-CCM, a 96 bit nonce is created by concatenating the last (rightmost) 32 bits of the VCID (VCID’) with the 64-bit counter C: VCID’||C.

To maintain synchronization even for large burst losses of XRM transformed packets, the full 64-bit counter is sent every time the 16-bit Least Significant Bit counter wraps. The counter is sent reliably as a HTTP capsule protocol [XX5] capsule on the corresponding HTTP/3 request QUIC stream.

The receiver of the transformed packet, i.e. the UPF for AS to UE traffic, shall perform replay protection to ensure that each 64-bit counter is only accepted once. In case the receiver detects reuse of the counter, the e2e packet shall be discarded (i.e. not forwarded).

NOTE X+1: The connect-UDP capsule carrying the full 64-bit counter needs to be defined in Stage 3.

### X.Y.7	VCID uniqueness

In forward mode, the VCID is selected by the proxy to ensure unique values that maps between external 4-tuple (IP addresses and ports) towards the target, end-to-end CID and thus direction. Further, the VCID value shall never be reused for a particular HTTP/3 sessions’ QUIC connection. This as the key used to encrypt and integrity protect the Media related information is derived from the QUIC connection’s established TLS session.

To avoid introducing any linkability between different VCID in case the VCID is changed for a particular end-to-end QUIC connection, a fresh key shall be exported for each VCID value and the counter shall be reset.

*** END OF CHANGES ***