3GPP TSG-SA3 Meeting #120	S3-250754

Athens, Greece, 17 - 21 February 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 2108 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | VFL Authorization when the External AF acts as a VFL Server |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN_SEC |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to the conclusion of KI#2 in the TR 33.784 v19.0.0, we propose to specify the authorization for Vertical Federated Learning (VFL) participants when external AF is VFL Server, and NWDAF(s) are VFL client(s) (NEF is involved) in a normative Annex in TS 33.501. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In case that external AF is VFL Server, and NWDAF(s) are VFL client(s) (NEF is involved): 1) The NEF includes requested analytics ID and AF ID in token request to NRF. The NRF checks whether the server AF is authorized to access the client NWDAF(s) for this Analytics ID. If the server AF is authorized, the NRF will issue an access token to the NEF. The NEF uses the access token to communicate with client NWDAF(s), and the NWDAF(s) verifies the access token, after successful verification, the NWDAF allows server AF’s request from the NEF. 2) The NRF uses the information included in token request and registered information for the authorization. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | VFL Authorization when the External AF acts as a VFL Server is not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex X.X1(New), Annex X.X1.1(New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***START OF THE CHANGE***

Annex X (normative):
Security aspects of enablers for Network Automation (eNA) for the 5G system (5GS)

# X.1	General

This Annex provides security requirements and procedures for the Network Automation features.

The feature for enablers for Network Automation by 5GS is described in 3GPP TS23.501[2] and 3GPP TS23.288 [105].

…

# X.10	Security for AI/ML model storage and sharing

The detailed procedure for secured and authorized AI/ML model sharing between different vendors is depicted in Figure X.10-1:

Figure X.10-1: Secured and authorized AI/ML model sharing between different vendors

0a. NF Service producer i.e. NWDAF containing MTLF registers its NF profile in the NRF with ML Model Interoperability indicator per Analytics ID as described in clause 5.2 of TS 23.288 [105]. The ML Model Interoperability indicator is a list of NWDAF providers (vendors) that are allowed to retrieve ML models from this NWDAF containing MTLF.

0b. NF Service consumer e.g., NWDAF registers at the NRF including its Vendor ID, if the NF Service consumer is MTLF, the NWDAF containing MTLF also registers its Interoperability indicator per Analytics ID.

0c. The model is stored in encrypted format unless both the AI/ML model producer (NWDAF MTLF) and storage platform (ADRF) are part of the same system and belong to the same vendor and operator security domain.

Storage of the model in encrypted format can be required by the trust model established to store and share AI/ML models. The trust model between AI/ML NF producer (NWDAF MTLF), storage platform (ADRF) and NF consumer (e.g., AnLF) is to be determined during the implementation phase among operator and the providers of the different platforms (MTLF, AnLF, ADRF). How the model is encrypted is vendor specific. Key distribution is not specified in this document.

1. 	If NWDAF containing MTLF determines to store ML model in ADRF, NWDAF containing MTLF triggers the Nadrf_MLModelManagement_StorageRequest as described in TS 23.288 [105], optionally including an allowed NFc list. The absence of allowed NFc list indicates that only the MTLF which stored the model is allowed to retrieve the model.

2. 	ADRF sends the response to NWDAF containing MTLF as described in TS 23.288 [105].

3. 	NF Service consumer e.g., NWDAF performs Nnrf_NFDiscovery_Request operation with the requested Analytics ID to select a suitable NF Service Producer e.g., NWDAF containing MTLF.

In case of the NF service consumer (e.g. NWDAF containing MTLF) request ML models on behalf of another ML model consumer (e.g., NWDAF containing AnLF), before step 3 the ML model consumer (e.g., NWDAF containing AnLF) obtains a token for the analytics ID from NRF same as step 4 which authorizes the ML model consumer to use model retrieval service provided by the NF service consumer and sends the model retrieval service request to the NF service consumer with the Analytics ID, the obtained token, the Vendor ID and the CCA of the ML model consumer.

4a. NF Service consumer e.g., NWDAF requests an access token from the NRF using the Nnrf_AccessToken_Get request operation. The token request message contains, besides the parameters described in clause 13.4.1.1.2, the Vendor ID of the NF Service consumer  and the Analytics ID. If the NF Service consumer is NWDAF containing MTLF, it also includes its Interoperability indicator per Analytics ID in the request.

In case of NF service consumer request ML models on behalf of another ML model consumer (e.g., NWDAF containing AnLF), the token request message shall also contain the NF instance ID and Vendor ID of the ML model consumer. The NF Service Consumer also includes the CCA of the ML model consumer that it has received in the service request from the ML model consumer.

4b. NRF checks whether the NF Service consumer is authorized to access the requested service in NWDAF containing MTLF. The NRF  verifies that the NF Consumer's Vendor ID is included in the NF Service Producer 's interoperability indicator for the Analytics ID. If NF Service consumer is a NWDAF containing MTLF, the NRF also verifies that NF Service consumer's Interoperability indicator is subset of the NF Service Producer’s interoperability indicator for the Analytics ID. If the authorization is successful, the NRF grants the token (token1) including the Analytics ID as specified in the clause 13.4.1.

In case of NF Service Consumer request ML models on behalf of another ML model consumer, the NRF also verifies that the Vendor ID of ML model consumer is included in the NF Service Producer's interoperability indicator for the Analytics ID and grants the token(token1) also including the NF Instance ID of ML model consumer in the sourceNfinstanceId IE attribute. The NRF also authenticates the ML model consumer based on one of the SBA methods described in clause 13.3.1.2.

5. 	NF Service Consumer performs Nnwdaf_MLModelProvision (Analytics ID, Vendor ID and token1) service operation at the NWDAF containing MTLF to retrieve ML models for the Analytics ID.

In case of NF Service Consumer request ML models on behalf of another ML model Consumer, the Nnwdaf_MLModelProvision service operation shall also contain the NF instance ID and Vendor ID of the ML model consumer and the CCA of the ML model consumer.

NOTE 1:	The NF Service Producer relies on the NRF to perform the authorization and to check the vendor ID of the NF Service Consumer. NF Service Producer cannot verify the vendor ID by itself.

6a. The NF Service Producer authenticates the NF Service Consumer and verifies the access token as specified in the clause 13.4.1.1.2 and ensures that the Analytics ID is included in the access token. If verification is successful, NWDAF containing MTLF determines the ML model to be shared for the requested Analytics ID and stored the NF instance ID of NF Service Consumer as part of allowed NF instance list for the ML model.

If the NF Service Consumer request ML models on behalf of the ML model Consumer, the NF Service Producer also authenticates the ML model consumer following clauses 13.3.2 and 13.4.1, and ensures that the ML model consumer is authorized according to the sourceNfinstanceId IE attribute of the access token provided by the NF Service Consumer. The NF Service Producer stores also the NF instance ID of Model Consumer as part of allowed NF instance list for the ML model.

NOTE 2: 	In the case a new ML model consumer comes at a later stage to request the model, which is already available at the NF Service Consumer, steps 4-7 apply. When the request is received by the NF Service Producer (i.e. the model producer), it authenticates the NF Service Consumer and verifies the access token provided along with the service request and sends to NF Service Consumer the access token verification response. NF Service Consumer based upon the response received, either sends the ML model to the ML model consumer or in the case of access token verification failure, the NF Service Consumer rejects the request received by the ML model consumer and does not send the model.

6b. If the determined ML model is stored in ADRF, and if the NF Service Consumer is not yet in the allowed NFc list stored at the ADRF, the NWDAF containing MTLF triggers the update of Nadrf_MLModelManagement_StorageRequest at the ADRF, with NF ID of NWDAF containing MTLF and Model ID, adding the NF Service Consumer to the allowed NFc list. The ADRF verifies that the requesting NWDAF containing MTLF is same as the one that stored the model. Then, ADRF stores the allowed NF instance list for the ML model referenced by the Model ID.

6c. ADRF sends the response to NWDAF containing MTLF which contains Model ID.

7. NWDAF containing MTLF sends Nnwdaf_MLModelProvision Notify to the NF Service Consumer with Model ID, the address of the determined ML model, which can be either the one stored in NWDAF containing MTLF or in ADRF,or ADRF(set) ID.  If the address of the determined ML model is provided, steps 8a to 10 are skipped.

If the ADRF(set) ID is provided, the following steps are applied:

8a. NF Service Consumer requests an access token from the NRF to be authorized to retrieve the model stored in ADRF as specified in clause 13.4.1.

8b. NRF verifies that the NF Service consumer e.g., NWDAF is authorized to access the service provided by the ADRF. If verification is successful, NRF grants the token (token2), based on the information provided in ADRF's NF profile.

9.  NF Service consumer e.g., NWDAF requests to retrieve the target model by sending   Nadrf_MLModelManagement_Retrieval Request as described in clause 10.3.4 TS 23.288 [105], including token2.

10. ADRF authenticates the NF Service Consumer and verifies the access token (token2) as specified in the clause 13.4.1.1.2. ADRF verifies also the NF Service Consumer’s NF ID is included in the allowed NF instance list for the ML model and/or is same as the NF ID of the MTLF that stored the model. If verification is successful, ADRF sends Nadrf_MLModelManagement_Retrieval Response to the NF Service Consumer, which contains the address of the stored model in ADRF.

11. NF Service Consumer retrieves the ML model from NWDAF containing MTLF or ADRF based on the ML model file address and decrypts the model per the vendor’s  implementation.

NOTE 3:	As per TS 23.288 [105] clause 10.3.2, how the NF Service Consumer downloads the ML Model is left for implementation.

# Annex X.X1: Authorization of Vertical Federated Learning participants involving NWDAF and AF

The authorization for Vertical Federated Learning (VFL) participants involving NWDAF and AF uses OAuth 2.0 based authorization as specified in clause 13.4.1, with the additions specified in the following sub-clauses.

## Annex X.X1.1: Authorization of VFL participants when the External AF acts as a VFL Server

Figure X.X1.1-1 depicts the authorization mechanism for External AF acting as VFL Server to initiate the Vertical Federated Learning process on the NWDAF containing MTLF(s) acting as VFL Client(s). The authorization is based upon the VFL capability type (VFL server, VFL client or both) and AF ID provided by the AF acting as VFL server during registration via NEF, and the Analytics ID and authorization information used for VFL provided by the NWDAF containing MTLF acting as VFL client during registration.

Figure X.X1.1-1: VFL authorization when the External AF acts as a VFL Server

Step 1a. The NWDAF containing MTLF acting as VFL client registers to the NRF with its supported VFL capability type (VFL client) per supported Analytics ID and Analytics ID(s). The registration request may contain the authorization information used for VFL (e.g., AF ID allowed for VFL).

Step 1b. The External AF acting as VFL server registers at the NEF with its supported VFL capability type (VFL server) and AF ID.

NOTE: 	External AF can register its data via OAM configuration at NEF.

Step 1c. The NEF registers to the NRF within its NF profile about the the AF information as specified in clause 6.2.2.3 of TS 23.288 and includes as part of the information the AF’s VFL related information, including supported VFL capability type (VFL server) and AF ID.

Step 2a, 2b. The External AF acting as VFL server sends a discovery request to NRF via NEF and receives the available NWDAFs containing MTLF acting as VFL client(s) as a response. The NEF enables this discovery procedure after authenticating and authorizing the AF.

NOTE: 	NEF anonymizes NWDAF instances ID(s) as specified in clause 6.2H.2.1.2 of TS 23.288 [105].

Step 3a. The External AF acting as VFL server sends the VFL service request (e.g., VFL preparation, sample alignment, feature alignment, VFL training, VFL inference) towards the NEF.

Step 3b. The NEF authorizes the External AF can trigger VFL service request towards NEF. The authentication and authorization between the NEF and the External AF acting as VFL server can be performed as specified in TS 33.501 [5] clause 12. If External AF is authorized, the NEF sends an access token request to the NRF to request the access token for the External AF acting as VFL server to perform VFL service request towards the VFL clients. The token request may contain the Analytics ID and AF ID for the requested VFL process.

Step 4. NRF checks whether the NF Service consumer (NEF) is authorized to access the requested service in VFL client (NWDAF). In case of the NF Service Consumer (NEF) request VFL service for the External AF acting as VFL server, the NRF also verifies that the AF ID is included in the authorization information used for VFL provided by the VFL client in Step 1a. If the authorization succeeds, NRF generates the access token(s) as specified in TS 33.501 [5] clause 13.4.1. The access token claims may include the Analytics ID and AF ID for the requested Vertical Federated Learning process.

NOTE: 	Fine-grained authorization can be done locally at the NWDAFs containing MTLF acting as VFL client(s) (NF Service Producer). Also, fine-grained authorization can be done locally at the NEF.

Step 5. The NRF sends the access token to the NEF, or rejects the request in case of failed authorization, as described in clause 13.4.1.

Step 6. The NEF sends the VFL service request (e.g., VFL preparation, sample alignment, feature alignment, VFL training, VFL inference) towards the VFL clients, with the obtained token.

Step 7, 8a, 8b. The NWDAF containing MTLF acting as VFL client verifies the received access token as specified in TS 33.501 [5] clause 13.4.1. The NWDAF containing MTLF acting as VFL client may also check the AF ID in the token for fine-grained authorization. In case of successful access token verification, the NWDAF containing MTLF acting as VFL client sends a success VFL service response to the External AF acting as VFL server, via the NEF.

Step 9. The Vertical Federated Learning procedure is performed between VFL server (External AF) and VFL client (NWDAF) via the NEF.

***END OF THE CHANGE***