3GPP TSG-SA3 Meeting #120	S3-250793

# Athens, Greece, 17 - 21 February 2025

Source:	Samsung

Title:	Update to solution #12

Document for:	Approval

Agenda item:	5.2

Spec:	3GPP TR 33.790

Version:	1.1.0

Work Item:	FS_NG_RTC_SEC_Ph2

Comments

This contribution proposes to change the name of the entities to be aligned with SA2.

* * * First Change * * * *

## 6.12	Solution #12: Solution for secure IMS based avatar communication using STIR/SHAKEN

### 6.12.1	Introduction

This solution addresses key issue #2: Security of IMS based Avatar Communication.

According to conclusion in TR 23.700-77 [2], rendering of avatar media can be performed by network, UE1, or UE2. This solution proposes security procedures for the three avatar calls to protect avatar ID and avatar representation.

This solution proposes to use STIR/SHAKEN framework for signing and verification of the avatar ID.

The avatar communication can be unidirectional or bidirectional. In this solution, only unidirectional avatar communication is described. When bidirectional avatar communication is used, UE2 also performs the operation same as UE1 described in the procedure.

### 6.12.2	Solution details

#### 6.12.2.1	Network centric IMS avatar call flow

Figure 6.12.2.1-1 Network centric IMS avatar call flow

1.	UE1 initiates an IMS session and establishes audio and video session connections with UE2. The bootstrap channel is established for both UE1 and UE2. UE1 may obtain its own avatar ID list from HSS/UDM.

2.	UE1 performs the XR media rendering negotiation. UE1 chooses an avatar ID to be used from the list and sends it to DCSF. DCSF checks whether the UE1 is allowed to use the avatar ID. DCSF may retrieve UE1's avatar ID list from HSS/UDM.

3.	DCSF requests Avatar representation by sending the UE1's avatar ID.

4.	BAR responds with the avatar representation.

5.	DCSF requests a signature to STI AS by sending the UE1's avatar ID.

6.	STI AS signs UE1's avatar ID and sends it to DCSF.

7.	DCSF sends the UE1’s avatar representation to MF for the rendering. UE1's avatar ID with the signature is sent to the terminating IMS network. The terminating IMS network invokes the STI VS to verify the signature.

8.	The UE1 sends data for the rendering (e.g., facial feature points).

9.	The MF performs the rendering using the UE1's avatar representation and the data received from UE1 in step 8.

10.	The rendered avatar media is sent as regular video media to UE2.

11.	The rendered avatar media is sent back to the UE1 as feedback.

#### 6.12.2.2	Sending UE centric IMS avatar call flow

Figure 6.10.2.2-1 UE1 centric IMS avatar call flow

1.	UE1 initiates an IMS session and establishes audio and video session connections with UE2. The bootstrap channel is established for both UE1 and UE2. UE1 may obtain its own avatar ID list from HSS/UDM.

2.	UE1 performs the XR media rendering negotiation. UE1 chooses an avatar ID to be used from the list and sends it to DCSF. DCSF checks whether the UE1 is allowed to use the avatar. DCSF may retrieve UE1's avatar ID list from HSS/UDM.

3.	DCSF requests Avatar representation by sending the UE1's avatar ID.

4.	BAR responds with the avatar representation.

5.	DCSF requests a signature to STI AS by sending the UE1's avatar ID.

6.	After STI AS signs UE1's avatar ID, STI AS sends the signature to DCSF.

7.	DCSF sends UE1's avatar representation to UE1 for the rendering. UE1's avatar ID with the signature is sent to the terminating IMS network. The terminating IMS network invokes the STI VS to verify the signature.

Editor's Note: The need to send the avatar ID and the signature verification is FFS

8.	The UE1 performs the rendering.

9.	The rendered avatar media is sent as regular video media to UE2.

#### 6.12.2.3	Receiving UE centric IMS avatar call flow

Figure 6.12.2.3-1 UE2 centric IMS avatar call flow

1.	UE1 initiates an IMS session and establishes audio and video session connections with UE2. The bootstrap channel is established for both UE1 and UE2. UE1 may obtain its own avatar ID list from HSS/UDM.

2.	UE1 performs the XR media rendering negotiation. UE1 chooses an avatar ID to be used from the list and sends it to DCSF. DCSF checks whether the UE1 is allowed to use the avatar ID. DCSF may retrieve UE1's avatar ID list from HSS/UDM.

3.	DCSF requests Avatar representation by sending the UE1's avatar ID.

4.	BAR responds with the avatar representation.

5.	DCSF requests a signature to STI AS by sending the UE1's avatar ID and avatar representation.

6.	STI AS signs them and sends the signature to DCSF.

7.	DCSF sends UE1’s avatar representation to UE2 for the rendering. UE1's avatar ID and avatar representation with the signature are sent to the terminating IMS network. The terminating IMS network invokes the STI VS to verify the signature.

8.	The UE1 sends data for the rendering (e.g., facial feature points).

9.	The UE2 performs the rendering using the UE1's avatar representation and the data received from UE1 in step 8.

Editor's Note: Whether DCSF can retrieve the avatar representation from BAR is FFS.

Editor's Note: Which entity (CSCF or DSCF) should transfer the avatar representation is FFS.

Editor's Note: The alignment with SA2 conclusions from TR 23.700-77 [2] is FFS.

Editor's Note: Whether to use SIP to transfer avatar ID and avatar representation is FFS.

### 6.12.3	Evaluation

This solution addresses the requirements of KI#2: Security of IMS based Avatar Communication.

In this solution, STIR/SHAKEN framework is used to sign and verify the avatar ID. In UE2 centric avatar call flow where avatar representation itself should be sent to UE2 via terminating IMS network, signed avatar representation is also transferred to the IMS network of UE2.

Editor's Note: Impact on the network functions is FFS.

Editor's Note: Further evaluation is FFS.

Editor's Note: Whether to use non-DC to transfer avatar ID and avatar representation is FFS.

* * * End of Changes * * * *