3GPP TSG-SA3 Meeting #120	S3-250836

# Athens, Greece, 17 - 21 February 2025

Source:		Xiaomi Technology Spain S.L

Title:	Pseudo-CR on updates to conclusion for KI#5 in TR 33.713

Document for:	Approval

Agenda item:	5.9

Spec:	3GPP TR 33.713

Version:	0.6.0

Work Item:	FS_Ambient_IoT_Sec

Comments

This pCR proposes to update the conclusion of KI#5 for FS_AIOT_SEC in TR 33.713. Based on the conclusion in TR 23.700-13, each AIoT device has an associated device profile, which can be stored in the third party or 5GC. With this device profile data, the device authentication credential can be securely managed. Therefore, it is proposed to revise NOTE 3 and add further clarification regarding the storage of device authentication credential.

* * * First Change * * * *

7.5	Conclusion on KI #5

1. 	The credential (including device ID and authentication credential) for AIoT device authentication can be owned by an operator or by a third party. The principles for device ID management are concluded in 8.2.1 in TR 23.700-13 [4]. The credential is stored in the AIoT device profile data as concluded in 8.2.2 in TR 23.700-13 [4].

2. 	For operator managed AIoT devices, the following principles are taken as the conclusion for KI#5:

Inventory-only Procedure:

- Authentication (i.e., at least one-way authentication; the 5G network authenticates the AIoT device) shall be supported.

NOTE 1: Details of the authentication procedure (e.g., one-way, parameter(s) used, etc.) are to be resolved during normative phase.

Inventory and Command Procedure:

- Mutual authentication between the AIoT device and the 5G network shall be supported.

- The authentication procedure is based on a challenge-response mechanism.

NOTE 2: Details of the mutual authentication procedure and details of the challenge-response mechanism (e.g., parameter(s) used for the 5G network to authenticate the AIoT device and for the AIoT device to authenticate the network) are to be resolved during normative phase.

3. 	Additional considerations during normative phase:

NOTE 3:	Where to store the AIoT device policy information will be based on SA2 and SA3 coordination.

* * * End of Changes * * * *