3GPP TSG-SA3 Meeting #120	S3-250880

Athens, Greece, 17 - 21 February 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | DraftCR | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR for TS 33.501 on the requirement of N6 deplay management protection. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, China Unicom, NOKIA?, Ericsson? |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE3_Ph3 |  |  |  |  |  | Date: |  |  | 2025-01-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This proposal is to add the security aspects related to enhancements of EAS and local UPF (re)selection concluded in the clause 7.1 of TR 33.749, i.e., The following guidelines from the security point of view for the N6 delay measurement between UPF PSA and EAS will be documented: - It is recommended to use of the measurement protocols supporting authentication, integrity, and anti-replay protection. - It is recommended not to use a delay measurement mechanism that requires sensitive information exchange but does not support confidentiality by the measurement protocols. - It is not recommended to use a measurement protocol without authentication, integrity and anti-replay protection in place. - It is not recommended to use a measurement protocol with disclosed vulnerabilities which may be exploited for attack or blocked by the operators. NOTE: The enablement of the security mechanisms available in the selected measurement protocol, i.e., the configuration and provision of corresponding security parameters, makes use of existing procedures to enable N6 delay measurement defined in TS 23.548 [7] and TS 23.502 [8]. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to add the security aspects related to enhancements of EAS and local UPF (re)selection in the Annex T. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | N6 delay management between 5GC N6 termination point (i.e. UPF) and the measurement endpoint at application side (i.e. EAS IP address(es)/Designated IP (range)) may not be securely protected. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex T.X. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Start of The First Change ***

T.X Security aspects related to enhancements of EAS and local UPF (re)selection

The following security guidelines of the existing mechanisms (e.g. ICMP, TWAMP, OWAMP or other protocols defined by IETF) for the N6 delay measurement between 5GC N6 termination point (i.e. UPF) and the measurement endpoint at application side (i.e. EAS IP address(es)/Designated IP (range)) as defined in TS 23.548 [98] are proposed:

- It is recommended to use of the measurement protocols supporting authentication, integrity, and anti-replay protection.

- It is recommended not to use a delay measurement mechanism that requires sensitive information exchange but does not support confidentiality by the measurement protocols.

NOTE 1: The details on the sensitive information are left for operator’s local policy.

- It is not recommended to use a measurement protocol without authentication, integrity and anti-replay protection in place.

- It is not recommended to use a measurement protocol with disclosed vulnerabilities which may be exploited for attack or blocked by the operators.

NOTE 2: The details on the vulnerabilities are left for operator’s local policy.

NOTE 3: The enablement of the security mechanisms available in the selected measurement protocol, i.e., the configuration and provision of corresponding security parameters, makes use of existing procedures to enable N6 delay measurement defined in TS 23.548 [98] and TS 23.502 [8].

*** End of The First Change ***