3GPP TSG-SA3 Meeting #120	S3-250888

# Athens, Greece, 17 - 21 February 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	Update conclusion for KI#2 Privacy of user sensitive information

Document for:	Approval

Agenda item:	5.18

Spec:	3GPP TR 33.721

Version:	0.6.0

Work Item:	FS_Metaverse_Sec

Comments

<Proposals, reason for change, abstract, comments if necessary (optional)>

The requirement of KI#2 in the present TR requires the 5G system to provide a means for privacy protection of user sensitive information during exposure of user specific information (e.g. user identity, user location) in localized mobile metaverse services through the application enabler layer.  The conclusion of KI#2 in the present TR proposes to reuse RNAA to address the requirement, e.g. The user information is only exposed if CCF obtains permission from the resource owner as specified in TS 33.122 [5].

However, in TS 33.122, the CCF only receives the API name in the Access Token Request and does not have knowledge of the sensitive information of the resource owner getting exposed with the API and so cannot get any resource owner authorization w.r.t sensitive information. This lack of transparency creates a potential privacy risk for the resource owner, that may cause regulation incompliance issue on the operator.

In TR 33.700-22 v0.5.0, it concluded in KI#1.3 that Normative work is recommended to support service operation level and resource level granularity in RNAA scenarios, however there’s still an Editor's Note that Whether ROF supports service operation level and resource level granularity is FFS. Without corresponding normative work agreed in CAPIF phase security WI, the KI#2 of metaverse security cannot be solved with RNAA based solution. A EN will be added for this concern.

In addition, SEAL security based mechanism defined in TS 33.434 was concluded in KI#1, 3, 4 for authorization of metaverse service consumer to access metaverse services. SIM-S instead of CCF issues the access token in SEAL security framework and SIM-S is kind of AEF of CAPIF. It’s unclear what’s the role of CCF in the SEAL security based solution, and whether RNAA or OIDC user authentication and authorization should be used to get authorization from the resource owner. Therefore, a EN will be added on how to get authorization from resource owner if SEAL security defined in TS 33.434 is applied for metaverse service request authorization.

Proposed Changes

* * * First Change * * * *

<Proposed change in revision marks>

# 7	Conclusions

## 7.2	Conclusion on Key Issue #2

The user information is only exposed if CCF obtains permission from the resource owner as specified in TS 33.122 [5].

NOTE:	whether CAPIF RNAA supports the case where the resource owner is different from the user of the application invoker is to be decided in normative phase, depending on the progress on CAPIF_Ph3-Sec.

NOTE: whether CAPIF RNAA supports service operation level and resource level granularity in RNAA is to be decided in normative phase, depending on the progress on CAPIF_Ph3-Sec.

Editor’s Note: Further conclusion on the reuse of user consent framework defined in TS 33.501 [6] Annex V for user privacy is FFS.

Editor’s Note: If SEAL security framework defined in TS 33.434 is applied to authorize metaverse service request, how to get authorization/consent from resource owner and where to request access token is FFS.

Editor’s Note: Further conclusions are FFS.

* * * End of Changes * * * *