3GPP TSG-SA3 Meeting #120	S3-250943

Athens, GR, 17 -21 February 2025

Source:	Google, Cisco Systems, AT&T, TELUS

Title:	pCR to the living document on: secure message transport using ACME to ACME-SBA

Document for:	Approval

Agenda item:	4.18

Spec:	3GPP TS 33.310

Version:	19.3.0

Work Item:	ACME_SBA

Comments

This document proposes to add the procedures for secure message transport in 5G SBA using ACME.

Proposed Changes

* * * First Change * * * *

YY.3	Secure message transport

It is assumed that the 5G NF is issued with the operator CA’s root certificate, which is used to validate the ACME server’s TLS certificate.

The communication between 5G NF and the CA server shall be done over HTTPS for authentication and confidentiality. Refer to YY.2 for the ACME account created by the NF.

When a 5G NF fetches a resource from an ACME server, the server should authenticate the requester and verify any access control as described in RFC 8555 [XX]. If an ACME client’s credentials have been compromised or there are reasons to believe they have been compromised, the CA should reject any requests from that ACME client. Refer to YY.6 for the ACME certificate revocation.

ACME for 5G SBA should use JWS based integrity protection as described in RFC 8555 [XX].

ACME for 5G SBA should use nonces to protect messages against replay-attacks.

The CA shall maintain a list of nonces that it has issued and require any signed request from the 5G NF to carry such a nonce as described in RFC 8555 [XX].

* * * End of Changes * * * *