3GPP TSG-SA3 Meeting #120	S3-250949

# Athens, Greece, 16-21 February 2025

Source:	Nokia

Title:	Determinding which NRFs or NRF Sets are allowed to act as authorization servers

Document for:	Approval

Agenda item:	4.11

Spec:	3GPP TS 33.501

Version:	19.1.0

Work Item:	TEI19, SBA

Comments

pCR against DRAFT CR (was S3-245334) for resolution of

Editor’s Note:  How NF determines which NRFs or NRF Sets are allowed to act as authorization servers is ffs. Details of information to be configuered in the NF Service Producer are ffs.

This pCR describes how NRF determines target NRF for forwarding tokens for NF type access in deployments with several NRFs.

Note: If an operator wants to limit functionality of an NRF of the NF Set, it needs to specify which NRF is allowed to act as authentication server.

This is addressing the following EN in change 2 of the DRAFT CR.

Editor’s Note:  How NF determines which NRFs or NRF Sets are allowed to act as authorization servers is ffs. Details of information to be configuered in the NF Service Producer are ffs.

Proposed Changes

*** BEGIN OF CHANGES 2 ***

##### 13.4.1.1.1	OAuth 2.0 roles

OAuth 2.0 roles, as defined in clause 1.1 of RFC 6749 [43], are as follows:

a.	The Network Repository Function (NRF) shall be the OAuth 2.0 Authorization server.

b.	The NF Service Consumer shall be the OAuth 2.0 client.

c.	The NF Service Producer shall be the OAuth 2.0 resource server.

OAuth 2.0 client (NF Service Consumer) registration with the OAuth 2.0 authorization server (NRF)

The NF Service registration procedure, as defined in clause 4.17.1 of TS 23.502 [8], may be used to register the OAuth 2.0 client (NF Service Consumer) with the OAuth 2.0 Authorization server (NRF), as described in clause 2.0 of RFC 6749 [43]. The client id, used during OAuth 2.0 registration, shall be the NF Instance Id of the NF. OAuth2.0 clients may also register with the NRF using OAM.

A Network Function that does not implement this option shall be able to get an access token from the NRF as long as the NRF is able to authenticate and authorize the Network Function during the NF access token get service request.

OAuth 2.0 resource server (NF Service Producer) registration with the OAuth 2.0 authorization server (NRF)

The NF Service registration procedure, as defined in clause 4.17.1 of TS 23.502 [8], shall be used to register the OAuth 2.0 resource server (NF Service Producer) with the OAuth 2.0 Authorization server (NRF). The NF Service Producer, as part of its NF profile, may include "additional scope" information related to the allowed service operations and resources per NF Service Consumer type.

Figure 13.4.1.1-1b NF Service Producer registers in NRF

1)	The NF Service Producer registers as OAuth 2.0 resource server in the NRF. The NF profile configuration data of the NF Service Producer may include the "additional scope". The "additional scope" information indicates the resources and the actions (service operations) that are allowed on these resources for the NF Service Consumer. These resources may be per NF type of the NF Service Consumer or per NF instance ID of the NF Service Consumer.

Editor’s Note:  How NF determines which NRFs or NRF Sets are allowed to act as authorization servers is ffs. Details of information to be configuered in the NF Service Producer are ffs.

2-3)	After storing the NF Profile, NRF responds successfully.

NOTE: 	Depending on network deployment and operator policy, the OAuth 2.0 authorization server (NRF) that issues an access token can be a different NRF than the NRF that the NF Service Producer is registered at, especially when issuing access token for accessing services of NF Service Producers of a specific NF type.

If the NRF that the Network Function registered at, is part of an NF Set, then the NF profile is available at all NRFs of the same set. However, each NRF of a set will have to be accessed with its specific public key. Hence, the NRF where the NF has registered before may be the preferred to request the token from, since the NF Service Producer would already know the OAuth Server's public key from its previous registration. Otherwise, the NF Service Producer needs to acquire the relevant public key information of the access token issuing NRF.

Authorized OAuth 2.0 authorization servers (NRFs) together with their X.509 certificates or information from where to get these certificates from may be configured in the NF Service Producer for access token validation.

Editor’s note: How NRF determines target NRF for forwarding tokens for NF type access in deployments with several NRFs is FFS.

*** END OF CHANGES 2 ***