3GPP TSG-SA3 Meeting #120	S3-251036-r7

# Athens, Greece, 17 - 21 February 2025			Merger of S3-250762, 428, 552, 662, 742, and 833

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Draft CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.401 | CR | draft | rev |  | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Security for Store and Forward Satellite operation – Split MME architecture |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Philips International B.V., InterDigital, Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_SEC |  |  |  |  |  | Date: |  |  | 2025-02-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Security for 5G satellite access phase 3 is not specified yet. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add content to the new normative annex in TS 33.401 to describe the security aspects of Store and Forward Satellite operation in a Split MME architecture. The additions cover the following: - Involvement of one or more MMEs-onboard and provisioning of the required information to perform/finish an authentication procedure. - NAS security termination point and NAS security context transfer responsibility. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | X.2 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Annex X (normative):

# X.1	General

Editor’s Note: This clause provides general description for Annex X.

*** Start of 1st Change ***

# X.2	Security aspects on Split MME architecture

Mutual authentication between the UE and the network in a split MME architecture may involve more than one satellite (i.e., more than one MME-onboard), in which case the ground segment of the network is responsible for the selection and provisioning of MME-onboard the same, or another satellite, with the necessary information (e.g., Authentication Vector) to perform or finish an authentication procedure. The MME on-board obtains the EPS authentication vectors when the feeder link is available, and stores the authentication vectors when the service link is unavailable.

The NAS security is terminated on the MME-onboard. The ground segment of the network ensures that the latest NAS security context of the UE is available at the MME-onboard.

NOTE: 	The distribution and synchronization of the latest NAS security context between the MME-ground and MME-onboard is out of the scope of 3GPP.

Editor’s Note: Whether the Annex X.2 is informative or normative is FFS.

*** End of Change ***