3GPP TSG-SA3 Meeting #120	S3-251063

Athens, Greece, 17 - 21 February 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.535 | CR | 0226 | rev | 1 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Notification about AKMA service disabling via NEF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | AKMA_Ph2 |  |  |  |  |  | Date: |  |  | 2025-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This proposal is to add notification about AKMA service disabling via NEF Clause 6.8 specifed the procedure in which AAnF directly notifies AF about AKMA service disabling, without notification about AKMA service disabling via NEF. In addition, services provided by NEF as described in Table 7.3.1-1 are not complete. Revision of S3-250716. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to add notification about AKMA service disabling via NEF and services provided by NEF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedure for notification about AKMA service disabling via NEF is missing. The services provided by NEF are not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.8, 6.8.X(new), 6.8.Y(new), 7.3.1, 7.3.2, 7.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Start of The First Change ***

6.8	Notification about AKMA service disabling

6.8.X	Notification to internal AF about AKMA service disabling

This procedure is used when the AKMA sessions have already been started (before roaming was detected), and as soon as PLMN change is detected at the AAnF, the AAnF may execute this procedure based on the roaming policy.

Figure 6.8.X-1: AAnF notification to internal AF about AKMA service disable

1.	 UE registers with a (H)PLMN.

2.	 UE is accessing the AF and key material is provided to AF as described in 6.2.1. While accessing the AAnF, AF may also provide the Notification URI.

3. UE is getting registered in a VPLMN and AAnF detects the PLMN change via the Nudm_EventExposure_Notification received from UDM.

4. AAnF determines if AF(s) have subscribed to receive notifications for AKMA service disabling and roaming policy is configured and restrict the AKMA access in the VPLMN; if yes, steps 5 and 6 are executed. Otherwise, steps 5 and 6 are skipped.

5. If AF(s) are determined at step 5, the AAnF shall send notifications to the subscribed AF(s) about AKMA roaming via Naanf_AKMA_ServiceDisableNotification. The A-KID is the transmitted A-KID for the corresponding AF, which is kept track of in step 8 in clause 6.2.1.

NOTE 1: 	The re-authentication of the UE might imply that the current A-KID at the AAnF is not identical to the original A-KID by which the AF previously requested the AKMA key. Therefore, the AAnF retrieves and uses the original A-KID in this message to the AF.

6. The AF shall send the response and based on the notification and internal policy, the AF may stop the UE service, may stop the encryption.

NOTE 2: By stopping the encryption (e.g., TLS 1.2 NULL cypher negotiation), LI interception could work in the VPLMN.

*** End of The First Change ***

*** Start of The Second Change ***

6.8.Y	Notification to external AF about AKMA service disabling

Figure 6.8.Y-1: AAnF notification to external AF about AKMA service disable via NEF

This procedure is used when AAnF notifies AF about AKMA service disabling via NEF.

1.	UE registers with a (H)PLMN.

2.	UE is accessing the AF and key material is provided to AF via NEF as described in 6.3. While accessing the AAnF, AF may also provide the Notification URI.

3.  For details about step 3, see step 3 in Figure 6.8.X-1.

4.  AAnF determines if AF(s) have subscribed to receive notifications for AKMA service disabling and roaming policy is configured and restrict the AKMA access in the VPLMN; if yes, steps 5-8 are executed. Otherwise, steps 5-8 are skipped.

5.  If AF(s) are determined at step 5, the AAnF sends notifications to the subscribed AF(s) via NEF about AKMA roaming. The AAnF sends Naanf_AKMA_ServiceDisableNotification to NEF. The notification includes the A-KID and the AF_ID. The A-KID is the transmitted A-KID for the corresponding AF, which is kept track of in step 4 in clause 6.3.

NOTE 1: 	The re-authentication of the UE might imply that the current A-KID at the AAnF is not identical to the original A-KID by which the AF previously requested the AKMA key. Therefore, the AAnF retrieves and uses the original A-KID in this message to the AF.

6. Once receiving the notification from AAnF, NEF shall send Nnef_AKMA_ServiceDisableNotification to AF.

7. The AF sends Nnef_AKMA_ServiceDisableNotification response to NEF and based on the notification and internal policy, the AF may stop the UE service, may stop the encryption.

8. Once receiving the response from AF, the NEF sends Naanf_AKMA_ServiceDisableNotification response to AAnF.

NOTE 2: 	By stopping the encryption (e.g., TLS 1.2 NULL cypher negotiation), LI interception could work in the VPLMN.

*** End of The Second Change ***

*** Start of The Third Change ***

7.3.1	General

The NEF exposes AKMA Application Key derivation service to the requester NF.

The following table shows the NEF Services and NEF Service Operations related to AKMA service.

Table 7.3.1-1: List of NEF Services

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_AKMA | ApplicationKey_Get | Request/Response | AF |
|  | ServiceDisableNotification | Request/Response | AF |

*** End of The Third Change ***

*** Start of The Fourth Change ***

7.3.2	Nnef_AKMA_ApplicationKey_Get service operation

Service operation name: Nnef_AKMA_ApplicationKey_Get.

Description: The NF consumer requests the NEF to provide AF related key material.

Input, Required: A-KID, AF_ID

Input, Optional: UEID not needed indication, Service Disable URI.

Output, Required: KAF, KAF expiration time.

Output, Optional: GPSI (external ID).

*** End of The Fourth Change ***

*** Start of The Fifth Change ***

7.3.3	Nnef_AKMA_ServiceDisableNotification service operation

Service operation name: Nnef_AKMA_ServiceDisableNotification

Description: NEF notifies the NF consumer about AKMA service is disabled.

NOTE: The AF is implicitly subscribed to receive Nnef_AKMA_ServiceDisableNotification service operation.

Input, Required: A-KID

Input, Optional: None

Output, Required: None

Output, Optional: None

*** End of The Fifth Change ***