3GPP TSG-SA3 Meeting #120	S3-251084

# Athens, Greece, 17 - 21 February 2025

Source:	Huawei, HiSilicon, NOKIA, NOKIA Shanghai Bell, Ericsson

Title:	Adding principle on ID verification to draftCR of MEC ph3

Document for:	Approval

Agenda item:	4.17

Spec:	3GPP TS 33.558

Version:	18.2.0

Work Item:	EDGE_Ph3

Comments

This contribution proposes to add the priciple on ID verification concluded in the clause 7.2 of TR 33.749

* * * First Change * * * *

5.1.X	Secure retrieval of 5G system UE lds and privacy related information

-	The non-permanent information used in Nnef_UEId API call invocation and Eees_UEIdentifier API call invocation should prevent the use of permanent subject to be abused/compromised by malicious applications.

-	The entity responsible for the generation of the non-permanent information should be part of the 5G system in HPLMN and should be verified by the 5G system in HPLMN. Alternatively, the non-permanent information can be generated by both the UE and the 5G system.

-	The non-permanent information can be used by the AF invoking the NEF APIs, so that the NEF APIs can identify the UE by using the non-permanent information.

- 	The non-permanent information can be used by the EAS/EEC invoking the Eees_UEIdentifier API, so that the EES APIs can identify the UE by using the non-permanent information.

NOTE 1:	The API invoker does not need to obtain the permanent 5G UE ID, which improves the privacy.

NOTE 2:	The principles above do not prevent usage of existing mechanisms of Nnef_UEId  API and UE Identifier API.

Example implementation specific mechanisms having the design principles above are captured in an informative Annex Z of this document.