3GPP TSG-SA3 Meeting #120	S3-251090

Athens, GR, 17 -21 February 2025

Source:	Google, Cisco Systems, AT&T, TELUS, NCSC, DSIT, OTD_US, MITRE-FFRDC, US National Security Agency, Johns Hopkins University APL

Title:	pCR to the living document on: secure message transport using ACME to ACME-SBA

Document for:	Approval

Agenda item:	4.18

Spec:	3GPP TS 33.310

Version:	19.3.0

Work Item:	ACME_SBA

Comments

This document proposes to add the procedures for secure NF instantiation and account creation in 5G SBA using ACME.

Proposed Changes

* * * First Change * * * *

## YY.2	Initial configuration

### YY.2.1	NF instantiation

It is assumed that the 5G NF is issued with the operator CA’s root certificate, which is used to validate the ACME server’s TLS certificate.

The communication between 5G NF and the CA server is over HTTPS for authentication and confidentiality.

When a 5G NF fetches a resource from an ACME server, the server authenticates the requester and verifies any access control as described in RFC 8555 [XX].

ACME for 5G SBA uses JWS based integrity protection as described in RFC 8555 [XX].

ACME for 5G SBA uses nonces to protect messages against replay-attacks as described in RFC 8555 [XX].

### YY.2.2	Account creation

In 5G SBA it is recommended that the ACME account creation requires authorisation and authentication. This prevents unauthorised entities from creating accounts and attempting to request certificates. Failure to do this could lead to resource exhaustion and to increasing the attack surface for obtaining mis-issued certificates.

Note: 	The ACME server can limit the number of accounts associated to the ACME client.

Editor's Note: details of mechanisms to provide authentication and authorisation at or before account creation are FFS

* * * End of Changes * * * *