3GPP TSG-SA3 Meeting #120	S3-251100

Athens, Greece, 17 - 21 February 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.117 | CR | 0200 | rev | 1 | Current version: | 19.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding Note to TC_RESTRICTED_REACHABILITY_OF_SERVICES |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BSI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_Maint |  |  |  |  |  | Date: |  |  | 2025-02-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | External port scanning may be ambiguous when dealing with UDP or rate limited systems. Switching to local port discovery helps, but there’s the need to match the IP/Port information with the local routing and firewall information. A Note could help the tester how and when to approach the local approach instead of the external scan. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a note. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The tester may not know when to switch to local port discovery and what information to gather when doing so. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** START OF 1st CHANGE **********

#### 4.3.2.2	Restricted reachability of services

Requirement Name: Restricted reachability of services

Requirement Reference: In accordance with industry best practice

Requirement Description:

The network product shall restrict the reachability of services so that they can only be reached on interfaces where their usage is required. On interfaces were services are active, the reachability should be limited to legitimate communication peers. This limitation shall be realized on the network product itself (without measures (e.g. firewall) at network side) according to the requirement detailed in clause 4.2.6.2.1 Packet Filtering.

Example:	Administrative services (e.g. SSH, HTTPS, RDP) shall be restricted to interfaces in the management network to support separation of management traffic from user traffic.

Threat References: TR 33.926 [4], clause 5.3.7.3, Insecure Network Services

Test Case:

Test Name: TC_RESTRICTED_REACHABILITY_OF_SERVICES

Purpose:

To verify that it is possible to bind the services only to the interfaces from which they are expected to be reachable.

Note: 	The test case developed for the requirement " 4.2.6.2.1 Packet Filtering" implicitly verifies that the network product permits to limit the reachability of the services only to legitimate communication peers,

Procedure and execution steps:

Pre-Conditions:

-	The vendor shall declare, in the documentation accompanying the network product if the network product supports the capability to restrict services reachability to only the nodes authorized to access them. In this case, the vendor shall detail how this capability can be configured.

-	A list of all required network protocols and services containing at least the following information shall be included in the documentation accompanying the Network Product:

-	protocol handlers and services needed for the operation of network product;

-	their open ports and associated services;

-	the configuration options;

-	and a description of their purposes.

-	The network product is configured such that the required network protocols and services (as described in the network product documentation) are setup and each service is bound to an IP address of a specific network interface (e.g. IP1 which is the ip address of if1). Configuration may occur automatically during the initialization phase of the network product or manually as defined in the network product administration documentation.

-	The network product shall have at least two interfaces enabled, if1 and if2 respectively configured with IP Address IP1 and IP2.

-	The tester has administrative privileges.

-	A tester machine equipped with a network port scanner tool is available.

Execution Steps

For every available interface if_n:

1.	The tester runs a network port scanner (e.g. nmap) or uses local network interface information on if_n and verifies that the configured services (according to the vendor documentation) are open/reachable.

2.	The tester runs a network port scanner (e.g. nmap) or uses local network interface information on all other available interfaces (except if_n) and verifies that the services configured for if_n are not open/reachable.

NOTE: 	It might not be possible for certain transport layer protocols (like UDP) to unambiguously detect whether a port is open or not by means of external port scanning. Also, external port scanning can be ineffective, if there are security measures present, e.g. like rate limiting. Local port discovery (e.g. with netstat, ss) in collaboration with collection of local route information (e.g. with ip route) could be applied in those cases.

Expected Results:

Services can be enabled on per-interface basis.

Expected format of evidence:

-	The network product configuration showing the mapping between interfaces and configured service.

-	Pcap files.

-	Screenshot.

-	Software name and version of the used port scanner, log of the executed commands.

-	Network port scanner results (e.g. files containing this results).

********** END OF 1st CHANGE **********