3GPP TSG-SA3 Meeting #120	S3-251133

Athens, Greece, 17 - 21 February 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.511 | CR | 0072 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of TC UP integrity check failure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, BSI (DE) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_Maint |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Misalignment of Execution Steps and Expected result. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Merger of S3-250529 (Nokia CR) and S3-250722 (BSI CR) The PDCP PDU does not provide any length indication for the receiving side. Let’s assume the UE is sending a PDCP PDU to the gNB and let’s assume the PDU has no MAC-I. Then for the gNB it is impossible to detect the missing MAC-I, rather it will take the last 4 byte of the PDU and will consider them to be a MAC-I. In the verification step, the gNB will discard the PDU, because of a failed integrity check and not because of a missing MAC-I. Plus editorial improvements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecessary test execution step and consequently wrong test result. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.1.4, 4.2.2.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 4.2.2.1.4	RRC integrity check failure

Requirement Name: RRC integrity check failure

Requirement Reference: TS 33.501 [2], clause 6.5.1

Requirement Description: The RRC integrity checks are performed both in the ME and the gNB. In case failed integrity check (i.e. faulty or missing MAC-I) is detected after the start of integrity protection, the concerned message is discarded. This can happen on the gNB side or on the ME side, as specified in TS 33.501 [2], clause 6.5.1.

Threat References: TR 33.926 [5], clause D.2.2.2, Control plane data integrity protection

Test Case:

Test Name: TC-CP-DATA-RRC-INT-CHECK_gNB

Purpose:

Verify that RRC integrity check failure is handled correctly by the gNB.

Pre-Conditions:

Test environment with a UE. The UE may be simulated. RRC integrity protection is activated at the gNB.

Execution Steps

The UE sends a RRC message to the gNB without MAC-I

or

The UE sends a RRC message to the gNB with a wrong MAC-I.

NOTE:	RRC integrity protection is provided by PDCP. In a PDCP message without MAC-I, the last 4 Bytes of the PCDP Data will be interpreted as a wrong MAC-I and therefore the integrity check will fail.

Expected Results:

The RRC message is discarded by the gNB.

Expected format of evidence:

Sample copies of the log files.

##### 4.2.2.1.5	UP integrity check failure

Requirement Name: UP integrity check failure

Requirement Reference: TS 33.501 [2], clause 6.6.4

Requirement Description: If the gNB or the UE receives a PDCP PDU which fails integrity check with faulty or missing MAC-I after the start of integrity protection, the PDU is discarded as specified in TS 33.501 [2], clause 6.6.4.2.

Threat References: TR 33.926 [5], clause D.2.2.4, User plane data integrity protection

Test Case:

Purpose:

Verify that UP integrity check failure is handled correctly by the gNB.

Pre-Conditions:

Test environment with a UE. The UE may be simulated. UP integrity protection is activated at the gNB.

Execution Steps

The UE sends a PDCP PDU to the gNB without MAC-I

or

The UE sends a PDCP PDU to the gNB with a wrong MAC-I.

NOTE:	In a PDCP PDU message without MAC-I, the last 4 Bytes of the PCDP PDU Data will be interpreted as a wrong MAC-I and therefore the integrity check will fail.

Expected Results:

The PDCP PDU is discarded by the gNB.

Expected format of evidence:

Evidence suitable for the interface e.g. Screenshot containing the operational results.