3GPP TSG-SA3 Meeting #120	S3-251143

Athens, Greece 17 – 21 Feburary 2025

Title:	Reply LS on security questions related to NAS layer overhead reduction for data transfer using control plane CIoT

Response to:	C1-247156 LS on security questions related to NAS layer overhead reduction for data transfer using control plane CIoT

Release:	Rel-19

Work Item:	NORDAT-CP

Source:	SA3

To:	CT1

Cc:

Contact person:	Li Hu

huli@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1	Overall description

SA3 thanks CT1 for the LS C1-247156 on security questions related to NAS layer overhead reduction for data transfer using control plane CIoT.

After discussion, SA3 has the answers for the following questions raised by CT1:

Question 1: Currently, SERVICE REQUEST message in EPS NAS uses short MAC and short sequence number. Would it be acceptable to use short MAC and/or short sequence number for the new NAS message format with minimized overhead for data transfer via NAS?

Answer 1: Short sequence number is acceptable from a security perspective and it is left to CT1 to decide between the risk of messages being discarded due to the extra possibility of sequence number mismatch due to the short sequence number and the gain of reducing overhead. However, employing a short MAC compromises the integrity strength and therefore shall not be used for user data transfer.

Question 2: Currently, all initial NAS messages contain Key Sequence Indicator (KSI). Would it be acceptable not to include the KSI in the new initial NAS message format with minimized overhead for data transfer via NAS?

Answer 2: If UE has a NAS security context in use, for messages like CONTROL PLANE SERVICE REQUEST, KSI can be omitted, otherwise, it cannot be omitted. SA3 leaves decision on whether to omit this KSI to CT1.

Question 3: In the NAS message format with minimized overhead for data transfer via NAS, only a portion of the message will be ciphered. Is it acceptable for the ciphered portion not to be byte-aligned? Not having to byte-align the ciphered portion would allow for more efficient message encoding.

Answer 3: From design principle of security algorithm, the EEA does not require messages to be byte-aligned since the LENGTH can be set to bit length as illustrated in section Annex B.1.1 of TS 33.401. However, there is concern on hardware impact on existing implementation.

2	Actions

To CT WG1:

ACTION: 	SA3 kindly asks 3GPP CT1 to take the above information into consideration

3	Dates of next TSG SA WG 3 meetings

SA3#121				        7-11 April 2025				   Goteborg, Sweden

SA3#122				        19-23 May 2025				       Fukuoka, Japan