3GPP TSG-SA3 Meeting #122	S3-251887

# Fukuoka, Japan, 19 – 23 May 2025

Source:	Lenovo

Title:	Updates to security procedure for CAPIF interconnection

Document for:	Approval

Agenda item:	4.22

Comments

This pCR proposes to update the security procedure for CAPIF interconnection in living doc S3-251720.

In TS 33.122 Clause 6.5.2.3 Method 3 – TLS with OAuth token, the API invoker sends the access token request to the CCF and the related preconditions are shown in the snippet below.

As per OAuth 2.0 [4], the CAPIF core function shall perform the functionality of the authorization server and provide the token endpoint, the API invoker shall perform the function of the client functionality, while the API exposing function shall perform the resource server functions. The API invoker client (client endpoint) shall be registered as a confidential client type with an authorization grant type of 'client credentials'. The authorization shall be previously arranged in the CAPIF core function. The access token shall follow the profile described in annex C.

NOTE: How the authorization is pre-arranged (pre-configured) with the CAPIF core function is out of scope of the present document

Whereas in the interconnection case the access token request is sent by the CCF-B to CCF-A on behalf of the API invoker i.e., the actual client, but it is not the forwarded access token request from the API invoker, instead it is an access token request sent by the CCF-B to the CCF-A, so this pCR provides the clarifications on the access token request message information sent from CCF-B to CCF-A to avoid the confusion that CCF-B is not considered as client here, but the API invoker is still considered as the client for which the access token is to be issued.

* * * First Change * * * *

## 6.X	Security procedures for CAPIF interconnection

### 6.X.1	General

The CAPIF provider A and CAPIF provider B host the CAPIF in their trust domains as specified in clause 6.2.2 in TS 23.222 [3]. The designated CAPIF core function of the CAPIF provider A interconnects with the designated CAPIF core function of the CAPIF provider B over CAPIF-6/6e interface.

The following clauses 6.X.2 and 6.X.3 details security aspects of the scenario where, that the API invoker is onboarded to CCF-B of the CAPIF provider B and the target AEF is registered to CCF-A of CAPIF provider A.

### 6.X.2	Security method negotiation

For security method negotiation procedure in CAPIF interconnection, clause 6.3.1.2 shall be followed with the following enhancement:

- The API invoker shall send the security method request to the CCF-B.

- In case where CCF-B is in possession of the security method(s) as specified in clause 8.25.3.1 of TS 23.222 [3], CCF-B shall select a security method to be used over CAPIF-2/2e reference point for each AEF based on the access scenarios and AEF capabilities.

- In case where CCF-B is not in possession of the security method(s), based on the AEF details received from the API invoker, CCF-B identifies the CCF-A where the AEF is registered and sends the request to CCF-A to either get the supported list of security method(s) of AEF or to get a selected security method. The request to CCF-A shall include AEF details and may include the API invoker ID and security method supported by API invoker (e.g., to enable CCF-A to select the security method). The CCF-A shall provide to CCF-B either the list of supported security methods of AEF or the selected security method. If the list of supported security methods of AEF is received, the CCF-B shall select a security method to be used over CAPIF-2/2e reference point for each AEF based on the access scenarios and AEF capabilities.

- The CCF-B shall send Security Method Response message to the API invoker indicating the selected security method for each AEF.

### 6.X.3	Authentication and authorization procedure

For the mutual authentication between the API invoker onboarded to CCF-B in CAPIF provider B and AEF registered to the CCF-A in CAPIF provider A, the procedures as defined in clause 6.5.2 shall be followed with the enhancements as specified in clause 6.X.3.1, 6.X.3.2 and 6.X.3.3.

Editor’s Note: Security procedure for CAPIF RNAA interconnection is FFS.

#### 6.X.3.1	Method 1: Using TLS-PSK

- The API invoker shall send the authentication initiation request message to the target AEF in CAPIF provider domain A for CAPIF interconnection. Based on the AEF details available at the API invoker, which indicates the AEF belongs to CCF-A, the authentication initiation request message shall include the information for identification of the CCF-B.

Editor’s Note:  Whether CCF-B ID is a sensitive information against AEF in different domain is FFS.

- The AEF shall request for security information (AEFPSK) from CCF-A to perform authentication and secure connection establishment with the API invoker, if the AEF does not have a security information. The request shall include the API invoker ID and the CCF-B ID (if received from the API invoker).

- When the CCF-A receives the request message from the AEF for security information, the CCF-A fetches security information based on API invoker ID, and the CCF-B ID.

- If the CCF-A does not have security information, CCF-A shall request the security information from CCF-B over CAPIF-6/6e reference point based on the received API invoker ID, and the available AEF details (including the service API interface information).

NOTE: The CCF-A can check the received API Invoker ID and CCF-B ID based on the information locally available and contacts the right CCF-B to fetch the security information.

- The CCF-B shall provide the security information related to the chosen security method to the CCF-A.

- The CCF-A shall provide the received security information to the AEF.

- The AEF sends the authorization information request to CCF-A. CCF-A sends the authorization information to the AEF. If CCF-A does not have sufficient information for authorization, CCF-A fetches additional information related to the API invoker from CCF-B based on the business relationship. The AEF performs authorization based on the obtained authorization information.

#### 6.X.3.2	Method 2: Using PKI

The API invoker onboarded to CCF-B and the AEF registered to CCF-A shall follow the procedure in subclause 6.X.3.1 with the following adaptation to establish dedicated secure session over CAPIF-2e using TLS based on certificate based mutual authentication.

- For fetching the security information related to the chosen security method (TLS-PKI) the CCF-B includes only the API invoker ID.

- The CCF-B provides the security information (API invoker's root CA certificate) to the AEF via CCF-A, for the AEF to validate the API invoker's certificate.

#### 6.X.3.3		Method 3: TLS with OAuth Token

- The API invoker shall send the access token request message to the onboarded CCF-B, CCF-B determines that the service API requested is provided by the AEFs in CAPIF provider domain A.

- The CCF-B sends the access token request message with API Invoker ID, list of service API information of AEF (according to Annex C.3.2 for the client_id and scope) to the CCF-A in CAPIF provider domain A. In interconnection scenario, the Onboard_secret is not included in the access token request message to the CCF-A. The CCF-A generates and provides an access token specific to the API invoker and returns it in an Access Token Response message to the API invoker via CCF-B as specified in clause 6.5.2.3.

- On CAPIF-2e, the API invoker authenticates to the AEF by establishing a TLS session with the AEF as specified in clause 6.X.3.1 or 6.X.3.2.

- With successful authentication to the AEF on CAPIF-2e, the API invoker shall initiate invocation of a 3GPP northbound API with the AEF including the access token in northbound API invocation request as per OAuth 2.0. The AEF verifies the integrity of the access token by verifying the CCF’s signature to validate the access permission for the requested service API as specified in clause 6.5.2.3.

Editor’s Note: Whether CCF-A will have sufficient information to authenticate the API invoker before including API invoker ID into the access token is FFS.

#### 6.X.3.4		Authentication and Authorization for CAPIF RNAA interconnection

Editor’s Note: Security procedure for CAPIF RNAA interconnection is FFS.

* * * End of Change * * * *