3GPP TSG-SA3 Meeting #122																			S3-251899

Fukuoka, Japan, 19 – 23 May 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.511 | CR | 0087 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Wrong test steps for IPSec testing in N2/Xn - 33.511 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_Maint |  |  |  |  |  | Date: |  |  | 2025-04-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There is a mismatch between the requirement in test case 4.2.2.1.16/17 and the test case 4.2.3.2.4 in 33.117. The requirement specifies that the transport protocol must be secured (integrity, confidentiality, and replay protection), but in test case 4.2.3.2.4, we only check that the DUT supports the protocol on the interface. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Rewrite the test case in 33.511 to check the messages transmitted in the interface, and check if the IPSec is fully enabled for them. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirement and the test case are checking 2 different things and are not appropiate |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.1.16, 4.2.2.1.17 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** FIRST CHANGE ****

##### 4.2.2.1.16	Control plane data confidentiality protection over N2/Xn interface

Requirement Name: Control plane data confidentiality protection over N2/Xn interface

Requirement Reference: TS 33.501 [2], clauses 9.2 and 9.4

Requirement Description: The transport of control plane data over N2 is integrity, confidentiality and replay-protected. The transport of control plane data and user data over Xn is integrity, confidentiality and replay-protected, as specified in TS 33.501 [2], clauses 9.2 and 9.4.

Threat References: TR 33.926 [5], clause D.2.2.1 – Control plane data confidentiality protection.

Test Case:

Test Name: TC-CP-Conf-N2-Xn

Purpose: To verify the confidentiality of the N2 and Xn interface is properly established

Pre-Condition:

- The gNB network product shall be connected in emulated/real network environments.

- The tester shall have access to the N2/Xn interface.

- The tester shall activate the protection mechanism for the N2/Xn interface for the ControlPlane

Execution Steps:

- The tester shall capture the data sent between the gNB and the AMF (N2 Interface) or between 2 gNBs (Xn interface)

- Tester gets all the packets for the selected interface (N2 or Xn)

- Tester filter for IPSec and NGAP packets.

Expected Results:

- The tester only gets IPSec packets when filter, which means no plain packets are transmitted and the confidentiality is covered.

**** END CHANGE ****

**** SECOND CHANGE ****

##### 4.2.2.1.17	Control plane data integrity protection over N2/Xn interface

Requirement Name: Control plane data integrity protection over N2/Xn interface

Requirement Reference: TS 33.501[2], clauses 9.2 and 9.4

Requirement Description: The transport of control plane data over N2 is integrity, confidentiality and replay-protected. The transport of control plane data and user data over Xn is integrity, confidentiality and replay-protected. as specified in TS 33.501 [2], clauses 9.2 and 9.4.

Threat References: TR 33.926 [5], clause D.2.2.2 – Control plane data integrity protection.

Test Case:

Test Name: TC-CP-Inte-N2-Xn

Purpose: To verify the integrity of the N2 and Xn interface is properly established

Pre-Condition:

- The gNB network product shall be connected in emulated/real network environments.

- The tester shall have access to the N2/Xn interface.

- The tester shall activate the protection mechanism for the N2/Xn interface for the ControlPlane

Execution Steps:

- The tester shall capture the data sent between the gNB and the AMF (N2 Interface) or between 2 gNBs (Xn interface)

- Tester gets all the packets for the selected interface (N2 or Xn)

- Tester filter for IPSec and NGAP packets.

Expected Results:

- The tester only gets IPSec packets when filter, which means no plain packets are transmitted and the integrity is covered.

**** END CHANGE ****